| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0182-v1 |
| --- | --- |

| Title: | EE1-1.1.4.b: Group convolution for HOP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents the test result of EE1-1.1.4.b. This test investigates applications of group convolution to different parts of HOP. The proposed change results in obvious kMAC/pixel reduction. Compared with existing HOP filter, kMAC/pixel is reduced by 12% to 27% reportedly. BD-rate changes on top of NNNC-6.0 anchor (NN intra + LOP) and NNVC-6.0 HOP anchor (NN intra + HOP) are reportedly summarized as below:

Applied to 3×1 conv:

Compared with NNVC-6.0 anchor: RA: -5.14%, -17.13%, -19.12%, LB: -5.83%, -14.37%, -14.51%, AI: -3.17%, -14.17%, -15.23%

Compared with NNVC-6.0 HOP anchor: RA: 0.26%, 2.72%, 1.03%, LB: 0.18%, -0.01%, -5.79%, AI: 0.15%, 0.69%, 0.73%

kMAC/pixel: 407

Applied to 1×3 conv:

Compared with NNVC-6.0 anchor: RA: -5.13%, -18.29%, -19.00%, LB: -5.84%, -14.73%, -16.23%, AI: -3.13%, -14.01%, -15.11%

Compared with NNVC-6.0 HOP anchor: RA: 0.26%, 1.12%, 1.17%, LB: 0.16%, -0.38%, -7.45%, AI: 0.18%, 0.88%, 0.87%

kMAC/pixel: 419

Applied to both 3×1 and 1×3 conv:

Compared with NNVC-6.0 anchor: RA: -4.84%, -17.13%, -18.06%, LB: %, %, %, AI: -2.99%, -13.72%, -14.93%

Compared with NNVC-6.0 HOP anchor: RA: 0.58%, 2.64%, 2.38%, LB: %, %, %, AI: 0.33%, 1.27%, 1.11%

kMAC/pixel: 349

Introduction

HOP filter architecture is shown in Fig. 1. Spatial-wise separable convolution is used in the residual block, i.e. 1x3 and 3x1 layers in the Fig. 1.

Proposed method

Network structure

To reduce the complexity of HOP filter, it is proposed to apply group convolution to the 1×3 and 3×1 layers in the residual block. In particular, group convolution is applied to 3×1 layers, 1×3 layers, and both layers as shown in Fig. 2, Fig. 3, and Fig. 4 respectively. Number of groups is set as 2 for all cases.

Fig. 1. Existing HOP model.

Fig. 2. Proposed HOP model, with group convolution applied to 3×1 layers.

Fig. 3. Proposed HOP model, with group convolution applied to 1×3 layers.

Fig. 4. Proposed HOP model, with group convolution applied to both 3×1 and 1×3 layers.

Inference

SADL is used for performing the inference of the proposed CNN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | Applied to 3×1: 1.23M Applied to 1×3: 1.27M Applied to both: 1.05M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | Applied to 3×1: 407 Applied to 1×3: 419 Applied to both: 349 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

HOP stage 3 training is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 200h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed method is integrated into NNVC-6.0 and tested according to the common test conditions defined in [1].

Applied to 3×1 conv

Compared with NNVC-6.0 anchor (nn intra + lop), the proposed method brings on average {-5.14%, -17.13%, -19.12}, {-5.83%, -14.37%, -14.51%}, and {-3.17%, -14.17%, -15.23%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.43% | -13.97% | -22.06% | 167% | 897% |
| Class A2 | -5.30% | -16.93% | -14.77% | 162% | 941% |
| Class B | -4.78% | -17.98% | -19.13% | 166% | 897% |
| Class C | -5.23% | -18.60% | -20.16% | 149% | 850% |
| Class E |  |  |  |  |  |
| Overall | -5.14% | -17.13% | -19.12% | 161% | 893% |
| Class D | -5.61% | -19.21% | -20.57% | 145% | 763% |
| Class F | -3.76% | -13.44% | -13.17% | 195% | 300% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.15% | -13.86% | -13.90% | 158% | 845% |
| Class C | -5.69% | -16.88% | -17.63% | 148% | 923% |
| Class E | -7.14% | -11.87% | -11.39% | 226% | 628% |
| Overall | -5.83% | -14.37% | -14.51% | 169% | 808% |
| Class D | -5.56% | -14.82% | -11.16% | 145% | 823% |
| Class F | -5.14% | -14.99% | -13.87% | 192% | 300% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -2.52% | -15.11% | -17.75% | 153% | 885% |
| Class A2 | -2.86% | -14.13% | -11.87% | 129% | 863% |
| Class B | -2.80% | -13.96% | -14.59% | 125% | 839% |
| Class C | -3.46% | -12.35% | -15.04% | 115% | 701% |
| Class E | -4.35% | -16.05% | -17.40% | 131% | 836% |
| Overall | -3.17% | -14.17% | -15.23% | 129% | 817% |
| Class D | -2.84% | -12.74% | -15.12% | 112% | 577% |
| Class F | -2.95% | -11.57% | -10.99% | 114% | 745% |

Compared with NNVC-6.0 HOP anchor (nn intra + hop), the proposed method brings on average {0.26%, 2.72%, 1.03%}, {0.18%, -0.01%, -5.79%}, and {0.15%, 0.69%, 0.73%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.02% | 5.22% | -0.02% | 122% | 156% |
| Class A2 | 0.42% | 3.02% | 5.41% | 122% | 158% |
| Class B | 0.29% | 2.33% | -0.34% | 120% | 157% |
| Class C | 0.31% | 1.11% | 0.26% | 115% | 151% |
| Class E |  |  |  |  |  |
| Overall | 0.26% | 2.72% | 1.03% | 120% | 155% |
| Class D | 0.25% | 0.33% | -0.68% | 115% | 155% |
| Class F | -0.02% | 0.60% | 0.38% | 126% | 164% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.16% | -0.10% | -6.12% | 121% | 156% |
| Class C | 0.23% | -0.79% | -2.79% | 116% | 157% |
| Class E | 0.15% | 1.17% | -9.23% | 131% | 160% |
| Overall | 0.18% | -0.01% | -5.79% | 121% | 157% |
| Class D | 0.25% | -0.61% | -4.18% | 114% | 156% |
| Class F | 0.06% | 0.31% | -3.32% | 125% | 153% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.05% | 0.50% | 0.34% | 118% | 159% |
| Class A2 | 0.27% | 1.38% | 2.19% | 113% | 161% |
| Class B | 0.08% | 0.54% | 0.29% | 109% | 159% |
| Class C | 0.17% | 0.36% | 0.29% | 108% | 154% |
| Class E | 0.18% | 0.89% | 1.00% | 111% | 154% |
| Overall | 0.15% | 0.69% | 0.73% | 111% | 157% |
| Class D | 0.15% | 0.90% | 1.14% | 107% | 143% |
| Class F | -0.14% | 0.55% | 1.36% | 105% | 148% |

Applied to 1×3 conv

Compared with NNVC-6.0 anchor (nn intra + lop), the proposed method brings on average {-5.13%, -18.29%, -19.00%}, {-5.84%, -14.73%, -16.23%}, and {-3.13%, -14.01%, -15.11%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.06% | -17.35% | -22.24% | 174% | 991% |
| Class A2 | -5.47% | -18.04% | -14.89% | 168% | 1050% |
| Class B | -4.87% | -18.85% | -19.28% | 173% | 998% |
| Class C | -5.27% | -18.49% | -19.32% | 153% | 929% |
| Class E |  |  |  |  |  |
| Overall | -5.13% | -18.29% | -19.00% | 167% | 988% |
| Class D | -5.67% | -18.72% | -19.95% | 147% | 810% |
| Class F | -3.73% | -13.72% | -13.20% | 203% | 303% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -5.09% | -14.77% | -15.49% | 164% | 936% |
| Class C | -5.65% | -17.33% | -19.11% | 154% | 1008% |
| Class E | -7.36% | -11.20% | -13.61% | 237% | 706% |
| Overall | -5.84% | -14.73% | -16.23% | 176% | 894% |
| Class D | -5.61% | -17.15% | -16.45% | 151% | 908% |
| Class F | -4.93% | -15.06% | -13.71% | 207% | 348% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -2.34% | -14.53% | -16.69% | 156% | 989% |
| Class A2 | -2.89% | -13.67% | -11.77% | 130% | 938% |
| Class B | -2.84% | -13.91% | -15.01% | 127% | 928% |
| Class C | -3.43% | -12.48% | -14.86% | 114% | 818% |
| Class E | -4.28% | -16.05% | -17.35% | 132% | 903% |
| Overall | -3.13% | -14.01% | -15.11% | 129% | 909% |
| Class D | -2.86% | -12.57% | -15.00% | 110% | 688% |
| Class F | -3.05% | -12.38% | -12.55% | 116% | 841% |

Compared with NNVC-6.0 HOP anchor (nn intra + hop), the proposed method brings on average {0.26%, 1.12%, 1.17%}, {0.16%, -0.38%, -7.45%}, and {0.18%, 0.88%, 0.87%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.37% | 0.41% | -0.29% | 127% | 172% |
| Class A2 | 0.24% | 1.44% | 5.25% | 126% | 177% |
| Class B | 0.21% | 1.25% | -0.51% | 125% | 175% |
| Class C | 0.27% | 1.27% | 1.30% | 118% | 165% |
| Class E |  |  |  |  |  |
| Overall | 0.26% | 1.12% | 1.17% | 124% | 172% |
| Class D | 0.18% | 0.91% | 0.04% | 116% | 165% |
| Class F | 0.02% | 0.24% | 0.33% | 131% | 166% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.22% | -1.07% | -7.70% | 125% | 173% |
| Class C | 0.28% | -1.27% | -4.30% | 121% | 172% |
| Class E | -0.08% | 1.96% | -11.21% | 138% | 180% |
| Overall | 0.16% | -0.38% | -7.45% | 127% | 174% |
| Class D | 0.20% | -3.14% | -8.49% | 118% | 173% |
| Class F | 0.29% | 0.12% | -3.32% | 135% | 177% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.24% | 1.19% | 1.59% | 120% | 178% |
| Class A2 | 0.24% | 1.95% | 2.28% | 113% | 175% |
| Class B | 0.04% | 0.60% | -0.25% | 110% | 176% |
| Class C | 0.21% | 0.20% | 0.49% | 107% | 179% |
| Class E | 0.25% | 0.85% | 1.06% | 112% | 167% |
| Overall | 0.18% | 0.88% | 0.87% | 112% | 175% |
| Class D | 0.12% | 1.07% | 1.26% | 105% | 170% |
| Class F | -0.24% | -0.50% | -0.05% | 107% | 167% |

Applied to both 3×1 and 1×3 conv

Compared with NNVC-6.0 anchor (nn intra + lop), the proposed method brings on average {-4.84%, -17.13%, -18.06%}, {%, %, %}, and {-2.99%, -13.72%, -14.93%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.15% | -16.14% | -21.52% | 191% | 1181% |
| Class A2 | -5.11% | -17.24% | -13.91% | 183% | 1250% |
| Class B | -4.51% | -17.48% | -17.94% | 187% | 1188% |
| Class C | -4.81% | -17.38% | -18.74% | 164% | 1100% |
| Class E |  |  |  |  |  |
| Overall | -4.84% | -17.13% | -18.06% | 181% | 1175% |
| Class D | -5.19% | -17.98% | -18.75% | 160% | 970% |
| Class F | -3.44% | -12.95% | -12.77% | 227% | 351% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -5.35% | -17.21% | -19.32% | 166% | 1175% |
| Class E | -6.81% | -13.34% | -12.81% | 274% | 844% |
| Overall |  |  |  |  |  |
| Class D | -5.42% | -17.82% | -16.38% | 162% | 1015% |
| Class F | -4.66% | -13.97% | -14.97% | 226% | 369% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -2.33% | -14.76% | -17.85% | 170% | 1182% |
| Class A2 | -2.72% | -13.55% | -11.64% | 137% | 1113% |
| Class B | -2.65% | -13.64% | -14.34% | 133% | 1119% |
| Class C | -3.25% | -11.84% | -14.18% | 119% | 953% |
| Class E | -4.13% | -15.48% | -17.31% | 139% | 1140% |
| Overall | -2.99% | -13.72% | -14.93% | 137% | 1092% |
| Class D | -2.72% | -12.08% | -14.17% | 111% | 731% |
| Class F | -2.75% | -11.20% | -11.52% | 118% | 1009% |

Compared with NNVC-6.0 HOP anchor (nn intra + hop), the proposed method brings on average {0.58%, 2.64%, 2.38%}, {%, %, %}, and {0.33%, 1.27%, 1.11%} BD-rate changes for {Y, Cb, Cr} under RA, LB, and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.27% | 2.02% | 0.69% | 140% | 205% |
| Class A2 | 0.64% | 2.51% | 6.51% | 138% | 210% |
| Class B | 0.58% | 3.02% | 1.20% | 136% | 208% |
| Class C | 0.75% | 2.71% | 2.05% | 127% | 195% |
| Class E |  |  |  |  |  |
| Overall | 0.58% | 2.64% | 2.38% | 135% | 204% |
| Class D | 0.70% | 1.93% | 1.59% | 127% | 197% |
| Class F | 0.32% | 1.22% | 0.84% | 146% | 192% |

|  | Low delay B Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | 0.60% | -1.14% | -4.72% | 131% | 200% |
| Class E | 0.51% | -0.46% | -10.44% | 159% | 214% |
| Overall |  |  |  |  |  |
| Class D | 0.40% | -3.60% | -8.35% | 127% | 193% |
| Class F | 0.58% | 1.66% | -4.56% | 148% | 187% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.25% | 0.96% | 0.24% | 130% | 213% |
| Class A2 | 0.41% | 2.12% | 2.47% | 120% | 207% |
| Class B | 0.24% | 0.98% | 0.60% | 115% | 212% |
| Class C | 0.39% | 0.98% | 1.36% | 112% | 209% |
| Class E | 0.40% | 1.63% | 1.15% | 118% | 210% |
| Overall | 0.33% | 1.27% | 1.11% | 118% | 210% |
| Class D | 0.27% | 1.67% | 2.31% | 106% | 181% |
| Class F | 0.07% | 0.93% | 1.21% | 109% | 200% |

Conclusions

This contribution presents a group convolution-based complexity reduction for HOP. The proposed filtering method brings an obvious reduction in kMAC/pixel. It is suggested to adopt the proposed design into HOP.

References

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AE2016, Geneva, CH, July 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).