| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0183-v1 |
| --- | --- |

| Title: | EE1-1.1.4.c/d: Separate models for HOP |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Yue Li, Kai Zhang, Li Zhang (Bytedance) | Tel: Email: | yue.li@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

- Abstract

This contribution presents the test result of EE1-1.1.4.c/d. This contribution investigates separate models for luma and chroma in test EE1-1.1.4.c, as well as a combination with simplifications in EE1-1.1.4.a and EE1-1.1.4.b in test EE1-1.1.4.d. The proposed change results in obvious kMAC reduction. Compared with existing HOP filter, separate models and simplified separate models could reduce kMAC/pixel by 1% and 18% respectively. In addition, simplified separate models reduce worst-case inference memory by 75% due to adoption of simplified feature extraction from EE1-1.1.4.a. BD-rate changes on top of NNNC-6.0 anchor (NN intra + LOP) and NNVC-6.0 HOP anchor (NN intra + HOP) are reportedly summarized as below:

EE1-1.1.4.c (Separate models):

Compared with NNVC-6.0 anchor: RA: -5.71%, -17.96%, -18.44%, AI: -3.66%, -14.45%, -15.38%

Compared with NNVC-6.0 HOP anchor: RA: -0.34%, 1.50%, 1.90%, AI: -0.36%, 0.30%, 0.52%

kMAC/pixel: 473 (366 + 107)

EE1-1.1.4.d (Separate models with simplifications in EE1-1.1.4.a and EE1-1.1.4.b):

Compared with NNVC-6.0 anchor: RA: -5.36%, -17.80%, -18.41%, AI: -3.46%, -14.91%, -15.74%

Compared with NNVC-6.0 HOP anchor: RA: 0.02%, 1.76%, 1.96%, AI: -0.15%, -0.13%, 0.20%

kMAC/pixel: 390 (298 + 92).

Introduction

HOP filter architecture is shown in Fig. 1. Spatial-wise separable convolution is used in the residual block, i.e. 1×3 and 3×1 layers in the Fig. 1. In addition, a single model is used for processing both luma and chroma components.

Proposed method

EE1-1.1.4.c (Separate models)

Considering that luma and chroma have different characteristics, separate models are designed to filter luma and chroma components, as shown in Fig. 2, Fig. 3.

Fig. 1. Existing HOP model.

Fig. 2. Proposed HOP luma model.

Fig. 3. Proposed HOP chroma model.

EE1-1.1.4.d (Separate models with simplifications)

To further simplify the separate models, it is proposed to integrate the simplifications from EE1-1.1.4.a and EE1-1.1.4.b into separate models, as shown in Fig. 4 and Fig. 5.

Fig. 4. Proposed HOP luma model with simplifications.

Fig. 5. Proposed HOP chroma model with simplifications.

Inference

SADL is used for performing the inference of the proposed NN filters in VTM. Both weights and feature maps are represented with int16 precision using a static quantization method. The network information in the inference stage is provided in Table 1.

Table 1. Network Information for NN-based Video Coding Tool Testing in Inference Stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 1.1.4.c: 1.42M 1.1.4.d: 1.23M |
|  | Parameter Precision (Bits) | 16 (I) |
|  | Memory Parameter (MB) | Total Parameters Number × 2 |
|  | Multiply Accumulate (kMAC/pixel) | 1.1.4.c: 473 1.1.4.d: 390 |
| Optional | Total Conv. Layers | 154 |
|  | Total FC Layers | 0 |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 144×144, 272×272 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage |  |
|  | Other information: |  |

Training

HOP stage 3 training is applied. The network information in the training stage is provided in Table 2.

Table 2. Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: A100-SXM-80GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 20 |
|  | Batch size: | 64 |
|  | Training time: | 200h |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC, QP {22, 27, 32, 37, 42} |
|  | Loss function: | L1, L2 |
| Optional | Number of iterations |  |
|  | Patch size | 128×128 |
|  | Learning rate: | 1e-4 |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |

Experimental results

The proposed method is integrated into NNVC-6.0 and tested according to the common test conditions defined in [1].

EE1-1.1.4.c (Separate models)

Compared with NNVC-6.0 anchor (nn intra + lop), the proposed method brings on average {-5.71%, -17.96%, -18.44%} and {-3.66%, -14.45%, -15.38%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.42% | -17.55% | -20.64% | 181% | 1215% |
| Class A2 | -6.13% | -17.84% | -14.81% | 176% | 1308% |
| Class B | -5.48% | -19.07% | -19.11% | 179% | 1128% |
| Class C | -5.89% | -16.98% | -18.67% | 161% | 1073% |
| Class E |  |  |  |  |  |
| Overall | -5.71% | -17.96% | -18.44% | 174% | 1163% |
| Class D | -6.33% | -18.65% | -19.16% | 158% | 934% |
| Class F | -4.04% | -12.94% | -12.96% | 224% | 367% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -2.76% | -16.04% | -17.13% | 162% | 1192% |
| Class A2 | -3.36% | -14.58% | -11.26% | 131% | 1178% |
| Class B | -3.30% | -14.19% | -15.72% | 125% | 1074% |
| Class C | -4.04% | -12.32% | -14.99% | 110% | 910% |
| Class E | -4.95% | -16.02% | -17.68% | 134% | 1078% |
| Overall | -3.66% | -14.45% | -15.38% | 129% | 1070% |
| Class D | -3.28% | -13.74% | -15.74% | 109% | 752% |
| Class F | -3.52% | -12.44% | -12.71% | 113% | 962% |

Compared with NNVC-6.0 HOP anchor (nn intra + hop), the proposed method brings on average {-0.34%, 1.50%, 1.90%} and {-0.36%, 0.30%, 0.52%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.00% | 0.04% | 1.84% | 133% | 211% |
| Class A2 | -0.45% | 1.69% | 5.40% | 133% | 220% |
| Class B | -0.44% | 0.83% | -0.34% | 130% | 197% |
| Class C | -0.40% | 3.27% | 2.14% | 124% | 191% |
| Class E |  |  |  |  |  |
| Overall | -0.34% | 1.50% | 1.90% | 129% | 202% |
| Class D | -0.53% | 1.12% | 1.07% | 125% | 190% |
| Class F | -0.31% | 1.25% | 0.59% | 144% | 201% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.19% | -0.81% | 1.05% | 125% | 215% |
| Class A2 | -0.25% | 0.78% | 2.85% | 114% | 219% |
| Class B | -0.44% | 0.20% | -1.12% | 109% | 203% |
| Class C | -0.43% | 0.40% | 0.34% | 104% | 199% |
| Class E | -0.45% | 0.97% | 0.63% | 113% | 199% |
| Overall | -0.36% | 0.30% | 0.52% | 112% | 206% |
| Class D | -0.31% | -0.25% | 0.38% | 104% | 186% |
| Class F | -0.73% | -0.61% | -0.36% | 104% | 191% |

EE1-1.1.4.d (Separate models with simplifications)

Compared with NNVC-6.0 anchor (nn intra + lop), the proposed method brings on average {-5.36%, -17.80%, -18.41%} and { -3.46%, -14.91%, -15.74%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.29% | -16.50% | -18.93% | 193% | 1361% |
| Class A2 | -5.74% | -17.84% | -19.03% | 188% | 1426% |
| Class B | -5.05% | -19.52% | -18.07% | 190% | 1193% |
| Class C | -5.53% | -16.58% | -17.97% | 170% | 1185% |
| Class E |  |  |  |  |  |
| Overall | -5.36% | -17.80% | -18.41% | 184% | 1267% |
| Class D | -5.94% | -17.46% | -17.33% | 167% | 992% |
| Class F | -3.71% | -12.86% | -12.65% | 239% | 443% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -2.71% | -15.33% | -16.53% | 171% | 1337% |
| Class A2 | -3.19% | -15.30% | -14.36% | 135% | 1306% |
| Class B | -3.13% | -15.29% | -15.83% | 129% | 1214% |
| Class C | -3.71% | -12.39% | -14.64% | 115% | 1030% |
| Class E | -4.68% | -16.81% | -17.64% | 139% | 1226% |
| Overall | -3.46% | -14.91% | -15.74% | 135% | 1206% |
| Class D | -2.95% | -12.70% | -14.39% | 111% | 843% |
| Class F | -3.18% | -12.06% | -12.52% | 116% | 1070% |

Compared with NNVC-6.0 HOP anchor (nn intra + hop), the proposed method brings on average {0.02%, 1.76%, 1.96%} and {-0.15%, -0.13%, 0.20%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations.

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | 0.13% | 1.34% | 4.01% | 141% | 236% |
| Class A2 | -0.04% | 1.93% | 0.11% | 141% | 240% |
| Class B | 0.01% | 0.26% | 0.96% | 137% | 209% |
| Class C | -0.01% | 3.82% | 3.05% | 131% | 211% |
| Class E |  |  |  |  |  |
| Overall | 0.02% | 1.76% | 1.96% | 137% | 221% |
| Class D | -0.12% | 2.64% | 3.45% | 132% | 202% |
| Class F | 0.04% | 1.43% | 1.08% | 154% | 243% |
|  | All Intra Main10 |  |  |  |  |
|  | BD-rate Over NNVC-6.0 HOP Anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -0.14% | 0.02% | 1.80% | 131% | 241% |
| Class A2 | -0.07% | 0.08% | -0.74% | 118% | 243% |
| Class B | -0.26% | -1.05% | -1.22% | 112% | 230% |
| Class C | -0.09% | 0.59% | 1.05% | 108% | 226% |
| Class E | -0.17% | 0.06% | 0.78% | 118% | 226% |
| Overall | -0.15% | -0.13% | 0.20% | 116% | 232% |
| Class D | 0.03% | 1.17% | 2.34% | 106% | 208% |
| Class F | -0.39% | 0.12% | 0.07% | 107% | 212% |

Conclusions

This contribution presents a separate models-based design for HOP. The proposed filtering method brings a reduction of 18% in kMAC/pixel and 75% in worst-case inference memory. It is therefore suggested to adopt the proposed design in EE1-1.1.4.d into HOP.

References

- E. Alshina, R. -L. Liao, S. Liu, A. Segall, Common test conditions and evaluation procedures for neural network-based video coding technology, JVET-AE2016, Geneva, CH, July 2023.

Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).