| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0186-v1 |
| --- | --- |

| Title: | AHG4: experiments related to VVC spatial scalability visual testing |  |  |  |
| --- | --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Information |  |  |  |
| Author(s) or Contact(s): | P. de Lagrange F. Urban | Email: | philippe.delagrange@interdigital.com |
| Source: | InterDigital |  |  |  |

_____________________________

# Abstract

This document reports about some experiments related to visual testing of VVC spatial scalability, as planned in JVET-AE2021, and after observations from expert viewing tests made during the previous JVET meeting, where the upscaled anchor was often found better than the dual-layer test. After investigation, it was found that moving the inter-layer reference first in the reference picture lists (instead of last) improved the situation. In addition, video sequences from LCEVC verification tests have been tested.

# Introduction

During the previous JVET meeting in Geneva, an expert viewing test has been performed to verify that dual-layer coding outperforms an upscaled anchor at comparable bitrate. The result has been reported in JVET-AE0288: it has been found that at least for CorsairJoy and TallBuildings test sequences, the quality of dual-layer is lower than the upscaled anchor. This is unexpected and can be interpreted as an encoder bug, since for the enhancement layer it has always the choice to take the upscaled prediction (base layer) without adding anything to it, so it should either do nothing or improve the quality.

The example of CorsairJoy has been investigated, because the upscaled anchor was almost identical to the base layer (same QP). It was found that the enhancement layer was actually degrading the visual quality compared to the base layer.

# Experiments

## Coding conditions

VTM 22.0 was used, with the --DMVREncMvSelect parameter set to 1. Only 2x scaling ratio was tested.

## Description of the visual problem

On inter pictures of CorsairJoy (enhancement layer), some erratic motion of blocks is observed in the smooth background below and to the right of the logo. This can extend to more than half of the picture width and is visually disturbing. Problems around the boots of the male dancer have also been reported. Better sharpness (especially the logo) was not judged worth those problems.

A potential explanation is that intra-layer prediction is chosen to preserve the sharpness of the logo, and the reference picture ID propagates from there. Analysis of reference picture maps confirms that visual artefacts correlate with wrong picture reference (choosing enhancement layer instead of base layer, whereas no enhancement happens).

## Layer QP offset

Increasing the QP offset of enhancement layer (compared to base layer) to +10 instead of +7 made the problem worse.

## Position of base layer in reference picture lists

In the current VTM, the base layer reference is inserted just after the last active reference picture by EncLib::xInitRPL function. This function has been patched for the experiment to put the base layer first instead of last. Reworking this to use the configuration files instead is considered, with a specific syntax for inter-layer pictures in the GOP structure specification, at it would be more flexible.

On CorsairJoy with EL QP offset +10 and +7, the situation is clearly improved, both visually and in terms of objective metrics: lower rate, better PSNR, better SSIM. However the problem is not totally gone: some artefacts related to a bad reference selection are still visible.

Figure 1 and Figure 2 detail the PSNR_Y and MS-SSIM_Y impact for QP(base)=34, with EL QP offset going up to +10. Nominal offset +7 is the black circle.

Other QP offsets have been explored, to conclude that the base layer in first position is beneficial as long as enhancement layer rate is not too large (QP offset is at least 5 or 6). This is not surprising: it makes more sense to favor base layer reference when it has a quality benefit.

TallBuildings sequence has been checked, and the visual benefit is clear, though pumping is still annoying (it is present on the base layer).

| Figure 1 – PSNR change for CorsairJoy | Figure 2 – MS-SSIM change for CorsairJoy |
| --- | --- |

## Lambda of enhancement layer

The lambda of the enhancement layer has been modified by a factor of 0.8 or 1.25, without a clear conclusion. The R/D points are shifted on the EL R/D curve, but without significant objective or visual impact.

## Other test sequences

It has been noted that during expert viewing in Geneva, people paid more attention to visual artefacts in smooth areas than the resolution improvement provided by the enhancement layer.

It is thus questioned if test content with more obvious difference between resolutions would be relevant, and if asking people to rate the uncompressed reduced resolution anchor would be a useful indication.

Following this idea, test sequences that were used for LCEVC verification tests (MPEG document w20173) have been encoded with dual-layer VVC. No reduced-resolution anchors have been created in lack of time to align bitrate. Base layer encodings can be used as useful proxy.

QPs indicated in Table 1 are either based on LCEVC VT (base layer and full-resolution anchor, shifted by +1 when HEVC was used), or recommendations considering the MOS values range. Gray values are already in the test plan (Marathon2, MountainBay2).

Table 1 – Extra test sequences and QP settings

| Test sequence & format | HD+UHD dual- layer QPs |  | UHD anchor QPs |  | Thumbnail |
| --- | --- | --- | --- | --- | --- |
|  | LCEVC | Rec. | LCEVC | Rec. |  |
| CatRobot1 3840×2160, 60fps, 600 frames | 24+31 38+35 33+40 37+44 | 22+29 27+34 32+39 37+44 | 28 33 38 43 | 28 33 38 43 |  |
| LupoPuppet 3840×2160, 50fps, 500 frames | 26+41 30+37 33+40 37+44 | 25+32 29+36 33+40 37+44 | 31 35 39 43 | 31 35 39 43 |  |
| Marathon2 3840×2160, 30fps, 300 frames | 26+33 29+36 32+39 36+43 | 26+33 30+37 35+42 40+47 | 30 34 38 42 | 32 36 41 46 |  |
| MountainBay2 3840×2160, 30fps, 300 frames | 29+36 33+40 37+44 40+47 | 24+31 28+35 32+39 36+43 | 34 38 42 46 | 30 34 38 42 |  |
| DrivingPOVLogo 3840×2160, 60fps, 600 frames | 24+31 27+34 33+40 37+44 | 26+33 31+38 36+43 40+47 | 28 32 38 42 | 32 37 42 46 |  |
| BoxeLogo 3840×2160, 60fps, 596 frames | 27+34 30+37 32+39 37+44 | 24+31 29+36 34+41 39+46 | 30 34 38 42 | 30 35 40 45 |  |
| TrafficHDLogo 1920x1080, 60fps, 600 frames | 22+29 24+31 29+36 31+38 | 20+27 24+31 28+35 32+39 | 26 30 34 38 | 26 30 34 38 |  |
| StarCraft 1920x1080, 60fps, 600 frames | 22+29 24+31 27+34 30+37 | 20+27 24+31 28+35 32+39 | 26 30 34 38 | 36 30 34 38 |  |

# Conclusion

It is suggested to run expert viewing test with base layer first in the reference picture lists, and including test sequence with more obvious difference between resolutions.

In addition, it is proposed to remove automatic insertion of inter-layer references in EncLib::xInitRPL, and rather specify them explicitly in the configuration files with extended syntax.