| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0194 |
| --- | --- |

| Title: | Non-EE2: On LIC flag in merge mode |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Y. Zhang V. Seregin H. Wang Z. Zhang C.- C. Chen H. Huang M. Karczewicz | Tel: Email: | yzh@qti.qualcomm.com |
| Source: | Qualcomm Incorporated |  |  |

_____________________________

# Abstract

This contribution proposes to signal LIC flag for inter-prediction merge modes and to apply BDOF for bi-predicted LIC. On top of ECM-10.0, the simulation results are reported as follows.

RA: -0.xx% / -0.xx% / -0.xx%; xx% enc, xx% dec

LB: -0.xx% / -0.xx% / -0.xx%; xx% enc, xx% dec.

# Introduction

In ECM [1], the LIC flag of an inter-prediction coding unit is implicitly derived for merge modes, except for adaptive BM merge mode and GPM mode which always turn LIC off. For the other merge modes, the LIC flag is derived by inheritance. When LIC flag is true, BDMVR and BDOF are disabled.

# Proposal

It is proposed to signal the LIC flag for inter-prediction merge modes, instead of inheriting the flag value from a merge candidate.

When a bi-predicted block applies LIC and meets BDMVR and/or BDOF conditions, the PU MVs are refined by minimizing bilateral matching cost and BDOF luma adjustment is applied to the block.

# Experimental results

This section reports the simulation results of the proposed method, according to common test conditions [2]. In the test, a LIC flag is signaled for regular merge mode and affine merge mode. The proposed method brings -0.xx% and -0.xx% gain for RA and LDB respectively.

Table 1. Simulation result

|  | Random Access Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.15% | -0.09% | 0.00% | 102% | 100% |
| Class E |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.03% | 0.14% | 0.00% | 103% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

|  | Low delay B Main 10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM10.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class C | -0.26% | -0.41% | 0.04% | 102% | 102% |
| Class E | -0.23% | -0.37% | 0.24% | 103% | 102% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class D | -0.08% | 0.01% | -0.04% | 102% | 101% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! |

# Conclusion

This contribution reports the results of signaling LIC flag for merge modes. Coding gain can be observed for both RA and LDB configurations. It is proposed to further study the proposed method in the next round of EE2.

# References

- https://vcgit.hhi.fraunhofer.de/ecm/ECM

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-Y2017, Jan. 2022.

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).