| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0210-v1 |
| --- | --- |

| Title: | AHG13: Exploration on the Capability of Frequency-Based Film Grain Synthesis |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Xuewei Meng, Wenhao Zhang, Scott Labrozzi | Author(s) or Contact(s): | xuewei.meng@disney.com wenhao.zhang2@disney.com scott.labrozzi@disney.com |
| Source: | Disney Streaming |  |  |

_____________________________

# Abstract

The state-of-the-art film grain synthesis framework cannot meet the needs of high-quality film grain synthesis. This deficiency may primarily stem from limitations in the denoiser, film grain modelling, or film grain synthesis. To isolate the denoiser's impact, we captured clean images and introduced film grain using Davinci Resolve. Then we evaluated the subjective results of film grain modelling and synthesis. We can see that for finer-grained film grain it is challenging for the frequency model to generate film grain effectively. For coarser-grained film grain, the AR model faces difficulties in generating film grain effectively. To further confirm the bottleneck of the film grain framework, we explored the capability of frequency-based film grain synthesis by generating 16470 film grain models and evaluating the 16470 synthesis results. Experimental results show that,

- In most cases, we can find a suitable film grain model for film grain synthesis, which means the capability of film grain SEI message and synthesis is sufficient.

- In the current film grain synthesis framework, the film grain modeling highly constraints the performance of film grain synthesis.

- The proposed metric SFGA in JVET-AF0209 can help to fine-tune the film grain models.

# Introduction

In the entertainment industry, film grain is widely present in TV and motion pictures. Although digital cameras do not produce film grain, film grain is often added to the captured videos for creative intent. When encoding these TV or motion picture videos, preserving film grain under limited bandwidth is a challenging task. To address this issue, film grain synthesis methods are proposed in video coding. A typical film grain framework can be described as shown in Figure 1.

Figure 1. A typical framework of film grain synthesis.

However, the state-of-the-art film grain synthesis framework cannot meet the needs of high-quality film grain synthesis, see the proposal [1]. The main reason may lie in the weak denoiser, film grain modelling, or film grain synthesis. We first remove the impact of denoiser by capturing the clean image and adding film grain by Davinci Resolve. Then we evaluate the subjective results of film grain modelling and synthesis, as shown in Figure 2 and Figure 3. We can see that for finer-grained film grain (Preset “16mm250D” and “35mm”), it is challenging for the frequency model to generate film grain effectively. For coarser-grained film grain (Preset “8mm 250D” and “8mm 500T”), the AR model struggles to generate film grain effectively.

(a)                           (b)                           (c)

Figure 2. Results of film grain synthesis. ((a). Ground-truth by Davinci Resolve preset “35mm”  (b) Frequency model synthesis result (c) AR model synthesis result)

(a)                           (b)                           (c)

Figure 3. Results of film grain synthesis. ((a). Ground-truth by Davinci Resolve preset “8mm 500T”  (b) Frequency model synthesis result (c) AR model synthesis result)

# Test method

## Select the clean and ground-truth images.

We first capture clean images by Sony-RX1RM2 with fixed ISO 100.

Figure 4. Clean images.

Then we crop five flat regions from the clean images.

Figure 5. Clean images (flat regions).

The ground-truth grainy images are generated by Davinci Resolve with presets “8mm 250D”, “8mm 500T”, “16mm”, “16mm 250D”, and “35mm”.

- Preset “8mm 250D”.       (b) Preset “8mm 500T”.     (c) Preset “16mm”.

(d) Preset “16mm 250D”.    (e) Preset “35mm”.

Figure 6. Film grain parameters in Davinci Resolve.

## Conduct the frequency-based film grain modeling and synthesis.

With the collected clean and ground-truth grainy images, frequency-based film grain modeling and synthesis process is conducted.

Figure 7. Generate test images by frequency model.

## Generate multiple film grain models based on the frequency-based SEI message.

There are lots of parameters in the frequency-based SEI message. As we select flat regions for the evaluation, the “SEIFGCNumIntensityIntervalMinus1Comp0//1/2” are set to 0. “SEIFGCIntensityIntervalLowerBoundComp0/1/2” is set to 0. “SEIFGCIntensityIntervalUpperBoundComp0/1/2” is set to 255. Other parameter as set as follows. There are 16470 models in total.

| Parameter | Settings |
| --- | --- |
| SEIFGCLog2ScaleFactor | 2, 3, 4, 5, 6, 7 |
| Strength | 0, 2, 4, 6, 8, 10, 12, 14, 22, 30, 62, 126, 254, 510, 1023 |
| Horizontal cut-off frequency | 2,3,4,5,6,7,8,9,10,11,12,13,14 |
| Vertical cut-off frequency | 2,3,4,5,6,7,8,9,10,11,12,13,14 |

## Conduct frequency-based film grain synthesis

With the 16470 models in Section 2.3, we conduct frequency-based film grain synthesis and derive 16470 grainy images for each clean image.

Figure 8. Generate test images by frequency model.

## Select the best synthesis results.

With the 16470 grain images, we select the best on based on the film grain objective metric proposed in [4].

# Experimental results

We first derive the noise power spectrum of the 16470 film grain results, which is shown in Figure 9.

Figure 9. Frequency domain film grain.

Then we evaluate the results. See the slides of this proposal.

# Conclusion

In this proposal, we explore the capability of frequency-based film grain synthesis method. Experimental results show that,

- In most cases, we can find a suitable film grain model for film grain synthesis, which means the capability of film grain SEI message and synthesis is sufficient.

- In the current film grain synthesis framework, the film grain modeling highly constraints the performance of film grain synthesis.

- The proposed metric SFGA in JVET-AF0209 can help to fine-tune the film grain models.

# References

[1] X. Meng, W. Zhang, S. Labrozzi. AHG13: On Film Grain Synthesis Subjective Evaluation. JVET-AC0199, 29th JVET meeting, teleconference, Jan. 2023.

[2] X. Meng, W. Zhang, A. Norkin, J. Sole, A. Duenas. “On Film Grain Synthesis Subjective Evaluation and Further Work”. JVET-AD0238, 30th JVET meeting, Antalya, Apr. 2023.

[3] X. Meng, W. Zhang, S. Labrozzi. “AHG13: On Film Grain Objective Metrics”. JVET-AD0241, 30th JVET meeting, Antalya, Apr. 2023.

[4] X. Meng, W. Zhang, S. Labrozzi. “AHG13: Frequency domain Film Grain Objective Metrics”. JVET-AF0209, 32nd JVET meeting, Hannover, Oct. 2023.

# Patent rights declaration(s)

Disney Streaming Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).