| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0216-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0056 (EE1-1.2.2: Content-adaptive LOP filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhenyu Dai | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AF0056[1] , which is also known as EE1-1.2.2. In EE1-1.2.2, an integrated signalling mechanism is studied for the content-adaptive neural network loop-filter to get a better trade-off between complexity and performance. Both training and inference are conducted in this crosscheck.

From the observation of the completed results, the performance difference between crosschecker’s model and proponent’s model are {Y -0.04%, U 0.23%, V -0.08%} for random access configuration. It is asserted that the performance gap is minor.

Introduction

The content-adaptive approach for neural network loop-filter, as detailed in JVET-AE0093[2] , is achieved by means of overfitting one of the models of default LOP1 filter[3] in NNVC 5.0. The generated weight-updates are coded with the MPEG-NNR standard[4]. [5] . JVET-AF0056 reports the results of EE1-1.2.2, which study an integrated signalling mechanism to seamlessly carry the MPEG-NNR bitstream and other related data within the video bitstream.

Crosscheck status

Training crosscheck

As required by the proponent of JVET-AF0056, the training crosscheck was conducted during this meeting cycle. All training scripts were provided by proponents of EE1-1.2.2. The crosschecker tried the best to match the training environment as same as the proponent’s. Table 1 shows the details of NN information during the crosschecker’s training stage.

Table 1 Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network Information in Overfitting Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU NVIDIA A100-SXM-80GB |
|  | Framework: | TensorFlow 2.8.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 200 epochs for class C&D, 100 epochs for class A,B,F |
|  | Batch size: | 64 |
|  | Loss function: | Weighted-MSE |
|  | Training time (for 1 model): | A1: 15 hours A2: 15 hours B: 7 hours C: 2 hours D: 1.5 hour F: 2 hours |
|  | Training data information: | JVET CTC RA mandatory sequences |
|  | Training configurations for generating compressed training data (if different to VTM CTC): |  |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 72x72 |
|  | Learning rate: | 1e-3 |
|  | Learning rate update strategy | Constant learning rate |
|  | Optimizer: | ADAM |
|  | Preprocessing: | Convert 144x144 YUV420 signal to 6 72x72 blocks. Normalize to 0~1 |
|  | Mini-batch selection process: |  |
|  | Training data update strategy: |  |
|  | Other information: |  |

Inference crosscheck

The proposed method was implemented on top of NNVC-5.0, and the model precision is SADL int16.

Simulation results

The simulation was conducted under the NNVC common test condition[6] , and both the default LOP1 loop-filter and the NN intra prediction tool are enabled. The test results are listed below.

Results over NNVC5.0 with LOP1 and NNIntra

Table 2. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.0 (LOP1+ NNIntra) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -1.82% | -2.81% | -7.78% | 116% | 106% |
| Class A2 | -0.01% | -6.42% | -7.02% | 116% | 104% |
| Class B | -1.72% | -8.66% | -5.21% | 115% | 105% |
| Class C | -1.11% | -5.42% | -3.27% | 116% | 104% |
| Class E |  |  |  |  |  |
| Overall | -1.23% | -6.18% | -5.57% | 115% | 105% |
| Class D | -0.79% | -8.01% | -6.97% | 112% | 107% |
| Class F | -1.12% | -7.88% | -4.97% | 113% | 106% |

Please note that the encoder and decoder times are not reliable.

Results over JVET-AF0056

Table 3. Performance of the proposed method (RA)

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC5.0 (AF0056) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.21% | 0.47% | -0.24% | 117% | 101% |
| Class A2 | 0.07% | 1.17% | 0.41% | 116% | 99% |
| Class B | -0.04% | -0.18% | -0.23% | 116% | 100% |
| Class C | 0.00% | -0.16% | -0.16% | 116% | 100% |
| Class E |  |  |  |  |  |
| Overall | -0.04% | 0.23% | -0.08% | 116% | 100% |
| Class D | 0.07% | -0.24% | -0.03% | 113% | 103% |
| Class F | -0.05% | -0.34% | 0.02% | 113% | 101% |

Please note that the encoder and decoder times are not reliable.

Conclusion

It is the opinion of this crosschecker that the proposed method passes this crosscheck.

Reference

- R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, M. M. Hannuksela, A. Hallapuro (Nokia). EE1-1.2.2: Content-adaptive LOP filter. JVET-AF0056. October 2023.

- R. Yang, M. Santamaria, F. Cricri, H. Zhang, J. Lainema, R. G. Youvalari, M. M. Hannuksela. AHG11: Content-adaptive neural network loop-filter. JVET-AE0093. July 2023.

- J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, T. Shao, A. Arora, P. Yin, F. Pu, T. Lu, S. McCarthy. EE1-1.1: Complexity Reduction on Neural-Network Loop Filter. JVET-AD0156. April 2023.

- ISO/IEC FDIS 15938-17. Information technology – Multimedia content description interface – Part 17: Compression of neural networks for multimedia content description and analysis.

- Test Model of Incremental Compression of Neural Networks for Multimedia Content Description and Analysis (INCTM), N0179. February 2022.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31th Meeting, Geneva, 11–20 July 2023.