| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0217-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0139 (EE1-3.1: neural network-based intra prediction with reduced complexity) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | Zhenyu Dai | Tel: Email: | daizhenyu@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the cross-check results of JVET-AF0139[1] , which is also known as EE1-3.1. In EE1-3.1, a refined architecture and weight sparsity policy is studied for the models of the neural network-based intra prediction mode to get a better trade-off between complexity and performance. Both training and inference are conducted in this crosscheck.

From the observation of the completed results, the performance difference between crosschecker’s model and proponent’s model are {Y x.xx%, U x.xx%, V x.xx%} for all intra configuration. It is asserted that the performance gap is minor.

Introduction

In JVET-AC0116[2] , a neural network-based intra prediction mode is added to the set of intra prediction modes in VTM-11-NNVC. To limit the complexity of the neural network-based intra prediction mode, each neural network in this mode features a relatively small architecture and weight sparsity is enforced. However, the architecture and weight sparsity policy are not tuned to each neural network. To address this, JVET-AD0212[3] proposed a refined architecture and weight sparsity policy adapted to each neural network. JVET-AF0139 reports the results of EE1-3.1 associated to JVET-AD0212.

Crosscheck status

Training crosscheck

As required by the proponent of JVET-AF0139, the training crosscheck was conducted during this meeting period. All training scripts were provided by proponents of EE1-3.1. The crosschecker tried the best to match the training environment as same as the proponent’s.

The training process has 4 cycles, this crosschecker is requested to cross-check the cycle0, cycle1 and cycle3 process by the proponent. The training of these 3 cycles can be finished in about 8 days, provided that many CPUs (about 1500 cores) are available to generate the training sets at each training cycle. Table 1 shows the details of NN information during the crosschecker’s training stage.

Table 1 Network Information for NN-based Video Coding Tool Testing in Training Stage

| Network information in training stage |  |  |
| --- | --- | --- |
| Mandatory | GPU type | GPU NVIDIA A100-SXM-80GB |
|  | Framework | Tensorflow 2.6 |
|  | Number of GPUs per task | 1 |
|  | Epoch | 15 |
|  | Batch size | 2048 |
|  | Training time | 8 hours to 3 days, depending on the training cycle. |
|  | Training data information | During the generation of the training sets for training a neural network for intra prediction, pairs of a block and its context are extracted from the partitioning of YUV frames encoded/decoded via NNVC-6.0. |
|  | Training configurations for generating compressed training data | QP belonging to {18, 19, 20, 21, 22, 27, 32, 37}. |
|  | Patch size | The neural network predicting w×h blocks is trained on pairs of a w×h block and its context of reference samples. |
|  | Learning rate | Decreasing from 1e-4 to 5e-8. |
|  | Optimizer | ADAM. |
|  | Loss function | SATD between the original block and its neural network prediction. |
|  | Preprocessing | See [2] . |

Inference crosscheck

The proposed method was implemented on top of NNVC-6.0, Table 2 shows the details of NN information during the crosschecker’s inference stage.

Table 2 Network Information for NN-based Video Coding Tool Testing in inference Stage

| Network information (inference) |  |  |
| --- | --- | --- |
| Parameter number | block size | Parameter number |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 77477 120816 182356 229048 194023 238768 291311 |
| Parameter precision | 16-bit signed integer. |  |
| MAC/pixel | block size | MACs/pixel |
|  | 4×4 8×4 16×4 32×4 8×8 16×8 16×16 | 4837 3773 2848 1789 3030 1865 1138 |
| Total conv. layers |  |  |
| Total FC layers | 4 cascaded fully-connected layers for the neural network predicting 16×16 blocks. 3 cascaded fully-connected layers for the other six neural networks. |  |
| Mem (MB) | 4.9 MB for the whole neural network-based intra prediction mode. |  |
| GPU Type | No GPU |  |
| CPU Type | Xeon 2.4GHz |  |
| Framework | SADL (C++ with a single thread) |  |

Simulation results

Firstly, a quick test was performed for each training cycle (cycle0, cycle1, cycle3) based on the AI single frame configuration. And then, for cycle1 and cycle3, the complete test was performed based on the AI CTC configuration[4] .

The test results are listed below, only the NN intra prediction tool is enabled, and the model is in SADL float precision. Please note that the encoder and decoder times are not reliable.

Results over JVET-AF0139 cycle0

Table 3. Performance of the proposed method (AI single frame)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC6.0 (AF0139 cycle0) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.02% | 0.32% | 0.04% | 115% | 123% |
| Class A2 | -0.03% | 0.23% | -0.03% | 100% | 103% |
| Class B | 0.00% | -0.01% | -0.75% | 92% | 108% |
| Class C | 0.02% | 0.84% | 0.45% | 91% | 112% |
| Class E | 0.05% | -0.31% | 0.23% | 96% | 99% |
| Overall | 0.00% | 0.23% | -0.07% | 97% | 109% |
| Class D | -0.08% | 0.12% | 0.81% | 94% | 97% |
| Class F | 0.08% | -0.14% | 0.06% | 97% | 112% |

Results over JVET-AF0139 cycle1

Table 4. Performance of the proposed method (AI single frame)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC6.0 (AF0139 cycle1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.08% | -0.95% | -0.22% | 126% | 170% |
| Class A2 | -0.01% | 0.18% | -0.07% | 121% | 146% |
| Class B | 0.15% | 0.22% | -0.38% | 122% | 144% |
| Class C | 0.08% | 0.53% | -0.18% | 125% | 145% |
| Class E | -0.19% | 0.47% | -0.27% | 118% | 131% |
| Overall | 0.01% | 0.13% | -0.24% | 122% | 146% |
| Class D | -0.04% | -0.32% | 0.61% | 132% | 127% |
| Class F | 0.16% | 0.38% | 0.01% | 106% | 133% |

Table 5. Performance of the proposed method (AI CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC6.0 (AF0139 cycle1) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class C | 0.01% | 0.01% | -0.04% | 107% | 110% |
| Class E | -0.01% | 0.00% | -0.19% | 106% | 110% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.03% | -0.13% | -0.14% | 101% | 105% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Results over JVET-AF0139 cycle3

Table 6. Performance of the proposed method (AI single frame)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC6.0 (AF0139 cycle3) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.01% | -0.81% | -0.59% | 95% | 98% |
| Class A2 | -0.15% | 0.01% | -0.29% | 96% | 103% |
| Class B | -0.22% | 0.25% | -0.57% | 98% | 111% |
| Class C | -0.12% | 0.18% | -0.55% | 96% | 107% |
| Class E | -0.24% | 0.22% | 0.27% | 96% | 107% |
| Overall | -0.15% | 0.01% | -0.38% | 96% | 106% |
| Class D | -0.04% | -0.38% | 0.27% | 100% | 102% |
| Class F | -0.04% | 0.00% | -0.07% | 95% | 111% |

Table 7. Performance of the proposed method (AI CTC)

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over NNVC6.0 (AF0139 cycle3) |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0.20% | -0.36% | -0.46% | 90% | 87% |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class B | -0.14% | -0.42% | -0.47% | 93% | 89% |
| Class C | -0.12% | -0.25% | -0.36% | 101% | 105% |
| Class E | -0.19% | -0.31% | -0.33% | 98% | 99% |
| Overall | #VALUE! | #VALUE! | #VALUE! | #VALUE! | #VALUE! |
| Class D | -0.13% | -0.31% | -0.21% | 89% | 102% |
| Class F | -0.11% | -0.11% | -0.14% | 90% | 97% |

Conclusion

It is the opinion of this crosschecker that the proposed method passes this crosscheck.

Reference

- T. Dumas, F. Galpin, P. Bordes (InterDigital). EE1-3.1: neural network-based intra prediction with reduced complexity. JVET-AF0139. October 2023.

- T. Dumas, F. Galpin, P. Bordes. EE1-3.2: neural network-based intra prediction with learned mapping to VVC intra prediction modes. JVET-AC0116, 29th JVET meeting, Teleconference, 11-20 January 2023.

- T. Dumas, F. Galpin, P. Bordes. AHG11: neural network-based intra prediction with reduced complexity. JVET-AD0212, 30th JVET meeting, Antalya, 21-28 April 2023.

- E. Alshina, R.-L. Liao, S. Liu, A. Segall. JVET common test conditions and evaluation procedures for neural network-based video coding technology. JVET-AE2016, 31th Meeting, Geneva, 11–20 July 2023.