| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0224-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0075 (Evaluation results of LCEVC) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Information |  |  |
| Author(s) or Contact(s): | Christian Lehmann | Email: | firstname.lastname@hhi.fraunhofer.de |
| Source: | Fraunhofer HHI, Einsteinufer 37, 10587 Berlin, Germany |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results of the LCEVC evaluation results reported in JVET-AF0075. The results have been reproduced and can be confirmed. Furthermore, an informal subjective evaluation has been performed and the examples giving in JVET-AF0075 can be confirmed as well.

# Test conditions and settings

The crosschecks includes 6 UHD sequences from classes A1 und A2 from JVET Common Testing Conditions (CTC) by using random access (RA) configurations and default encoder configuration.
Following codecs have been tested as anchors:

- HM-16.25 reference software for H.265/HEVC ( base config: encoder_randomaccess_main10.cfg)

- VTM-19.0 reference software for H.266/VVC (base config: encoder_randomaccess_vtm.cfg)

The LCEVC reference software LTM-5.4.1 was used from:

https://mpeg.expert/software/MPEG/LCEVC/LTM/-/tree/LTM_5_4_1

by compiling the software based on the given documentation (README.md and SoftwareManual_LCEVCTestModelLTM.pdf). As the tag LTM_5_4_1 includes older versions of the base codecs, the binaries of the anchor (HM-16.25 respectively VTM-19.0) were used by copying the binaries and encoder configuration into the documented path “external_codecs/HM” respectively “external_codecs/VTM”.

For all encodings, decodings and PSNR calculation an AMD server (AMD EPXC 7502P) running Ubuntu 20.04 (gcc 10.5.0) was used.

# Test results

## Objective test results

Table 1: VTM-19.0 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) vs. 
 VTM-19.0 (2160p single layer) as anchor

| Sequence | Over VTM 19.0 (single layer) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |
|  | PSNR Y | PSNR Cb | PSNR Cr | PSNR YCbCr |
| Tango2 | -0.11% | -2.30% | 9.29% | 1.24% |
| FoodMarket4 | 6.03% | 4.99% | 5.31% | 5.87% |
| Campfire | 32.66% | 156.97% | 59.76% | 52.80% |
| CatRobot | 49.23% | 49.91% | 80.22% | 53.49% |
| DaylightRoad2 | 86.81% | 17.06% | 48.41% | 67.43% |
| ParkRunning3 | 14.95% | 370.64% | 220.60% | 33.05% |
| Overall | 31.59% | 99.55% | 70.60% | 35.65% |

Table 2: HM-16.25 (1080p base layer) + LTM-5.4.1 (2160p enhancement layer) vs. 
HM-16.25 (2160p single layer) as anchor

| Sequence | Over HM 16.25 (single layer) |  |  |  |
| --- | --- | --- | --- | --- |
|  | Random Access |  |  |  |
|  | PSNR Y | PSNR Cb | PSNR Cr | PSNR YCbCr |
| Tango2 | -8.04% | 4.76% | 7.81% | -3.82% |
| FoodMarket4 | -5.75% | 7.83% | 5.74% | -2.93% |
| Campfire | 6.99% | 153.47% | 19.89% | 24.39% |
| CatRobot | 17.72% | 34.20% | 55.32% | 23.48% |
| DaylightRoad2 | 42.87% | 12.06% | 41.83% | 39.64% |
| ParkRunning3 | -6.45% | 255.76% | 177.94% | 8.57% |
| Overall | 7.89% | 78.01% | 51.42% | 14.89% |