| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0226 |
| --- | --- |

| Title: | AHG12: SGPM with IntraTMP and IBC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Philippe Bordes, Ya Chen, | Email: | karam.naser@interdigital.com, |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

SGPM is an intra mode with prediction signal generated by blending 2 intra prediction modes with specific split line. This contribution proposes further including block-vector based prediction obtained from IntraTMP or IBC modes of the neighboring blocks in the candidate list of SGPM. The results are obtained:

AI:

RA:

# Introduction

SGPM is an intra mode similar to the inter GPM mode, where 2 regular intra predictors are used instead of inter predictors. In this mode, a candidate list is built with each entry containing one partition split and two intra prediction modes as shown in Figure 1.

Figure 1 Spatial GPM candidates.

The list is template-based reordered using  where SAD between the prediction and reconstruction of the template is used for ordering. The template size is fixed to 1.

On the other hand, IntraTMP and IBC are intra tools that use block vectors to copy reference block pixels located within the recontructed area. It is previously shown that both IBC and IntraTMP provides significant coding gain. However, the combination of IntraTMP/IBC with regular intra prediction and split line is under-exploited in ECM. Specifically, the IBC-GPM mode (JVET-AC0112) combines regular intra prediction with IBC but not IntraTMP.

# Proposal

This contribution proposes extending the idea of geometric prediction combing block-vector based prediction and regular intra prediction from IBC-GPM to SGPM.  That is, SGPM mode tests block vectors of neighboring CU’s in addition to regular prediction modes. Specifically, the following is proposed:

- Obtain block vectors of all merge candidates

- Select the best block vector according to SATD template cost

- Test the best block vector (if available) inside SGPM candidate list construction.

The difference with ECM-10 SGPM design is depicted in the Figure 2 below. It is about generalizing the two intra predictors from regular to regular or block-vector based:

Figure 2 modified SGPM process.

Identifiers intra_pred_0 and intra_pred_1 correspond to be regular prediction or block-vector based obtained from neighboring IntraTMP/IBC information.

It should be noted that there are no other changes regarding the signaling, number of candidates or encoder search.

# Experimental results

The proposal was implemented in ECM-10.0 and tested with CTC-AI and RA conditions. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | -0.06% | -0.06% | 99.6% | 100.1% | 96.4% |
| Class A2 | -0.05% | -0.05% | -0.06% | 99.9% | 100.7% | 96.6% |
| Class B | -0.03% | -0.03% | -0.08% | 100.0% | 100.6% | 99.9% |
| Class C | -0.03% | 0.02% | 0.03% | 100.4% | 101.0% | 99.9% |
| Class E | -0.05% | -0.06% | -0.13% | 100.8% | 102.5% | 100.0% |
| Overall | -0.03% | -0.03% | -0.06% | 100.2% | 100.9% | 98.8% |
| Class D | 0.00% | -0.12% | -0.22% | 98.7% | 102.8% | 100.0% |
| Class F | -0.22% | -0.06% | -0.05% | 100.5% | 105.1% | 99.7% |
| Class TGM | -0.45% | -0.45% | -0.43% | 99.0% | 105.7% | 99.5% |

# Conclusion

This contribution proposes adding block-vector based prediction to SGPM mode. This results in 0.03% gain in AI with higher gains in SC classes. It is proposed to study this in the next round of EE.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).