| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 |  |  |  | Document: JVET-AF0248-v1 |
| --- | --- | --- | --- | --- |
| Title: | Non-EE2: TIMD fusion optimization |  |  |  |
| Status: | Input document to JVET |  |  |  |
| Purpose: | Proposal |  |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Médéric Blestel, Chuan Zhou, Zhuoyi Lv | Tel: Email: | pandrivon@ofinno.com mblestel@ofinno.com chuan.zhou@vivo.com zhuoyi.lv@vivo.com |  |
| Source: | Ofinno LLC, vivo Mobile Communication Co., Ltd |  |  |  |

_____________________________

# Abstract

Contribution JVET-AE0137 introduced a non-angular intra predictor (among DC or Planar) in the TIMD fusion process, additionally to the two selected TIMD modes. Besides, the location-dependent sample-based fusion process from DIMD is adjusted for TIMD. Contribution JVET-AF0132 proposes an optimization method for reference lines selection in TIMD fusion process. This contribution merges both methods. On top of ECM-10.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

- AI: -0.06%, 0.01%, -0.06%  {100.0%, 98.7%}

- RA: X%, X%, X%  {X%, X%}

# Introduction

## TIMD

The TIMD process [1] is based on a template cost analysis in which several intra modes -- from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes -- are tested. The two intra modes (IPM1 and IPM2) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost under the condition that SATDIPM2<2.SATD(IPM1).

For TIMD mode with blending, the first mode uses reference line l. While for the second mode, whether to use reference line l or l+1 depends on the following conditions:

- If all of the following conditions are true, l+1 is chosen.

- the current block is not ISP block

- the second mode is angular prediction mode

- the second mode does not represent non-fractional angles

- Otherwise, l is chosen.

In this contribution, the fusion process are modified.

## TIMD fusion process with a non-angular intra mode

JVET-AE0137 [2] introduces a third non-angular intra prediction mode IPMNA in the TIMD fusion process.

First, the non-angular intra mode IPMNA (among DC and Planar) with the lowest SATD cost is added to the TIMD fusion process if it is different from the two selected TIMD intra modes (IPM1 and IPM2) and SATDIPMNA<k.SATDIPM1 with k=1.5.

The weights wi used during the TIMD fusion process are computed from SATD cost:

wi=sumSATD-SATD(IPMi)2×sumSATD

with

sumSATD=j=13SATD(IPMj)

The logic behind the non-angular prediction activation condition is based on already computed SATD costs.

Additionally, location-dependent sample-based blending of the DIMD fusion process [1] is re-used but the location-dependent criterion applying to amplitudes of the selected predictors is replaced by a SATD cost-based criterion. The location-dependent criterion is determined from a ratio of the normalized SATD of the selected TIMD predictors computed in ABOVE and LEFT template area, as follows:

If norSATDA<18 .norSATDL, locDepi=1 (vertical)

Else if norSATDL<18 .norSATDA, locDepi=2 (horizontal)

Otherwise, locDepi=0 (diagonal)

where:

- locDepi is the location-dependent parameter value associated with the i-th selected TIMD mode (e.g. i belongs to [0;2]). A value of 0 indicates no location-dependency (diagonal sample-based blending), a value of 1 indicates vertical location-dependency of the i-th selected TIMD mode, a value of 2 indicates horizontal location-dependency of the i-th selected TIMD mode,

- norSATDA is the normalized SATD cost associated to a selected TIMD candidate and computed in ABOVE template:

norSATDA=1hT×wCU

- norSATDL is the normalized SATD cost associated to a selected TIMD candidate and computed in LEFT template:

norSATDL=1wT×hCU

with:

- hT, the height of the (ABOVE) template area,

- wT, the width of the (LEFT) template area,

- hCU, the height of the current CU,

- wCU, the width of the current CU.

Furthermore, CU size adaptations are undertaken. The maximum weight deviation of the sample-based blending (range ∆i) becomes CU-size dependant where bigger CUs (greater than 128 samples) have less deviations. Additionally, location-dependency is forced to 0 for small CU size (width or height less than 8 samples).

Eventually, if one of the two templates is not available, the location-dependency state is set accordingly to the present template. In case of LEFT template presence only, the location-dependency is set to 2 (horizontal), otherwise in case of ABOVE template presence only, the location-dependency is set to 1 (vertical).

## TIMD fusion optimization

JVET-AF0132 [3] proposes an optimization method for reference lines selection in TIMD blending mode.

The modified conditions of reference line determination are as follows:

- If all of the following conditions are true, l+1 is chosen.

- the current block is not ISP block

- both the first mode and second mode are angular prediction mode

- all of the following conditions are false

- abs(predModeIntra1 – predModeIntra2) is greater than Threshold. The value of Threshold is set to 8 or 4.

- (predModeIntra1 - EXT_HOR_IDX) * (predModeIntra2 - EXT_HOR_IDX) is less than 0.

- (predModeIntra1 - EXT_VER_IDX) * (predModeIntra2 - EXT_VER_IDX) is less than 0.

- Otherwise, l is chosen.

# Proposal

It is proposed to merge contributions JVET-AE0137 [2] with JVET-AF0132 [3], both related to TIMD blending aspects.

# Experimental results

The proposed method is implemented on top of the ECM-10.0. Simulations are performed following the common test conditions of EE2 [4]. The simulation results are shown below.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.06% | 0.02% | -0.07% | 100.0% | 97.9% | 99.8% |
| Class A2 | -0.05% | -0.04% | -0.05% | 99.4% | 98.9% | 100.2% |
| Class B | -0.06% | 0.04% | -0.06% | 100.2% | 99.4% | 100.0% |
| Class C | -0.07% | 0.02% | 0.02% | 99.9% | 97.5% | 100.0% |
| Class E | -0.05% | -0.04% | -0.17% | 100.4% | 99.6% | 100.0% |
| Overall | -0.06% | 0.01% | -0.06% | 100.0% | 98.7% | 100.0% |
| Class D | -0.03% | -0.11% | -0.02% | 100.5% | 97.7% | 100.0% |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

Table 1. Performance of the proposed method on top of ECM-10.0 (AI)

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |
| Class B |  |  |  |  |  |  |
| Class C |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |
| Class D |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |

Table 2. Performance of the proposed method on top of ECM-10.0 (RA)

For convenience, performance of JVET-AF0132 ([3]) on top of ECM-10.0 is reported as follows:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.02% | 0.03% | -0.07% | 100.0% | 100.3% | 100.0% |
| Class A2 | -0.01% | -0.08% | -0.06% | 99.8% | 100.1% | 100.0% |
| Class B | -0.03% | 0.01% | -0.19% | 100.0% | 100.4% | 100.0% |
| Class C | -0.05% | -0.01% | 0.05% | 99.6% | 100.6% | 100.0% |
| Class E | -0.04% | -0.08% | -0.22% | 99.7% | 99.5% | 100.0% |
| Overall | -0.03% | -0.02% | -0.10% | 99.8% | 100.3% | 100.0% |
| Class D | -0.01% | 0.00% | -0.17% | 100.0% | 100.9% | 100.0% |
| Class F | -0.03% | -0.11% | -0.05% | 100.0% | 100.8% | 100.0% |
| Class TGM | -0.02% | 0.03% | -0.07% | 100.0% | 100.3% | 100.0% |

Table 3. Performance of JVET-AF0132 on top of ECM-10.0 (AI)

# Conclusion

Results from this contribution demonstrate that the proposed method brings BD-Rate coding gains compared to ECM-10.0. Furthermore, it is observed that the encoding and decoding times are almost the same as the anchor’s. It is recommended to study this method in next round of ECM EE2.

# 5 References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 10 (ECM 10),” JVET-AE2025, July 2023.

- P. Andrivon, M. Blestel, “Non-EE2 : On TIMD Fusion”, JVET-AE0137, July 2023.

- C. Zhou, Z. Ly, “Non-EE2 : Optimization of TIMD blending mode”, JVET-AF0132, October 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AE2017, July 2023.

# 6 Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).