| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0256-v2 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0193 (AHG11: Decoder complexity optimization for NNVC in-loop filter) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Crosscheck Report |  |  |
| Author(s) or Contact(s): | Du Liu | Email: | du.liu@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution reports the inference crosscheck results for JVET-AF0193. It is reported that the crosscheck results match the results in JVET-AF0193.

# Introduction

JVET-AF0193 proposes an encoder only method to skip CTUs to be filtered by NN when the NN filter gain is small. The method computes the squared error between original samples and reconstruction samples and the square error between original samples and NN filtered samples, and then computes the difference between the two errors, which is the filter gain. When the filter gain is smaller than a threshold, NNLF is disabled for the current CTU, and the prmId is set to -1. No extra signalling is needed. The threshold used in the test is 0.01.

# Results

The simulations were run with NnlfOption=4 and NnIntraPred=1. The first simulation is on top of NNVC-6.0 anchor (NN Intra and HOP enabled). The second simulation is on top of NNVC-6.0 unified LOP (NN Intra and LOP enabled).

## On top of HOP

Anchor and test simulations were run in the same cluster environment but not on the same machine. Noise may exist in the encoding and decoding times.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.02% | 0.04% | 0.06% | 95% | 90% |
| Class A2 | 0.08% | 0.12% | 0.19% | 98% | 84% |
| Class B | 0.07% | -0.01% | -0.09% | 102% | 91% |
| Class C | 0.02% | -0.01% | -0.03% | 101% | 100% |
| Class E |  |  |  |  |  |
| Overall | 0.05% | 0.03% | 0.01% | 99% | 92% |
| Class D | 0.00% | -0.01% | -0.01% | 105% | 108% |
| Class F | -0.02% | 0.08% | 0.00% | 101% | 96% |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 HOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | 0.13% | -0.32% | -0.92% | 101% | 90% |
| Class C | -0.01% | -0.63% | -0.44% | 99% | 96% |
| Class E | 0.31% | -1.50% | -2.65% | 104% | 78% |
| Overall | 0.13% | -0.72% | -1.19% | 101% | 89% |
| Class D | 0.01% | -0.74% | -0.27% | 98% | 97% |
| Class F | -0.08% | -0.30% | -0.26% | 100% | 91% |

## On top of LOP

Class C and Class D were crosschecked.

Anchor and test simulations were run in the same cluster environment but not on the same machine. Noise may exist in the encoding and decoding times.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 LOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.02% | -0.03% | -0.04% | 100% | 96% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.02% | -0.05% | 0.00% | 99% | 98% |
| Class F |  |  |  |  |  |

|  | Low-delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-6.0 LOP |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C | -0.11% | -0.43% | -0.62% | 98% | 92% |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D | 0.08% | -1.01% | -0.35% | 101% | 100% |
| Class F |  |  |  |  |  |

# Patent rights declaration(s)

Ericsson does not have any current or pending patent rights relating to the technology described in this contribution.