| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0281-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0139 (EE1-3.1: neural network-based intra prediction with reduced complexity) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check report |  |  |
| Author(s) or Contact(s): | P.Garus, D. Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE1-3.1: neural network-based intra prediction with reduced complexity (JVET-AF0139), in which a reduce complexity nn-based intra prediction method is proposed. Original method of NN-based intra prediction of NNVC is modified with changes to the architecture and updated weight sparsity policy for each model. The method under cross-check reportedly provides the complexity reduction of 62% (4.8 kMAC/pixel, down from 7.8 kMAC/pixel) at the cost of bd-rate penalty of {0.2%, 0.2%, 0.1%} and {0.1%, 0.0%, 0.0%} for AI and RA configurations, respectively.

At the moment of this contribution, the cross-check of training is still ongoing. The cross-checker was able to successfully complete training cycle 1 and finalizing training cycle 2. The proponent provided all required source code, models and description in time. However, due to IT infrastructure setup of the cross-checker, the training of the models has taken longer than expected. The completion of cycle 2 training is expected to be completed and reported by November 10, 2023.

# Introduction

NN-Intra prediction of NNVC, original proposed in JVET-AC0116[2] , utilizes 7 NN-models implemented through fully-connected network architecture. Each model trained independently for the respective block partitioning {4x4, 4x8, 4x16, 4x32, 8x16, 8x16 and 16x16} and utilizes a weight sparsity enforcement mechanism. The number of non-zero weights in the layer returning repIdx is limited to 10% for block size 4x4 and limited to 50% for the remaining block sizes. Block diagram of the cross-checked method is shown in Figure 1 and aspects of cross-checked methods vs the codebase are summarized in Table 1.

Table 1 architecture policy for each neural network in common SW base NNVC and proposed solution (EE1-6).

| Architecture | NN-Intra in common SW base | Proposal |
| --- | --- | --- |
| Number of hidden layers | For 16x16, 3. For the other six neural networks, 2. | Same. |
| Number of neurons per hidden layer | 1216. | For 4x4, 8x4, and 16x16, 1216 for the last hidden layer, and 640 for the other hidden layers. For the other four neural networks, 1216. |
| Biases | For all layers. | Only for the layer returning repIdx and the layer returning grpIdx1 and grpIdx2. |

Figure 1. Prediction of the current w×h block from its context of reference samples via the neural network fh, w . , θh,w, parametrized by θh,w, belonging to the neural network-based intra prediction mode. Here, w=8 and h=4. The dimensions of the context of reference samples, “preprocessing”, “postprocessing”, repIdx, grpIdx1, grpIdx2 are explained in NNVC description document.

# Cross-check results

The training of the proposed method is conducted through 4 sequential cycles.

For the first cycle, initial 7 models are trained over the training data is provided by NNVC6.0 with all nn-tools disabled. The cross-checker conducted the training data preparation on CPU cluster and the training was completed on GPU. With IT infrastructure of the cross-checker, training of initial model has taken more than 14 days, for block sizes 16x16.

The training of the second cycle was ongoing at the moment of this contribution. The completion of cycle 2 training is expected to be completed and reported by November 10, 2023.