| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0282-v1 |
| --- | --- |

| Title: | Crosscheck of JVET-AF0181 (EE1-1.1.4.a: Simplified feature extraction for HOP) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Cross-check report |  |  |
| Author(s) or Contact(s): | D. Rusanovskyy | Email: | dmytror@qti.qualcomm.com |
| Source: | Qualcomm |  |  |

_____________________________

# Abstract

This contribution reports the cross-check results for EE1-1.1.4.a (JVET-AF0181), in which reduce complexity head block is proposed. Compared with NNVC-6.0 HOP anchor (nn intra + hop), cross-checked method is reportedly brings on average {0.24%, 0.86%, 0.52%} and {0.09%, 0.22%, 0.36%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations. The run time ratio reported by the proponent is 97% for encoder and 87% for decoder on average between RA and AI.

Training and inference testing is ongoing. The cross-checker was able to reproduce inference bd-rate results for all completed QP points of AI and RA. The encoder/decoder run-time reported by the cross-checker is in the around 100% for encoder and decoder. However, encoder run-time for Class A1/A2 are matching to that reported by proponents. Training of the EE1-1.1.4.a model shared by the proponent is ongoing and observed behaviour of validation loss functions follows the expected pattern.

# Introduction

To reduce the worst-case inference memory consumption, sub-test EE1-1.1.4.a (JVET-AF0181) proposes to set the stride of convolution as 2 for all input feature extraction layers as shown in Fig. 1 and accordingly remove the stride of 2 in the transition layer. Claimed worst-case inference memory is reduced by 75%, and kMAC/pixel is reduced by 4.2%. The simulation results reported in JVET-AF0181 is shown in Tables 1 and 2 for RA and AI, respectively.

Table 1. Comparative bd-rate results reported by proponents: test vs NNVC6.0 HOP anchor, nnIntra ON.

|  | Test vs NNVC6.0 HOP Anchor |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.13% | 0.68% | -1.33% | 96% | 86% | 0.17% | 0.53% | 1.77% | 95% | 90% |
| Class A2 | 0.31% | 0.73% | 4.81% | 96% | 89% | 0.19% | 0.97% | 1.65% | 98% | 88% |
| Class B | 0.18% | 0.68% | -0.97% | 95% | 87% | -0.03% | -0.09% | -0.56% | 98% | 90% |
| Class C | 0.34% | 1.30% | 0.55% | 95% | 82% | 0.12% | -0.31% | -0.39% | 100% | 89% |
| Class E |  |  |  |  |  | 0.08% | 0.38% | 0.17% | 96% | 85% |
| Overall | 0.24% | 0.86% | 0.52% | 95% | 86% | 0.09% | 0.22% | 0.36% | 98% | 89% |
| Class D | 0.21% | 0.19% | -0.15% | 95% | 85% | 0.10% | 0.29% | 0.85% | 102% | 84% |
| Class F | 0.11% | 0.41% | -0.32% | 92% | 85% | -0.32% | 0.12% | -0.18% | 98% | 88% |

Figure 1. Tested filter architecture (JVET-AF0181).

Compared with NNVC-6.0 HOP anchor (nn intra + hop), cross-checked method is reportedly brings on average {0.24%, 0.86%, 0.52%} and {0.09%, 0.22%, 0.36%} BD-rate changes for {Y, Cb, Cr} under RA and AI configurations. The run time ratio reported by proponent is 97% for encoder and 87% for decoder on average between RA and AI.

# Cross-check of the Inference

Inference testing of models shard by the proponents is ongoing.

Inference results of proponent’s shared models is reported in Table 2.

Table 1. Comparative bd-rate results reported by cross-checker: test vs NNVC6.0 HOP anchor, nnIntra ON.

|  | BD-rate: Test vs nnvc5_NNIn_hop_QC |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | All Intra |  |  |  |  | Random Access |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.17% | 0.53% | 1.77% | 95% |  |  |  |  | 106% | 99% |
| Class A2 | 0.19% | 0.97% | 1.65% | 96% |  |  |  |  |  |  |
| Class B | -0.03% | -0.09% | -0.56% | 101% |  | 0.18% | 0.68% | -0.97% | 91% | 101% |
| Class C | 0.12% | -0.31% | -0.39% | 104% |  | 0.34% | 1.30% | 0.55% | 100% |  |
| Class E | 0.08% | 0.38% | 0.17% | 113% |  |  |  |  |  |  |
| Overall | 0.09% | 0.22% | 0.36% | 100% |  |  |  |  |  |  |
| Class D | 0.10% | 0.29% | 0.85% | 104% |  | 0.21% | 0.19% | -0.15% | 93% |  |
| Class F |  |  |  |  |  |  |  |  |  |  |

Cross-checker was able to reproduce inference bd-rate results for all completed QP points. The encoder/decoder run-time reported by cross-checker is in the range of 95-110% for encoder and 100% for decoder. Encoder run-time for certain Class A are matching to that reported by proponents.

# Cross-check of the training

Training of the EE1-1.1.4.a model shared by the proponent is ongoing. Figure 2 presents current validation L1 losses for the HOP anchor (black line), test ee1-1.1.2.b (orange) and cross-checked method (green). For cross-checked method, the behaviour of validation loss functions and its deviation from the anchor losses follows the expected pattern.

Figure 2. Validation L1 loss function for HOP Anchor (black line), EE1-1.1.2.b (red line) and cross-checked method (green line).