| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, Hannover, DE, 13–20 October 2023 | Document: JVET-AF0291 |
| --- | --- |

| Title: | EE2 related: Encoder time reduction of Test-2.11 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Karam Naser, Fabrice Le Léannec, Philippe Bordes, Franck Galpin, Antoine Robert Saverio Blasi Ivan Zupancic Jani Lainema Chuan Zhou Zhuoyi Lv | Email: | karam.naser@interdigital.com firstname.lastname@nokia.com chuan.zhou@vivo.com |
| Source: | InterDigital, Nokia, Vivo |  |  |

_____________________________

# Abstract

This contribution proposes an encoder-side optimisation of tests in EE2-2.11 tests. It is asserted that this corresponds to a minimal change to the EE2-2.11x branch. The simulation results on AI are as follows for Test-2.11d:

# Introduction

Test-2.11 is a combination of the following subtests:

- Test 2.1: DIMD Merge: When using DIMD Merge, the DIMD information extracted from neighbouring blocks is used to compute the intra prediction for the current block. A new Merged Histogram of Gradients (MHoG) is computed for the current block based on the HoGs of neighbouring blocks. Only neighbouring blocks encoded with DIMD or with DIMD Merge are considered.

- Test 2.2 DIMD with filtered template: When using DIMD with filtered template, the template is filtered using a 3x3 filter operator before obtaining the gradient histogram. In EE2-2.2a, the gradient histogram derivation uses 3x2 and 2x3 gradient operators, respectively. In EE2-2.2b, the same 3x3 Sobel gradient operators as in ECM are used, while using unfiltered samples for gradient computation at positions where no filtered samples are available.

- EE2-2.5b: It is to consider block vectors from neighboring blocks, which are coded by IntraTMP or IBC, and use them to compute predictor candidates for TIMD cost evaluation. Specifically, the TIMD mode selection process, based on template SATD cost, includes additionally prediction obtained using block vectors from both direct neighbors and non-adjacent ones. The modified TIMD process can therefore result in combining regular and IntraTMP/IBC predictions or combining two different IntraTMP/IBC predictions. This enriches the prediction quality of the current TIMD mode which is limited to regular mode. Additionally, IntraCIIP is used, where the intra prediction mode is blended with IntraTMP using CIIP blending method.

It is observed that the EE2-2.11 tests are associated with increased encoding time. The reason behind this increase is that when DIMD merge or DIMD with filtered template are tested, one additional full RDO check is added. Specifically, for DIMD merge, the following piece of code causes additional RDO:

On the other hand, for DIMD with filtered template, the following piece of code is associated with additional RDO:

This contribution proposes removing the additional RDO to keep the encoding time unchanged, while retaining overall gain.

# Proposal

To reduce the encoding time, it is proposed to reduce the number of added RDO. The corresponding change to EE2-2.11x software branch is shown below as a screenshot of the diff file (also attached to this contribution):

# Experimental results

The proposed method is implemented on top EE2.2-11d. The following results are obtained:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 FULL |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.18% | 0.08% | -0.01% | #DIV/0! | 97.7% | 100.3% |
| Class A2 | -0.22% | -0.10% | -0.15% | #DIV/0! | 99.4% | 100.3% |
| Class B | -0.12% | 0.05% | -0.14% | 100.8% | 97.6% | 100.2% |
| Class C | -0.03% | -0.03% | 0.11% | 100.6% | 99.1% | 100.4% |
| Class E | -0.10% | 0.07% | -0.09% | 101.0% | 98.2% | 100.3% |
| Overall | -0.12% | 0.02% | -0.06% | #DIV/0! | 98.3% | 100.3% |
| Class D | 0.01% | 0.11% | -0.08% | 100.8% | 98.2% | 100.4% |
| Class F | -0.14% | 0.08% | -0.07% | 101.3% | 102.8% | 100.3% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! |

For information, the performance of the EE test is shown below:

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.20% | 0.01% | -0.14% | 102.0% | 97.7% | 99.1% |
| Class A2 | -0.24% | -0.11% | -0.11% | 100.5% | 96.1% | 99.5% |
| Class B | -0.18% | -0.04% | -0.12% | 104.7% | 100.5% | 99.9% |
| Class C | -0.13% | 0.06% | -0.03% | 105.9% | 103.5% | 100.0% |
| Class E | -0.20% | 0.05% | -0.06% | 107.3% | 104.4% | 100.0% |
| Overall | -0.19% | -0.01% | -0.09% | 104.2% | 100.6% | 99.7% |
| Class D | -0.09% | -0.01% | -0.12% | 103.8% | 103.7% | 100.0% |
| Class F | -0.13% | 0.10% | -0.03% | 103.6% | 100.6% | 99.9% |
| Class TGM | -0.02% | 0.00% | -0.03% | 101.9% | 99.1% | 99.7% |
|  | Random Access Main 10 |  |  |  |  |  |
|  | Over ECM-10.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.10% | -0.25% | -0.20% | 100.8% | 99.6% | 100.1% |
| Class A2 | -0.07% | -0.28% | -0.13% | 99.6% | 100.7% | 100.1% |
| Class B | -0.11% | 0.07% | -0.21% | 100.6% | 99.6% | 100.0% |
| Class C | -0.06% | -0.18% | -0.26% | 100.8% | 99.6% | 100.2% |
| Class E |  |  |  |  |  |  |
| Overall | -0.09% | -0.13% | -0.20% | 100.5% | 99.8% | 100.1% |
| Class D | -0.04% | 0.01% | -0.32% | 101.9% | 99.8% | 100.3% |
| Class F | -0.04% | 0.06% | 0.17% | 101.7% | 104.3% | 100.1% |
| Class TGM | -0.11% | -0.16% | -0.26% | 97.8% | 98.1% | 100.1% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).