| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, Online, 17-26 January2024 | Document: JVET-AG0041v1 |
| --- | --- |

| Title: | AhG11/AhG14 teleconference |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Report |  |  |
| Author(s) or Contact(s): | E. Alshina, F. Galpin, D. Rusanovskyy | Tel: Email: | elena.alshina@huawei.com franck.galpin@interdigital.com dmytror@qti.qualcomm.com |
| Source: | AhG chairs |  |  |

_____________________________

# Abstract

This document provides notes from teleconference on Neural Network-based Video Coding (NNVC) technologies conducted jointly by AhG11 and AhG14 on November 16, 2023.

# Agenda

- EE1-0, HOP retraining and luma/chroma balance changes.

- NN intra training cross check results.

- integration of some bug fix in NNVC-7.1.

- clarification of algorithm description for Unified Filter usage in JVET-AF2019.

# EE1-1.0 HOP re-training and luma/chroma balance changes

As was agreed during 32rd meeting of JVET this round of EE1 tests will include:

- EE1-1.0: HOP re-training and luma/chroma balance changes (JVET-AF0155, JVET-AF0180), possible tuning from JVET-AF0150, JVET-AF0296

Training conducted by Interdigital and Qualcomm independently.

In this test, original HOP.1 architecture is used.

Training strategy: original HOP stageIII dataset is used. New training strategy is tested:

- Loss function YUV weighting is changed from 6:1:1 to 12:1:1

  - 0.4% Luma gain 4% Chroma drop

- Learning rate from 0.0001 to 0.0002

  - 0.1% Luma, 0.8% Chroma (both gain)

- Batch size 64 to 32

  - almost same performance, but less memory

- ChromaQPOffset +1

  - about 1% Luma gain and 10% Chroma drop (based on study in JVET-AF0155)

Validation loss have been shown. Training Stage III, random initialization. Model integerized. Test results presented all intra full results, random access partial data.

Suggestion for HOP: 12:1:1, learning rate 0.0002, ChromaQPOffset +1, Batch size 32, no change in learning rate scheduler.

Relatively to VVC anchor performance of HOP NNLF only (w/o NN Intra):

12% (Luma) 12% (Chroma) gain (random access cfg).

Model and training scripts are ready, results expected by Nov. 20.

Agreed to be added to NNVC7.1 (call HOP-2) and committed to EE1-0 on Nov. 20. Use NNVC-7.1 HOP-2 as comparison point for EE1 test improving HOP.

# NN-Intra training cross-check

JVET-AF0139 conditionally adopted, awaiting training cross-check. Training cross-check conducted by OPPO.

Compared to the “old” NN-Intra in NNVC-6.0 kMAC/pxl reduction 7.8 to 4.8, performance drop 0.4% (all Intra) and only 0.1% (in random access).

Agreed to be added to NNVC7.1 (call NNIntra-2) but keep old (NNIntra-1) enabled by default.

# NNVC-7.1 bugs

Release candidate for NNVC-7.0 was released on 3rd of November 2023. Bugs reported:

- signalling of scale values changes to original intention (both LOP and HOP filter), performance impact is minor by partial data 0.01%.

- cleaning of code

Agreed: accumulate all bug fixes and release NNVC-7.2 around start of JVET 33rd meeting.

Agreed: use NNVC7.1 as code base and anchor for EE1 and AhG11 proposal (performance is the same as NNVC-7.0 for LOP and NN-Intra).

# Clarification of algorithm description for Unified Filter usage in JVET-AF2019

Code on filter usage aspect does what was agreed, but code quality is low. In description (JVET-AF2019) some parts are missed.

Tencent and Bytedance will work together to clean up code and provide missing parts for algorithm description.

# Software location

https://vcgit.hhi.fraunhofer.de/jvet-af-ee1/VVCSoftware_VTM/-/tree/EE1-X.X to be forked from NNVC-7.1