| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0044-v1 |
| --- | --- |

| Title: | AHG9: Copyright SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, Arianne Hinds, Gilles Teniou | Tel: Email: | {swenger, ahinds, teniou} @global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Proposed for VSEI is an SEI message to convey copyright information in a video stream, to enable marking the video stream with a copyright related statement such as a copyright notice.  Two alternative proposals are offered: free-form st(v) text, and syntax-structured copyright information.  No preference between the two options were indicated by the authors.

# Introduction

An SEI message allowing marking a coded video stream, or parts thereof, with a copyright marker has been part of H.263 Annex W’s “Picture Message” SEI (MTYPE equal to 2).  H.263’s “picture message” SEI consisted basically of scope information, an integer with “purposes” (such as: copyright, close caption, undefined, but also bit-oriented information typically for error resilience purposes) and, for purposes requiring text, something similar to what we call today st(v).  No similar message is currently included in H.26[4..6] or VSEI.

The author has seen, when performing bitstream analysis of competitor implementations many years in the past, H.264 coded video with an associated copyright statement included in an SEI message, squatting on a non-standard codepoint (PayloadType == 0x88).  While such practice may be deplorable to an ITU or MPEG standardization expert, it’s not unheard and is described here in support of our assertion that a standardized copyright SEI is necessary.

Certain system layer specs such as the MPEG-4 file format (and possibly others) include copyright information messages.  In the above-mentioned example, interestingly, the copyright information present in the system layer was different (in year and ownership) from that in the H.264 video elementary stream.  At the time, the author attributed that difference to the fact that the audio associated with the video was a translation of the original content, produced later.  We mention this as an example for scenarios where copyright messages in the system layer only are not sufficient or desirable.

What is needed, therefore, is metadata in the video elementary stream that is marked and reserved for copyright-related information.

# Proposal

In its simplest form, copyright information can be in the form of a free-form st(v) string, which can be used to convey information such as “Copyright Tencent 2023, all rights reserved”.  By convention or through a requirement in the VSEI semantics, the string could alternatively be required to be carry xml, json, or similarly structured information.  The advantage of this approach is that it simplifies automatic processing.  However, the cost in additional bits is substantial.  Insofar, we do not propose a structured string.

Alternatively, the commonly used elements of copyright information—owner, year of creation, licensing/use information—could be coded using VSEI syntax.  While doing so is more efficient than free-form text, it requires decoding to make the full message visible—not a problem for a decoder or a receiving system, but also not intuitively visible from a hex/ascii dump of the stream.  The latter may have certain disadvantages, as it may be easier to prove that a user of the copyright knew or should have known that the content is subject to restrictions.

We believe that both options have merits, and don’t have a preference for either.

## Common syntax in both alternatives

It is proposed to use the cancel_flag and persistence_flag for persistence information.  Doing so is proposed as the source content may be composed of pictures that have different copyright information associated with, independently of how those pictures are arranged in CVSs.

We observe that in countries following the Berne convention, a copyright notice commonly includes four elements:

- The string “Copyright” or the “©” symbol, or a local translation thereof.

- An identification of the owner of the copyright, typically a personal or company name; freeform string st(v)

- The year of creation of the copyrighted material.

- The use/licensing information, such as “all rights reserved”.  Freeform string st(v).

## Copyright SEI as a free-form string

This SEI message consists of the persistence information, along with a free-form string.  We recommend providing non-normative guidance of recommended content for the copyright information in the form of an informative note.

### Copyright Information SEI message syntax

| copyright_information( payloadSize ) { | Descriptor |
| --- | --- |
| ci_cancel_flag | u(1) |
| if( !ci_cancel_flag ) { |  |
| ci_persistence_flag | u(1) |
| ci_information | st(v) |
| } |  |
| } |  |

### Copyright Information SEI message semantics

ci_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous copyright information SEI message in output order that applies to the current layer. ci_cancel_flag equal to 0 indicates that copyright information follows.

ci_persistence_flag specifies the persistence of the copyright information SEI message for the current layer.

ci_persistence_flag equal to 0 specifies that the copyright information applies to the current decoded picture only.

ci_persistence_flag equal to 1 specifies that the copyright information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a copyright information SEI message is output that follows the current picture in output order.

ci_information specifies copyright information that pertains to the picture(s) in the persistence scope defined by ci_cancel_flag and ci_persistence_flag.

Informative note: A typical content of ci_information includes the string “Copyright” or the “©” symbol, information identifying the copyright owner such as a personal or company name, the year of creation of the content according to the Gregorian calendar, and a use/licensing statement such as “all rights reserved”.

## Structured copyright SEI

Here, we consider the following in the design:

- We don’t think we need to place the string “Copyright” or the “©” symbol in the bitstream because the semantics make it unambiguously clear that the SEI message relates to copyright, and therefore, when interpreting it, the string “Copyright” or the letter “©” is to be thought as a header for the remainder of the information.

- As for the year of creation of the copyrighted material: since 1978, and with few exceptions, content is copyright protected for the life of the author plus 70 years.  Extensions of the 70 years are under discussion but not agreed.  A “magic” date in copyright law is the year 1886, when the Berne convention was first signed.  This may be a reasonable first year for content from which copyright could be counted; older work is in the public domain now.  An 8-bit unsigned integer would allow for coding the copyright for material with a creation date in year 2142, which is surely enough for the practical lifetime of the standards currently under development.  Hence u(8) is proposed.  We are flexible for longer integers.  An unoptimized option would be u(12), to be interpreted in year of the Gregorian calendar.

With this, we arrive at the following syntax and semantics.

### Copyright Information SEI message syntax

| copyright_information( payloadSize ) { | Descriptor |
| --- | --- |
| ci_cancel_flag | u(1) |
| if( !ci_cancel_flag ) { |  |
| ci_persistence_flag | u(1) |
| ci_owner | st(v) |
| ci_year_plus1886 | u(8) |
| ci_information | st(v) |
| } |  |
| } |  |

### Copyright Information SEI message semantics

ci_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous copyright information SEI message in output order that applies to the current layer. ci_cancel_flag equal to 0 indicates that copyright information follows.

ci_persistence_flag specifies the persistence of the copyright information SEI message for the current layer.

ci_persistence_flag equal to 0 specifies that the copyright information applies to the current decoded picture only.

ci_persistence_flag equal to 1 specifies that the copyright information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a copyright information SEI message is output that follows the current picture in output order.

ci_owner specifies the owner of the copyright that pertains to the picture(s) in the persistence scope defined by ci_cancel_flag and ci_persistence_flag.

ci_year_plus1886 specifies the year of creation of the the picture(s) in the persistence scope defined by ci_cancel_flag and ci_persistence_flag minus 1886 years.

ci_information specifies copyright information.

Informative note: A typical content of ci_information is a use/licensing statement such as “all rights reserved”.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).