| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0045-v1 |
| --- | --- |

| Title: | AHG9: AI marking SEI |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Stephan Wenger, Arianne Hinds, Gilles Teniou | Email: | swenger, ahinds, teniou @global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The contribution proposes syntax and semantics for an SEI message addressing the purpose of marking one or more coded pictures as created or modified by generative AI.  It follows up on the concerns raised in Hannover related to contribution JVET-AF0145.  As requested in the previous meeting, the subject contribution contains evidence and references to legislative initiatives mandating the marking of content created or modified by generative AI, as well as a short summary why a copyright SEI message would not be an appropriate mechanism to convey AI marking.  Two solutions are proposed: a free-form text SEI marker (comparable what was proposed in JVET-AF0145) and a structured syntax containing fields that are currently envisioned in the legislative processes reviewed by the authors.

# Introduction

The Tencent contribution JVET-AF0145 was presented and discussed in Hannover and referred to for further study.  Several concerns were raised, some of which are addressed in this contribution.  A technical solution is presented which ought to address the concerns as they relate to the AI marking part of JVET-AF0145.

# Problem Statement

We assert that AI marking is rapidly becoming a requirement for video streams which were created or modified by generative AI.

From the Hannover meeting, the authors recall three distinct concerns, two of which are also mentioned in the Hannover meeting report.

1. JVET-AF0145 addressed, in a single SEI message, two only loosely related topics: a) marking of content as generative AI produced, and b) instructing a generative AI to produce content.  It was remarked, in the context of JVET-AF0145 but also of other contributions, that SEI messages ought to have a clearly defined and reasonably narrow scope, so that it’s easy to claim product support of this or that SEI message.  We concur with that notion.  We address that concern in the subject contribution by focussing on the marking of one or more pictures as created or modified by generative AI, both procedurally (through this contribution focussing exclusively on the marking aspect) and technically (by modifying the syntax proposed in JVET-AF0145 for the single marking purpose).

2. A second concern was that the need for generative AI marking was not clear.  We address that concern by providing evidence that regulatory/legislative activities requiring AI marking are well underway in several legislations, with text excepts from EU and US sources.  We also argue that AI marking is necessary at the elementary stream level and with a per picture granularity and exactness.

3. A third concern relates to the interaction of copyright and AI marking.  Our recollection is that the meeting as a whole (and certainly this proponent) were under the impression that we currently have a copyright SEI message.  After investigating, it turns out that we don’t and haven’t since H.263++.  This oversight is addressed in a different contribution by Tencent.  That said, per the meeting report, “It was was [sic!] requested to investigate whether the copyright SEI might be sufficient to signal that a certain part of the video was generated by a specific generative AI.”  We investigated that question with the help of our legal folks and report our conclusion.  In short, copyright and AI marking address different use cases, different audiences, AI marking needs in practice to be machine parsable whereas for copyright markings, a free-form string is likely fine, and AI created content, in many legislations, cannot carry copyright (except maybe copyright inherited from the source material).  We therefore conclude that using a copyright-oriented SEI message to convey AI markings would be actively misleading, and in our opinion, must be avoided.

The following subsection address the two later concerns in some detail.  As for the first concern, please refer to the detailed, narrow proposal outlined in section 3 below.

## Need for generative AI marking

In Hannover, we justified the need of AI marking through an SEI message by referring to legislative or regulatory activities ongoing in many legislations, without providing specifics.  We were asked to provide pointers towards such activity and do so here.  It should be emphasized that the work cited is not the law, nor regulation; they are proposals in the legislative process in the EU and the US.  However, both are reasonably advanced in their respective processes and have comparatively broad political support.

Going out on a limb here, we summarize the marking requirements for generative AI created/modified content strewn out over hundreds of pages of legislative proposals in as follows (and refer to Appendix A for details):

In the EU AI act, content that’s “deep fake” needs to be conspicuously and visibly marked and the identity of the deep faker need to be disclosed.  Other content created or modified by a generative AI for certain defined, well-intended purposes must be “disclos[ing] [of] the existence of such generated or manipulated content in an appropriate clear and visible manner that does not hamper the display of the work.

In the US’ bipartisan Senate proposal, content created by a generative AI needs to have a conspicuous visible marker and, in addition, metadata that identifies the generative AI engine and the date of production.

# Discussion

In this section, we want to address a few questions we consider likely to come up.

Q1: Why marking metadata in the video/elementary stream and not at the system layer?

We believe that we need marking metadata, likely in addition to other less easily removable technology such as watermarking and sample-based content labelling, for each video segment being generated by AI, primarily because there are many scenarios where only one media of a multimedia session gets tinkered with by AI, while other media may stay original.  A very simple example may be what has been shown in Hannover in document JVET-0145—the Panda riding a bicycle.  Clearly, in this example, the associated audio could remain identical and untouched by the AI, hence no marking.  Another example may be that the audio stream is recreated by the AI in a different language.

Q2: Why SEI when “conspicuous marking” may be required?

What may become required in according to all legislative proposals reviewed is conspicuous marking in the video stream, but such marking is not to interfere with the artistic intent of the content.  We think, such a visible marker will likely take the form of a logo—perhaps the dreaded paper clip—included in the source video and hence also in the coded video bitstream.  However, all legislative proposals also require—at least in some cases—textual information and/or dates, in particular things like the identification of the generative AI engine in use, and the date of generation of the content, or the identification of the person instructing the A I engine.  Exposing the user to this information is neither compatible with artistic requirements, nor required by the legislative proposals.  Insofar, metadata seems the right approach.

Q3: Why SEI and not VUI or a new NAL unit type

A new NAL unit type for information like this, when we have SEI and VUI available, seems an overkill.  Conceivably, generative AI could have been involved in use for a subset of the pictures that are coded into a CVS.  Also, the VUI content has been quite stable and focussed on color and sampling etc.  Insofar, of VUI vs. SEI, SEI seems to be the way to go.

Q4: Use the Copyright SEI message?

Overloading an SEI message reserved (by name) for copyright related information is wrong from a legal viewpoint.  Copyright is a term of art.  AI labelling is rapidly becoming a term of art.  Those two terms have nothing to do with each other.  Worse, in at least some legislations, AI cannot create copyrighted material; they do not qualify as authors.  More detailed are provided in Appendix B.

Q5: Should be we have “Legal Stuff” SEI message, to be used for all things like copyright and AI labelling?

We are open to such an idea, but only if we restrict all copyright, AI labelling, and perhaps other “legal” content to free-form strings.  In that case, we could have a design with a “purpose” integer and an st(v) for the notice, following the example in H.263 Annex W (where the free-form string could be, for example, copyright info, but also closed caption).  However, we also think that there may be a value in a structured approach for the various elements that copyright, marking, etc. may require.  In that case, the structures would be different, and differentiating those through an index in a single SEI versus using the SEI type as the demultiplexing point seems wrong, given the discussions we had in Hannover.

# Proposal

We propose two alternative designs.  In the first design, the AI marking information is a free-form string.  This design obviously allows for the highest flexibility when choosing the AI marking language, and is our preference.  As an alternative, we also document how a more structured approach to AI marking could look like.  Likely, such a design, in most cases, would allow for bitrate savings.  However, as different jurisdictions will likely (at least initially) have different requirements for AI marking, and since an ideal syntax would address all of those, we arrive at a relatively complex solution that still needs one free-form text field to be future proof.

## Common syntax in both alternatives

It is proposed to use the cancel_flag and persistence_flag for persistence information.  Doing so is proposed as the source content may be composed of pictures that have different copyright information associated with, independently of how those pictures are arranged in CVSs.

## AI Marking SEI using a free-form string (preferred)

### AI Marking SEI message syntax

| ai_mark( payloadSize ) { | Descriptor |
| --- | --- |
| ai_mark_cancel_flag | u(1) |
| if (!ai_mark_cancel_flag) { |  |
| ai_mark _persistence_flag | u(1) |
| ai_mark_information | st(v) |
| } |  |
| } |  |

### AI Marking SEI message semantics

ai_mark_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous AI marking SEI message in output order that applies to the current layer. Ai_mark_cancel_flag equal to 0 indicates that copyright information follows.

ai_mark_persistence_flag specifies the persistence of the AI mark SEI message for the current layer.

ai_mark_persistence_flag equal to 0 specifies that the AI Marking information applies to the current decoded picture only.

ai_mark_persistence_flag equal to 1 specifies that the AI Marking information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an AI Marking SEI message is output that follows the current picture in output order.

ai_mark_information specifies AI marking information that pertains to the picture(s) in the persistence scope defined by ai_mark_cancel_flag and ai_mark_persistence_flag.

Informative note: the content of ai_mark_information will typically be chosen based on the legal requirements for AI marking as they may exist at the location of the AI engine that generates the AI-based content as well as at the intended place of consumption of the coded video bitstream containing the AI marking SEI message.

## AI Marking SEI using structured syntax (alternative)

### AI Marking SEI message syntax

| ai_mark( payloadSize ) { | Descriptor |
| --- | --- |
| ai_mark_cancel_flag | u(1) |
| if (!ai_mark_cancel_flag) { |  |
| ai_mark _persistence_flag | u(1) |
| ai_tool_id_string | st(v) |
| ai_timestamp_string // RFC 3339 | st(v) |
| ai_person_id_presence_flag | u(1) |
| if ( ai_person_id_presence_flag ) |  |
| ai_person_id | st(v) |
| ai_instruction_presence_flag | u(1) |
| if( ai_instruction_presence_flag ) |  |
| ai_instruction | st(v) |
| ai_base_content_presence_flag | u(1) |
| if ( ai_base_content_presence_flag ). // base content implies modification |  |
| ai_base_content | st(v) |
| ai_other_regulatory | st(v) |
| } |  |
| } |  |

### AI Marking SEI message semantics

ai_mark_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous AI marking SEI message in output order that applies to the current layer. Ai_mark_cancel_flag equal to 0 indicates that copyright information follows.

ai_mark_persistence_flag specifies the persistence of the AI mark SEI message for the current layer.

ai_mark_persistence_flag equal to 0 specifies that the AI Marking information applies to the current decoded picture only.

ai_mark_persistence_flag equal to 1 specifies that the AI Marking information SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with an AI Marking SEI message is output that follows the current picture in output order.

ai_tool_id_string specifies the generative AI tool used to generate content of the coded picture(s) within the scope of the AI Marking SEI message.  If no tool_id were provided, ai_tool_id_string is of zero length.

ai_timestamp_string specifies the date and time when the generative AI process than created the content of the coded picture(s) within the scope of the AI Marking SEI message ended.  ai_tiemstamp_string is variable length string following the format as specified in RFC 3339.

ai_person_id_presence_flag equal to 1 specifies that ai_person_id is present.  ai_person_id_presence_flag equal to 0 specifies that ai_person_id is not present.

ai_person_id, when present, specifies the natural or juridical person that instructed the generative AI to perform the generation of the content of the coded picture(s) within the scope of the AI Marking SEI message.

ai_instruction_present_flag equal to 1 specifies that instruction is present.  ai_instruction_id equal to 0 specifies that ai_instruction is not present.

ai_instruction, when present, specifies the instructions the generative AI engine used in the generation of the content of the coded picture(s) within the scope of the AI Marking SEI message.  ai_instruction is a free-form string that can be, for example, in the form of an URI pointing to a set of instrcutions, or a free-form text containing the instructions itself.  Parsing and interpreting ai_instruction is out of scope of this Recommendation|International Standard.

ai_base_content_present_flag equal to 1 specifies that ai_base_content is present.  ai_base_content_present_flag equal to 0 specifies that ai_base_content is not present.

ai_base_content, when present, specifies the content on which the generative AI engine based its generation of the content of the coded picture(s) within the scope of the AI Marking SEI message.  ai_base_content is a free-form string that can be, for example, in the form of an URI pointing to video (in a form acceptable to the generative AI engine), or a free-form text containing instructions for the AI engine of how to obtain the content.  Parsing and interpreting ai_base_content is out of scope of this Recommendation|International Standard.

ai_other_regulatory is a free-form string and specifies information pertaining to AI marking not covered by the other syntax elements of the AI marking SEI message.  if no such other information is available, ai_other_regulatory shall be of zero length.

# Appendix A: Details of legislative activity

## EU AI act, AI marking requirements

The EU AI act is arguably the first comprehensive legislative framework related to AI in society.  The commission proposed a draft to the parliament in 2022, and in June 2023, the parliament proposed hundreds of suggested amendments.  A good source for both is here: https://www.europarl.europa.eu/doceo/document/TA-9-2023-06-14_EN.html (and scroll down to the Artificial Intelligence Act).  A summary can be found here: https://www.europarl.europa.eu/news/en/headlines/society/20230601STO93804/eu-ai-act-first-regulation-on-artificial-intelligence

Marking requirements are strewn out over the several hundred pages, but most relate to “high risk” AI hardware and software (for example, AI in aviation, rail, law enforcement, and so forth).  These require pre-approval before marketing the AI machine or content by an authorized approver, and the application of the European “CE” label. However, there’s also labelling requirements for the lower risk category “generative AI”.

In the legislative proposal as amended (first link above), amendment 399 says basically that generative AI needs to use marking according to article 52.

Article 52 has many paragraphs, and sub-paragraphs, but an interesting one for our purpose here is para 3 (amendment 486, 487), which read (emphasis added by us):

3.  Users of an AI system that generates or manipulates text, audio or visual content that would falsely appear to be authentic or truthful and which features depictions of people appearing to say or do things they did not say or do, without their consent (‘deep fake’), shall disclose in an appropriate, timely, clear and visible manner that the content has been artificially generated or manipulated, as well as, whenever possible, the name of the natural or legal person that generated or manipulated it. Disclosure shall mean labelling the content in a way that informs that the content is inauthentic and that is clearly visible for the recipient of that content. To label the content, users shall take into account the generally acknowledged state of the art and relevant harmonised standards and specifications.

3a.  Paragraph 3 shall not apply where the use of an AI system that generates or manipulates text, audio or visual content is authorized by law or if it is necessary for the exercise of the right to freedom of expression and the right to freedom of the arts and sciences guaranteed in the Charter of Fundamental Rights of the EU, and subject to appropriate safeguards for the rights and freedoms of third parties. Where the content forms part of an evidently creative, satirical, artistic or fictional cinematographic, video games visuals and analogous work or programme, transparency obligations set out in paragraph 3 are limited to disclosing of the existence of such generated or manipulated content in an appropriate clear and visible manner that does not hamper the display of the work and disclosing the applicable copyrights, where relevant. It shall also not prevent law enforcement authorities from using AI systems intended to detect deep fakes and prevent, investigate and prosecute criminal offences linked with their use.

## Bipartisan US Senate proposal (Schatz/Kennedy AI Labeling Act)

In the US and when compared to Europe, legislative activity on AI is a lot less coordinated.  At least four different bills were brought to the Senate alone, some specific ot their purpose, others more of a framework nature.  Specific to generative AI marking is the bipartisan bill brought by Sen. Schatz and Kennedy https://www.schatz.senate.gov/download/ai-labeling-act-of-2023.

With respect to AI marking of video, the following excerpt appears of particular relevance (emphasize by us):

(A) IN GENERAL.—Each generative artificial intelligence system that, using any means or facility of interstate or foreign commerce, produces image, video, audio, or multimedia AI-generated content shall include on such AI-generated content a clear and conspicuous disclosure that meets the requirements of subparagraph (B).

(B) DISCLOSURE REQUIREMENTS.—A disclosure required under subparagraph (A) shall meet each of the following criteria:

(i) The disclosure shall include a clear and conspicuous notice, as appropriate for the medium of the content, that identifies the content as AI-generated content.

(ii) The output’s metadata information shall include an identification of the content as being AI-generated content, the identity of the tool used to create the content, and the date and time the content was created.

(iii) The disclosure shall, to the extent technically feasible, be permanent or un- able to be easily removed by subsequent users.

Our understanding here is that under B(i) above there’s a need for visual marking, but also under (ii) a need for metadata that includes the identity of the AI engine, and a timestamp.

# Appendix B: Copyright Notices and AI markers, why they are not the same

## Copyright

Whenever an author or performer creates a work (an article, book, video, sound, …), the author, through the act of creation alone, gets the right to restrict others from using this work.  This right to restrict others is known as “Copyright”.    “Using” in copyright law, means things such as distribute, copy, perform, adapt, build upon, etc.

In many countries, copyright can also be registered, which is a process that makes the Government officially aware that the owner owns the copyright.  Doing so has advantages when enforcing copyright but is not required.

Copyright notices commonly take a form such as “© Tencent, 2023, all rights reserved”, or “Copyright Stephan Wenger, 2023, this document is available under the terms of the CC-BY 4.0 license”, and can relate to both registered and unregistered work.  Those notices provide the potential user with information that a) the work is under copyright (and not in the public domain), b) the owner of the copyright, c) the year of creation, and d) terms that the owner sets for the use of the work.  The term “all rights reserved” means that, absent a license obtained from the author, the user cannot use (such as: distribute, copy or make derivative works of) the owner.

Copyright law has been largely harmonized starting in 1886 through the Berne Convention.  It’s fair to say that the basics described above apply to essentially all Berne convention countries, and that there are only a handful of countries left that are not signatories of the Berne contention.

Whether or not AIs can own copyright, or whether work created by the AI is subject to copyright owned by the AI or the person who instructed the AI to create the work, is unsettled law.  In Western countries, the trend is that AIs can neither create nor own copyright, as they are not “persons” in the sense of the law.

## AI Markers

Certain AI related legislation targets the marking of AI generated content by the AI engine.  One technical option for implementing such a marking requirement is the subject of this proposal.  Law and regulation related to marking requirements are, as far as we know, in the drafting stages in Western jurisdictions.  Two examples (US and EU) were provided in Appendix A above.

## Interaction between copyright notices/marking and AI marking

The following table lists the overlap (or better: the  virtual non-existence of overlap) between a copyright notice and an AI marker according to the US (Schatz/Kennedy) and EU (AI Act)

|  | Copyright Notice | AI Marker US | AI Marker EU |
| --- | --- | --- | --- |
| Preamble/Use | “Copyright” or “©” |  |  |
| Owner | Owner |  |  |
| Date | Date of creation | Date of generation; time also required (date of creation is different from date of creation of source content) | Date of generation (date of creation is different from date of creation of source content) |
| AI tool identity |  | AI tool identity |  |
| AI user (person providing instructions) |  |  | Name (deep fake only) |
| Source |  |  | Source (deep fake only) |
| Source copyright |  |  | Source copyright (deep fake only) |

As can be seen in above table, the only overlap in copyright notices and AI marking is a date field, and even that only at first glance.  In that regard, it needs to be emphasized that the date of creation in copyright law is the date when the author created the work, whereas in the AI cases, the date of generation is the date when the AI engine generated its output.  The first one has copyright associated with it, meaning the author holds certain rights.  The second does not.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).