| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0046-v1 |
| --- | --- |

| Title: | Application-required NAL Units |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Gilles Teniou, Stephan Wenger | Email: | (teniou,swenger)[at]global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

The present contribution follows up on JVET-AF0149 from the Hannover meeting by Tencent.  JVET-AF0149v4 was briefly discussed om Hannover but referred to further study, as substantial changes were introduced during the Hannover meeting.  This contribution is substantially similar to option 2, sub-option with req_nu_count_minus1.  It is argued that certain NAL unit types or SEI message types can be ignored by an encoder but are in practice required for an underlying system to work.  Such NAL units or SEI messages are called “required NAL units”.  Homogenous system standards frequently make such NAL units and/or SEI messages mandatory to implement and use in both the sending and receiving system.  If a bitstream designed for such a homogenous system were transported in a heterogenous environment, then middleboxes in the network may not be aware of the required nature of the NAL units and dispose of them.  The contribution argues this problem can be addressed by introducing a prefix NAL unit type that marks n following NAL units as required.

# Problem Statement

This problem statement section is copy-pasted from JVET-AF0149, with minimal editorial changes.

A common service architecture of video delivery includes network elements in charge of content filtering and adaptation. This can be illustrated in the 2 examples below:

First, in conversational services (as shown in the figure 1) from [1] where a multi-point immersive teleconferencing system, as defined in 3GPP TS 26.114 MTSI [2], relies on the Media Resource Function (MRF) as the central Media Control Unit (MCU)

Figure 1: Multi-party immersive conferencing system

In large conference topologies that are supported using MSMTSI (Multi-Stream Multimedia Telephony Service for IMS) as defined in Annex S of the MTSI standard [2], the support for multiple receivers with varying capabilities is provided through a media processing entity in the media path known as the Media Resource Function (MRF). The MRF, in the case of immersive conferencing scenario, can provide viewport-dependent processing 360-degree video to multiple clients and transcoding functionalities.

Another example is related to multimedia streaming in which the video signal crosses several network entities before being received by the end users. This is illustrated in figure 2 from [3].

Figure 2: TV distribution networks and adaptations

In audiovisual distribution scenarios network elements include video content processing functions, storage, and adaptation in the cloud CDNs.

In the 2 above scenarios, some Media Aware Network Entities (MANE) are in charge of interpreting the video signal. The purpose of a MANE may be selective forwarding of parts of the media data to react to network congestions, media switching, media mixing, archival, and similar tasks commonly performed by a service provider rather than an end user.

In case of bandwidth constraints or processing limitations, some NAL Units identified as not relevant for the video decoding may be dropped. In fact, some system standards, especially those that specify the transmission of video over bandwidth-constrained links (e.g., RTP payload format for H.264 [5]), include language suggesting that when facing network congestion requires the network or to drop data from the NAL unit stream. SEI messages, that are, by definition, not required for the decoding of luma or chroma sample data, are usually identified as the prime ones to be discarded when facing network congestion or computation capacity limitations.

However, recent advances in certain fields of video decoding including video coding for machines and neural network based guided post filters make it advisable for a system to forward related SEI messages even in the presence of the above cited limitations.

For audio-visual distribution, the DVB TS 101 154 [4], mandates the support of SEI messages.  For example, in clause 5.14.1.6 of [4] the requirements for HEVC are as follow:

Decoding:	HEVC IRDs shall support the use of Supplemental Enhancement Information of the following message types:

- Picture Timing SEI Message;

- Recovery Point SEI Message;

- "User data registered by Recommendation ITU-T T.35 SEI message" syntactic element [19] user_data_registered_itu_t_t35 as defined in clause B.8a.

Similar requirements for VVC are defined in 5.15.1.8.1

Decoding:	VVC IRDs shall support the use of Supplemental Enhancement Information of the following message types:

- Frame-field information SEI message;

- Alternative transfer characteristics information SEI message.

If sending and receiving systems were operated not in their native environment as specified in the system standard but in a network environment where MANEs are configured to drop SEI messages once congestion occurs, a DVB receiver would identify an interoperability issue potentially leading to a fatal error or a deny of service. Such scenarios may become increasingly important as network convergence progresses.

Also under development in MPEG are certain enhancements of encoder and decoder technologies, for example related to Video Coding for Machines (VCM) or employing guided post filters like a Neural Network based Post Filter (NNPF).  Such systems may employ certain SEI messages under the assumption that they are delivered to the receiving system.

For example, the NNVC SEI message and its related activation SEI message, as present in H.274, guide a neural network based post filter to optimize the decoder output.  If those SEI messages were not received in the NAL unit stream, the result could be less than optimal post-filtered video as the guidance information of the post filter is not available.

Therefore, a mechanism is needed to inform network-based elements that certain SEI messages are required from a receiving system perspective, even if they are not required from a luma/chroma sample decoding perspective.

# Technical Issues raised during the review of JVET-AF0149

In this section we address in more detail the technical questions and issues that were raised in Hannover.

## Use of the Manifest SEI message (or similar SEI message describing other SEI message’s content) for essentiality marking

A general trend observable in the standardization of media transport in places such as the IETF has been, for years, to enable end-to-end encrypted content.  End-to-end encrypted content, however, makes media-aware processing of the content difficult, because a media-aware network element (MANE) cannot “see” syntax elements that are encrypted.  For RTP-based transport, the trend seems to expose certain (small) pieces of information outside of the security context that a MANE can “see” and hence act on.  For the ITU/MPEG-based video technologies, that part is, very generally speaking, the syntax elements present in the NAL unit header: layer and sublayer information, and NAL unit type.  Anything deeper in the video syntax is not visible to the MANE as it is encrypted.  Therefore, we do not believe that the SEI message type information, let alone something as deep syntactically as the manifest SEI would be an appropriate solution.

A secondary, and probably surmountable implementation problem would be that MANEs handle thousands of streams in parallel on a general purpose processor, and hence have stricter computational complexity  restrictions than decoders.

## Error resilience

Copied from JVET-AF0149v4, with the remark that nu_req refers to a NAL unit that precedes required NAL units and marks them as required.

Because such a concern was raised, we comment on the aspect of error resilience as follows:

- Formally and from a narrow video coding standards’ viewpoint, there is no issue as this nu-req NAL unit type is unspecified and hence will be ignored by a decoder.

- If a legacy transmission chain does not understand the nu_req NAL unit type, depending on its design, it will either forward the weird NAL unit, or it will drop it.  If it forwards it, the mechanism works fine further downstream, though that particular transmission chain will not specifically act on it and do what it always did (including potentially dropping required NAL units.  In either case, we are no worse off than we are today.

- If in a modern transmission chain the nu_req is correctly interpreted but NAL unit losses occur, then we can envision the following subcases

  - A nu_req gets lost.  The transmission chain is free to remove the unmarked (through the loss) required NAL unit.  There’s nothing we can do in this case.  However, an encoder is free to include multiple redundant copies of the nu_req in the bitstream to increase the statistical likelihood that at least one of those nu_req messages get through.

  - The required NAL unit following a nu_req gets lost.  In this case, the receiving application would get into trouble, and further, a NAL unit were marked as “required” that’s not really “required in the sense of this design.  From a receiving application’s viewpoint, there’s nothing what we can do except rely on general error resilience design considerations (build for robustness, and worst case re-sync to the stream).  As for the bitstream syntax, and from a standards viewpoint, marking the wrong NAL unit as required does not have any effect, as the decoder throws that information away anyway.

## Irrelevance of the proposed NAL unit for decoder operations

The Hannover meeting report states:

The proposal would only be appropriate if the new NAL unit type would not have any impact on decoder operations (neither legacy decoders than newly defined ones). For the decoder, existing mechanisms such as manifest, prefix, etc. are already defined for the given purpose.

We agree with that statement.

# Proposal

The proposal presented here is substantially the same as the proposal of section 3 in JVET-AF0149, with req_nu_count_minus1 being present.

The required NAL unit mechanism is presented here for VVC.  We may want to consider a similar proposal for the older video codecs H.264 and H.265, as the mechanism could be “bolted on” there without substantial problems.

One NAL unit type that was previously unassigned can be assigned to indicate an empty NAL unit (i.e., a NAL unit with a zero-length RBSP), or a NAL unit with minimal control information in its RBSP, so to indicate that following NAL unit(s), potentially regardless of type, are “required”.  We cannot prescribe here mandatory decoder action on NAL units marked with a Required NAL Unit prefix; this would be a fundamental change in the concepts of optional NAL units, including SEIs.  However, the semantics can say that a NAL Unit marked as “Required”, when dropped from a NAL unit stream through actions of a MANE, would adversely affect the user experience, possibly to a point that the decoded bitstream would be useless for the receiving system.  As one example, at the time of writing, certain TVs and set-top-boxes responsive to DVB-protocol encapsulated HEVC bitstreams reportedly are unable to correctly act on bitstreams lacking the Access Unit Delimiter NAL unit.  That NAL unit is optional in HEVC (and AVC, and VVC), but its use is mandated by DVB specs.  If in a heterogenous transmission system involving, for example, a webrtc feed into a DVB broadcast, the DVB-compliant encoder were to include AUDs, the coupled webrtc-based transmission chain (including MANEs) would drop them for bandwidth or whatever reasons (which they are free to do so, according to their spec), and the webrtc-transported bitstream were fed into a DVB transport for transmission to users, user devices may fail because the webrtc devices have dropped the AUDs.

## Syntax

| req_nu_rbsp( ) { | Descriptor |
| --- | --- |
| req_nu_count_minus1 } | u(8) |

## Semantics

When a REQ_NU NAL unit is present, the immediately following req_nu_count_minus1+1 NAL units are considered as essential for the application.  The reaction of a decoder to these NAL units is unspecified.

From a decoder’s viewpoint, the reception of this NAL unit is a no-op.  In other words, while a smart decoder could possibly take a clue from the fact that an encoder or sending system bothered to put that NAL unit into the bitstream and act accordingly (whatever accordingly may mean), from a standard’s viewpoint, the decoder can ignore it.  That’s why above semantics say “unspecified”.

## Changes in Table 5 (in section 7.4.2.2 NAL unit header semantics)

### 7.4.22	NAL unit header semantics

| Table 5 – NAL unit type codes and NAL unit type classes |  |  |  |
| --- | --- | --- | --- |
| nal_unit_type | Name of nal_unit_type | Content of NAL unit and RBSP syntax structure | NAL unit type class |
| 0 | TRAIL_NUT | Coded slice of a trailing picture or subpicture* slice_layer_rbsp( ) | VCL |
| 1 | STSA_NUT | Coded slice of an STSA picture or subpicture* slice_layer_rbsp( ) | VCL |
| 2 | RADL_NUT | Coded slice of a RADL picture or subpicture* slice_layer_rbsp( ) | VCL |
| 3 | RASL_NUT | Coded slice of a RASL picture or subpicture* slice_layer_rbsp( ) | VCL |
| 4..6 | RSV_VCL_4.. RSV_VCL_6 | Reserved non-IRAP VCL NAL unit types | VCL |
| 7 8 | IDR_W_RADL IDR_N_LP | Coded slice of an IDR picture or subpicture* slice_layer_rbsp( ) | VCL |
| 9 | CRA_NUT | Coded slice of a CRA picture or subpicture* slice_layer_rbsp( ) | VCL |
| 10 | GDR_NUT | Coded slice of a GDR picture or subpicture* slice_layer_rbsp( ) | VCL |
| 11 | RSV_IRAP_11 | Reserved IRAP VCL NAL unit type | VCL |
| 12 | OPI_NUT | Operating point information operating_point_information_rbsp( ) | non-VCL |
| 13 | DCI_NUT | Decoding capability information decoding_capability_information_rbsp( ) | non-VCL |
| 14 | VPS_NUT | Video parameter set video_parameter_set_rbsp( ) | non-VCL |
| 15 | SPS_NUT | Sequence parameter set seq_parameter_set_rbsp( ) | non-VCL |
| 16 | PPS_NUT | Picture parameter set pic_parameter_set_rbsp( ) | non-VCL |
| 17 18 | PREFIX_APS_NUT SUFFIX_APS_NUT | Adaptation parameter set adaptation_parameter_set_rbsp( ) | non-VCL |
| 19 | PH_NUT | Picture header picture_header_rbsp( ) | non-VCL |
| 20 | AUD_NUT | AU delimiter access_unit_delimiter_rbsp( ) | non-VCL |
| 21 | EOS_NUT | End of sequence end_of_seq_rbsp( ) | non-VCL |
| 22 | EOB_NUT | End of bitstream end_of_bitstream_rbsp( ) | non-VCL |
| 23 24 | PREFIX_SEI_NUT SUFFIX_SEI_NUT | Supplemental enhancement information sei_rbsp( ) | non-VCL |
| 25 | FD_NUT | Filler data filler_data_rbsp( ) | non-VCL |
| 26 | REQ_SEI_NUT | Required NAL unit req_nu_rbsp( ) | non-VCL |
| 27 | RSV_NVCL_27 | Reserved non-VCL NAL unit types | non-VCL |
| 28..31 | UNSPEC_28.. UNSPEC_31 | Unspecified non-VCL NAL unit types | non-VCL |

## Changes in section 7.4.2.4.3 Order of Pus and their associated AUs

##### 7.4.2.4.3 Order of PUs and their association to AUs

An AU consists of one or more PUs in increasing order of nuh_layer_id. The order NAL units and coded pictures and their association to PUs are described in clause 7.4.2.4.4.

There can be at most one AUD NAL unit in an AU. When an AUD NAL unit is present in an AU, it shall be the first NAL unit of the AU, and consequently, it is the first NAL unit of the first PU of the AU.

When vps_max_layers_minus1 is greater than 0, there shall be one and only one AUD NAL unit in each IRAP or GDR AU.

There can be at most one OPI NAL unit in an AU. When an OPI NAL unit is present in an AU, it shall be the first NAL unit following the AUD NAL unit, if any, and otherwise shall be the first NAL unit of the AU.

There can be at most one EOB NAL unit in an AU.

When an EOB NAL unit is present in an AU, it shall be the last NAL unit of the AU, and consequently, it is the last NAL unit of the last PU of the AU.

A VCL NAL unit is the first VCL NAL unit of an AU (and consequently the PU containing the VCL NAL unit is the first PU of the AU) when the VCL NAL unit is the first VCL NAL unit of a picture, determined as specified in clause 7.4.2.4.4, and one or more of the following conditions are true:

–	The value of nuh_layer_id of the VCL NAL unit is less than or equal to the nuh_layer_id of the previous picture in decoding order.

–	The value of ph_pic_order_cnt_lsb of the VCL NAL unit differs from the ph_pic_order_cnt_lsb of the previous picture in decoding order.

–	PicOrderCntVal derived for the VCL NAL unit differs from the PicOrderCntVal of the previous picture in decoding order.

Let firstVclNalUnitInAu be the first VCL NAL unit of an AU. The first of any of the following NAL units preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, specifies the start of a new AU:

–	AUD NAL unit (when present),

–	OPI NAL unit (when present),

–	DCI NAL unit (when present),

–	VPS NAL unit (when present),

–	SPS NAL unit (when present),

–	PPS NAL unit (when present),

–	Prefix APS NAL unit (when present),

–	PH NAL unit (when present),

–	Prefix SEI NAL unit (when present),

–	NAL unit with nal_unit_type equal to RSV_NVCL_27 (when present),

–	NAL unit with nal_unit_type in the range of UNSPEC28..UNSPEC29 (when present).

NOTE – The first NAL unit preceding firstVclNalUnitInAu and succeeding the last VCL NAL unit preceding firstVclNalUnitInAu, if any, is one of these types of NAL units.

It is a requirement of bitstream conformance that, when present, the next PU of a particular layer after an EOS NAL unit that belongs to the same layer shall be an IRAP or GDR PU.

##### 7.4.2.4.4 Order of NAL units and coded pictures and their association to PUs

A PU consists of zero or one PH NAL unit, one coded picture, which comprises of one or more VCL NAL units, and zero or more other non-VCL NAL units. The association of VCL NAL units to coded pictures is described in clause 7.4.2.4.5.

When a picture consists of more than one VCL NAL unit, a PH NAL unit shall be present in the PU.

When a VCL NAL unit has sh_picture_header_in_slice_header_flag equal to 1 or is the first VCL NAL unit that follows a PH NAL unit, the VCL NAL unit is the first VCL NAL unit of a picture.

The order of the non-VCL NAL units (other than the AUD, OPI, and EOB NAL units) within a PU shall obey the following constraints:

–	When a PH NAL unit is present in a PU, it shall precede the first VCL NAL unit of the PU.

–	When any DCI NAL units, VPS NAL units, SPS NAL units, PPS NAL units, prefix SEI NAL units, NAL units with nal_unit_type equal to RSV_NVCL_27, or NAL units with nal_unit_type in the range of UNSPEC_28..UNSPEC_29 are present in a PU, they shall not follow the last VCL NAL unit of the PU.

–	When any DCI NAL units, VPS NAL units, SPS NAL units, or PPS NAL units are present in a PU, they shall precede the PH NAL unit (when present) of the PU and shall precede the first VCL NAL unit of the PU.

–	NAL units having nal_unit_type equal to SUFFIX_SEI_NUT, FD_NUT, or RSV_NVCL_27, or in the range of UNSPEC_30..UNSPEC_31 in a PU shall not precede the first VCL NAL unit of the PU.

–	When any prefix APS NAL units are present in a PU, they shall precede the first VCL NAL unit of the PU.

–	When any suffix APS NAL units are present in a PU, they shall follow the last VCL NAL unit of the PU.

–	When an EOS NAL unit is present in a PU, it shall be the last NAL unit among all NAL units within the PU other than other EOS NAL units (when present) or an EOB NAL unit (when present).

–	When a REQ_NU NAL unit is present, the immediately following req_nu_count_minus1+1 NAL units are considered as essential for the application.  The reaction of a decoder to the REQ_NU NAL units is unspecified.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).