| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0049-v1 |
| --- | --- |

| Title: | AHG9: SEI message extension of VVC for computer-generated hologram use |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hiroki Kojima, Keisuke Nonaka, Ryota Koiso, Kei Kawamura, Haruhisa Kato 2-1-15 Ohara, Fujimino-shi, Saitama Japan | Tel: Email: | +81 70-3508-8188 hk-kojima@kddi.com ki-nonaka@kddi.com ry-koiso@kddi.com ki-kawamura@kddi.com ha-katou@kddi.com |
| Source: | KDDI Corp. (KDDI Research, Inc.) |  |  |

_____________________________

- Abstract

This contribution proposes an extension of SEI message on VVC for realizing computer-generated holography use. Transmission of object wave, wich is the intermidiate data representation of computer-generatad hologram, is suitable for the practical use in terms of high computational complexity on object wave generation and the data flexibility of object wave. Thus, we focus on how to transmit object wave by using the existing video codec. In this document, we show the flow of the object wave transmission for computer-generated hologram use and the results of preliminary study of the flow by using the exsisting video codec. Our proposed extension provides the ability to compress the object wave and convert object wave into computer-generated holograms tailored to the playback device, enabling a wide range of use of computer-generated holograms.

Introduction

Computer holography is a promising 3D display technology that realizes highly realistic 3D video experiences by taking a 3D model as input and recording its light propagation information in a digital fringe pattern called a computer-generated hologram (CGH) as shown in Fig. 1. An example of CGH is shown on the web site [1]. This 3D display technology can deliver visual experiences without the burden on the user, e.g. virtual reality sickness, and are not inferior to the real world. Because of these characteristics, computer holography is desired for practical use which requires realistic virtual visual experiences, such as remote meeting, remote coaching and remote surgery.

To realize CGH applications over network, the following problems should be addressed:

- The CGH calculation requires a considerable amount of computation and computing resources, and it is hard to generate CGH by using received 3D model data on a low-performance user terminal such as a mobile device.

- The CGH generation process requires information on the playback device (optical system), making it impossible to broadcast one data to multiple devices.

Sending object wave, which is the intermediate data for CGH calculations, instead of CGH overcomes the above restrictions. The object wave can be adjust to the playback device optics. Thus, transmitting object wave to playback devices solves the problem of data flexibility in a broadcast distribution described in the problem 2. In addition, the computational complexity of conversion from object wave to CGH is much lower than the one of object wave from a 3D model.

The object wave is complex data and consists of two parts: a real part and an imaginary part in the 2D plane. Each part of the data has similar structure to video format. Thus, existing video codecs can be applied by several processing shown in Section 2. In addition, object wave adjustment to the playback device optics can be carried out by using information that used in the object wave calculation. These indicates some supporting information (SEI message) needs to be sent to the playback devices for realization of object wave transmission using existing video codecs.

This document proposes the framework and the SEI message extension of the existing video codec, VVC, for object wave compression that realizes the future CGH use.

Figure 1: Flow for generation and reproduction of computer holography

Holography Overview

Workflow in CGH playback with object wave generation and compression

The workflow of CGH playback, through object wave generation and compression, is shown in Fig. 2 and follows:

- Object wave calculation (Fig. 2 (i)): Object wave is calculated as the light from a 3D model object to a two-dimensional plane by using light wave propagation techniques. The two-dimensional light wave distribution on the target plane in 3D space is obtained as light amplitude and phase distributions. In the usual calculation process, the amplitude and phase are converted to complex numbers.

- Mapping from float to integer (Fig. 2 (ii)): The complex representation of object wave is stored as a floating-point number of the amplitude and phase parts for each sampling point, respectively. In other words, object wave cannot be input to video encoders as it is. Therefore, object wave must be converted to integers to available video format. As mentioned above, an object wave consists of a pair of two-dimensional distribution data of amplitude and phase parts. To treat the pair of the data as a single bitstream, these are packed into one frame (Fig. 2 (b)).

- Encoding and decoding with video codec (Fig. 2 (iii)): The integerized object wave is input to the encoder, and an encoded bit stream is generated. Then, the bitstream is sent to the receiver and the integerized object wave is decoded from the bitstream.

- Inverse mapping from integer to float and unpacking (Fig. 2 (iv)): By using the decoded object wave, we perform an inverse transformation from the integerized object wave to the original floating-point representation. In addition, we unpack the amplitude and phase parts of the packed data for the following operations.

- Adjustment of the object wave to match the optics of the receiver device (Fig. 2 (v)): The resolution and the inverse-mapped object wave is adjusted according to the pixel pitch of the display and the wavelength of the reference light on the playback devices by using the metadata included in SEI. Also, when the object wave located at a midpoint between the 3D model and the playback device is decoded, the object wave on the display is calculated by propagation calculation by using the metadata included in SEI.

- Conversion from object wave to hologram: A CGH is calculated as the interference fringe pattern between the inverse-mapped object wave and the reference light defined by the optics of the playback device. The CGH is displayed by the light modulator on the playback device, and the 3D image is reproduced by irradiating the reference light.

Figure 2: Flow of CGH playback through object wave generation and compression

Preliminary study of object wave compression

Object wave consists of two-dimensional distribution of amplitude and phase parts on complex numbers, which can be treated as a type of video data, e.g., an 8-bit quantized video at each sampling point for each RGB color component. Thus, the object wave can be compressed by VVC. Here, we show the results of a preliminary study on object wave compression by VVC. The detail of this study is as follows.

We use two 3D models named “butterfly” and “checker_ball” as the input for object wave calculation (see Fig. 3). We use WaveField Tools, which is distributed on the website [2] for the object wave calculation, and the calculated object wave is shown in Figs. 4 and 5. Note that the original object wave is represented as the floating-point number for each sampling point, but the figures are the images that have already been integerized for visualization and making it input data of VVC.

Figs. 4 and 5 show the amplitude and phase images of the object wave for each 3D model.

- butterfly                    (b) checker_ball

Figure 3: 3D models for the preliminary study

Amplitude                        Phase

Figure 4: Integerized object wave of the 3D model (butterfly)

Amplitude                        Phase

Figure 5: Integerized object wave of the 3D model (checker_ball)

To confirm the benefits of encoding the consecutive object waves as video with VVC, we conducted coding experiments using VTM-20.0 in both all-intra mode and random-access mode with several QPs (QP=22, 27, 32, 37). The object waves have 8-bit depth with RGB color (4:4:4) format and 4K x 4K resolution for each amplitude and phase parts, respectively, and these parts are finally packed into 4K x 8K format. Since this is the preliminary study to analyze the basic behavior of object wave compression, we conducted coding simulations with 30 frames of object wave for each content.

The simulation of reconstructed images from the object waves are shown in Fig. 7. These images are rendered from three viewpoints, in the front of the object (Center), position 10° off-center to the left (Left 10°), and position 10° off-center to the right (Right 10°). We also use WaveField Tools for the simulation. The images in Fig. 7 show that the object’s appearance changes depending on the viewpoint. It also shows that as the QP increases, there is degradation around the moving parts of the 3D model, such as the wings of the butterfly.

Figure 6: Reconstructed simulation images from the original and compressed the object wave (The 15th frame of “butterfly”). Descriptions at the bottom of the image show the viewpoint position.

We also evaluate the rate-distortion (RD) curve as the objective evaluation. We carried out two experiments measuring the distortion in two data domain; Experiment 1 calculates PSNR between the input object wave and the decoded object wave, and Experiment 2 calculates PSNR between the reconstructed images from the corresponding object waves, as shown in Fig. 9. Figs. 10 and 11 show the RD curve of Experiment 1, and Fig. 12 shows the RD curve of Experiment 2. The PSNRs shown in these figures are calculated by averaging the PSNR of each color channel. We can see that both the object wave and the reconstructed image show the PSNR degradation with decreasing bit rate, just like the existing video. In addition, in any domain, the coding efficiency of random-access is improved compared to all-intra, confirming the effectiveness of encoding continuous object wave with a video codec.

Figure 9: Outline of RD curve calculation

Figure 10: RD curve of Experiment 1 (coding rate vs PSNR of the object wave). The numbers described near the plots indicate QP values (butterfly)

Figure 11: RD curve of Experiment 1 (coding rate vs PSNR of the object wave). The numbers described near the plots indicate QP values (checker_ball)

Figure 12: RD curve of Experiment 2 (coding rate vs PSNR of the reconstructed image). The numbers described near the plots indicate QP values

Syntax and semantics of our proposed SEI message

Our SEI message provides the necessary information to realize the flow of CGH playback shown in Section 2.1. The syntax and semantics are described as follows:

Table 1 Syntax of object wave parameters

| object_wave_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| object_wave_cancel_flag | u(1) |
| if( !object_wave_cancel_flag ) { |  |
| amplitude_max | se(v) |
| amplitude_min | se(v) |
| pixel_pitch_x | ue(v) |
| pixel_pitch_y | ue(v) |
| wavelength_r | ue(v) |
| wavelength_g | ue(v) |
| wavelength_b | ue(v) |
| object_depth | ue(v) |
| object_wave_depth | ue(v) |
| object_wave_persistence_flag | u(1) |
| } |  |
| } |  |

object_wave_cancel_flag equal to 1 indicates that the SEI message cancels the persistence of any previous object wave parameters SEI message in output order that applies to the current layer. object_wave_cancel_flag equal to 0 indicates that film grain modelling information follows.

amplitude_max specifies the maximum value of the amplitude part of the object wave represented in floating point before integerization. This value and amplitude_min are used to convert the decoded integer object wave value to floating-point number.

amplitude_min specifies the minimum value of the real part of the object wave represented in floating point before integerization. This value and amplitude_max are used to convert the decoded integer object wave value to floating-point number.

pixel_pitch_x specifies pixel pitch along x-axis used for object wave calculation on the encoder side. On the decoder side, this value is used to compensate for pixel pitch mismatch between the calculation and the pixel pitch of the display on the playback device.

pixel_pitch_y specifies pixel pitch along y-axis used for object wave calculation on the encoder side. On the decoder side, this value is used to compensate for pixel pitch mismatch between the calculation and the pixel pitch of the display on the playback device.

wavelength_r specifies wavelength of red color used for object wave calculation on the encoder side. On the decoder side, this value and object_depth are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

wavelength_g specifies wavelength of green color used for object wave calculation on the encoder side. On the decoder side, this value and object_depth are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

wavelength_b specifies wavelength of blue color used for object wave calculation on the encoder side. On the decoder side, this value and object_depth are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

object_depth specifies object_depth used for object wave calculation on the encoder side. On the decoder side, this value and the wavelength (wavelength_r, wavelength_g, and wavelength_b) are used to compensate for wavelength mismatch between the calculation and the reference light on the playback device.

object_wave_depth specifies the distance from the plane where the object wave is recorded to the display. It is used in the propagation calculation at the decoder side to obtain the object wave on the display.

object_wave_persistence_flag specifies the persistence of the object wave parameters SEI message for the current layer.

object_wave_persistence_flag equal to 0 specifies that the object wave parameters SEI message applies to the current decoded picture only.

object_wave_persistence_flag equal to 1 specifies that the object wave parameters SEI message applies to the current decoded picture and persists for all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer in an AU associated with a copyright information SEI message is output that follows the current picture in output order.

Conclusion

In this contribution, we propose the SEI message extension on existing video codec, VVC. This extension provides the ability to compress the object wave and convert object wave into computer-generated holograms tailored to the playback devices, enabling a wide range of use of computer-generated holograms.

We propose the following timeline for standardization.

- July 2024: WD

- October 2024: CD

- October 2025: FDIS

References

[1] KDDI Research and Kansai University, “Achieving color animation of holography while maintaining large size and a wide viewing angle, “ KDDI Research, https://www.kddi-research.jp/english/newsrelease/2022/050901.html, accessed on June 2023.

[2] Kyoji Matsushima, “WaveField Tools, “ Kansai University, http://www.laser.ee.kansai-u.ac.jp/wavefieldtools/, accessed on June 2023.

Patent rights declaration(s)

KDDI Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).