| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0051-v1 |
| --- | --- |

| Title: | AHG9: On design for new SEI RBSP and SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hendry Junghak Nam Seung hwan Kim Jaehyun Lim | Email: | dr.hendry@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

# Abstract

This contribution proposed new SEI RBSP and SEI message to be included in the TuC for future extensions of VSEI to overcome both issues of handling large SEI payload size and important / application-required SEI message.

The proposed points are:

- Define a new RBSP for SEI which may be called versatile SEI RBSP. A versatile SEI RBSP contains only one versatile SEI message.

- In addition to containing one versatile SEI message, the bits of the first byte of a versatile RBSP are allocated for signalling information that may be important to be accessible for entities other than the decoder such as MANEs or any other system interfaces. As for now, we may assign one flag to indicate whether the SEI message is important / application-required so that it should not be randomly removed while the rest 7 bits are reserved.

- A versatile SEI message data structure is designed as follow:

  - Payload type

  - Payload data

- A versatile SEI RBSP may be carried in NAL unit that is defined by codec specification. Existing codecs may add two new NAL unit types to carry prefix versatile SEI RBSP and suffix versatile SEI RBSP.

# Introduction

In the 32nd JVET meeting, contributions JVET-AF0148 and JVET-AF0149 presented problems that the current design of SEI message is asserted to have. The asserted issues and discussed possible solutions are as follows:

- The current structure design is not efficient to carry sei with large payload size. It was pointed out that the number of bits needed to signal the payload size grows proportionally to the size of the payload, that is, it needs 8 bits for every 256 bytes of payload. That was asserted not an issue before because SEI messages usually have relatively small payload (i.e., just few bits or bytes). However, recent SEI messages may have significant size of a payload which makes this issue need to be addressed. Discussion on the solution for this issue includes designing a new data structure for SEI message in the payload size is signaled using fixed 16 bits, which is asserted enough for payload up to 64 kilo-bytes.

- SEI messages may be dropped by media-aware network elements (MANEs) during transport from sender to receiver. While SEI messages are usually optional, some applications may rely on them so that the dropping of SEI messages needed by the application at receive side may cause problem. Discussion on the solution for this issue includes several options such as defining a new NAL unit type for SEI that should not be dropped by MANEs when they see it, a new NAL unit that serves as prefix to NAL unit(s) that are required and should not be dropped by MANEs.

# Discussion

## Regarding large payload size

Regarding the issue of large payload size, a new design for SEI data structure so called lsei_message () has been included in TuC for future extensions of VSEI (JVET-AF2032). The syntax table for lsei_message () is shown below.

| lsei_message( ) { | Descriptor |
| --- | --- |
| lsei_position | u(2) |
| lsei_relevance | u(2) |
| lsei_reserved | u(4) |
| lsei_payload_type_byte | u(8) |
| lsei_payload_size_16bits | u(16) |
| lsei_payload( lseiPayloadType, lseiPayloadSize ) |  |
| } |  |

As it can be seen that the idea is simply fixing the signalling of size using a particular fixed number of bits (i.e., 16 bits as for now). In our assertion, this may not be the best solution considering the potential complexity for encoder to have to write down the size first when the payload size is big.

Compared to any other RBSP, to our understanding, only for SEI message we need to first signal the size prior to the payload. This design choice was made because in the past SEI message is anticipated to be small so we can allow multiple SEI messages to be carried with one SEI RBSP which we asserted for efficiency reason. The consequence of such design is we need to signal size prior to the payload of each SEI message included in the RBSP.

For large size SEI, it does not make much sense anymore to have multiple SEI messages in one SEI RBSP. If one large SEI RBSP contains one large SEI message, there is no need for signalling of payload size. Such design would also be good for encoder to avoid potential complexity / delay when handling the generation of such SEIs.

## Regarding important / application-required SEI message

Several solutions have been proposed in JVET-AF0149, which includes assigning new NAL unit types for important / application-required SEI  message or defining a new prefix NAL unit that indicates that NAL units (i.e., may be SEI message or other type of NAL units) that follow the prefix NAL unit are important and should not be dropped. However, these proposed solutions seem to be overkill as NAL unit types are usually limited and better used to indicate more important feature(s).

If our concern is about SEIs that should not be randomly dropped by MANEs, we may use the opportunity of designing the new SEI data structure as we try to address the issue of large SEI payload to address this issue as well using the same solution space. That is, we may include a flag in the beginning of the new SEI RBSP or the new SEI message structure to indicate the importance of the SEI message.

Including importance information in the beginning of the new SEI RBSP or new SEI message structure aligns well with the design suggested in section 2.1 above such that one SEI RBSP contains one SEI message. We may not want to combine an important SEI message with other (i.e., possibly less important) SEI message(s) in the same SEI RBSP.

Considering this, it may be a good solution to design the new SEI message structure and SEI RBSP to cover not only the issue of large payload size but also important / application-required SEI message.

# Proposal

The following are proposed to be included into the TuC for future extensions of VSEI:

- Define a new RBSP for SEI which may be called versatile SEI RBSP. A versatile SEI RBSP contains only one versatile SEI message.

- In addition to containing one versatile SEI message, the bits of the first byte of a versatile RBSP are allocated for signalling information that may be important to be accessible for entities other than the decoder such as MANEs or any other system interfaces. As for now, we may assign one flag to indicate whether the SEI message is important / application-required so that it should not be randomly removed while the rest 7 bits are reserved.

- A versatile SEI message data structure is designed as follow:

  - Payload type

  - Payload data

- A versatile SEI RBSP may be carried in NAL unit that is defined by codec specification. Existing codecs may add two new NAL unit types to carry prefix versatile SEI RBSP and suffix versatile SEI RBSP.

# Text implementation

## Versatile SEI RBSP

Syntax:

| vsei_rbsp( ) { | Descriptor |
| --- | --- |
| vsei_importance | u(1) |
| vsei_reserved | u(7) |
| versatile_sei_message() |  |
| rbsp_trailing_bits( ) |  |
| } |  |

Semantic:

Versatile Supplemental enhancement information RBSP contains information that is not necessary to decode the samples of coded pictures from VCL NAL units. A VSEI RBSP contains one VSEI message.

vsei_importance equal 1 indicates that the versatile SEI message may be an important or required. Vsei_importance equal 0 indicates that the versatile SEI message does not have particular importance.

NOTE – Entity that is aware of this flag may use this information when it needs to make decision whether or not to deliver / drop the verstatile SEI message.

vsei_reserved is reserved for future use and shall be ignored.

## Versatile SEI message

Syntax:

| versatile_sei_message( ) { | Descriptor |
| --- | --- |
| vsei_payload_type_byte | u(8) |
| vsei_payload( vseiPayloadType ) |  |
| } |  |

Semantic:

Versatile Supplemental enhancement information RBSP contains information that is not necessary to decode the samples of coded pictures from VCL NAL units. A VSEI RBSP contains one VSEI message.

Each versatile SEI message consists of the variables specifying the importance and type payloadType of the SEI message payload. Versatile SEI message payloads are specified in XXX.

NOTE – The NAL unit byte sequence containing the SEI message might include one or more emulation prevention bytes (represented by emulation_prevention_three_byte syntax elements).

vsei_payload_type_byte is a byte of the payload type of a versatile SEI message.

# Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).