| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0055-v4 |
| --- | --- |

| Title: | AhG10: CTU-Level Lagrange Multiplier and QP Adaptation for VVC Low-Delay Configuration |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hongwei Guo, Ce Zhu, Lei Luo, Junjie Chen (UESTC) Yongkai Huo, Yutian Liu (Transsion) | Email: | hwguo@uestc.edu.cn eczhu@uestc.edu.cn luolei1010@uestc.edu.cn 202221010107@std.uestc.edu.cn yongkai.huo@transsion.com yutian.liu@transsion.com |
| Source: | University of Electronic Science and Technology of China, Transsion Holdings |  |  |

_____________________________

- Abstract

This contribution proposes an updated version of the method in contribution JVET-AF0089.

This contribution proposes a CTU-level Lagrange multiplier and QP adaptation for VVC low-delay configuration. The proposed method was implemented on VTM-23.0. Compared to the original VTM-23.0, the reported overall PSNR-Y, U, V BD-rate and {EncT, DecT} variations in low-delay B and low-delay P are shown below, respectively.

Over VTM-23.0:

LDB: -3.59%, -4.72%, -3.78%  {103%, 100%}

LDP: -3.44%, -4.47%, -3.05%  {102%, 101%}

It is recommended to adopt this proposed scheme into VTM under low-delay configuration.

Introduction

This contribution proposes an updated version of the method in contribution JVET-AF0089 [3].

VVC employs a hierarchical QP setting at frame-level within a GOP. In comparison to a flat QP setting scheme, the hierarchical QP setting tends to perform better, as it enhances the coding quality of key frames that are frequently referenced. In a similar vein, we propose an optional CTU-level Lagrange multiplier and QP adaptation scheme, aiming to enrich the coding quality of frequently referenced regions within a frame.

Proposal

In this proposal, we first estimate a distortion propagation factor (DPF) for each 32×32 block in the frame to be encoded. Then, a weight coefficient of each CTU is calculated based on the DPFs obtained. Finally, the adaptive Lagrange multiplier and QP for each CTU are determined according to the weight coefficient and frame-level Lagrange multiplier.

Estimating DPFs

The frame to be encoded is first pre-encoded with a 32×32 block integer pixel inter-prediction. The purpose of pre-encoding is to roughly obtain the coding distortion of a block and its motion compensation predicted (MCP) error, which will be used to calculate the DPF of a block. Note that the QP adopted for pre-encoding is set as the highest temporal level QP, and the pre-encoding does not perform both in-loop filter and sample adaptive offset. For B or P frame, if the temporal level of the frame to be encoded is 0 (i.e., the frame is a key frame), the DPF is calculated by:

(1)

where  and  are the coding distortion of a block and its MCP error, both of which are quantified using Mean Squared Error (MSE), and  is the number of frames in the next GOP.

If the temporal level of the frame to be encoded is more than 0 (i.e., the frame is not a key frame), the DPF is calculated by:

(2)

where  is GOP size, and rPOC is relative picture order count (POC) in a GOP, which can be calculated by . Obviously,  represents the number of the subsequent frames in the current GOP.

Furthermore, to mitigate potential inaccuracies in calculating DPFs resulting from scene changes, we implement a threshold mechanism designed to detect such cases.

(3)

(4)

where β represents the introduced threshold, while  stands for the input QP of the encoder.  is the Mean Absolute Error (MAE) of predicted residuals derived from the pre-encoded frame, and  denotes the occurrence of the scene change.

Calculating weight coefficients

Based on the DPFs obtained, the weight coefficient is calculated by:

(5)

(6)

(7)

(8)

where  is the DPF of a block, which is obtained from subsection 2.1.  is the number of blocks in a frame, and  is the number of blocks in the jth CTU.

Lagrange multiplier and QP adaptation

Finally, the Lagrange multiplier and QP of a CTU are determined by:

(9)

(10)

where ,  and are the Lagrange multiplier, weight coefficient and QP of jth CTU in the frame to be encoded, and  is the Lagrange multiplier of the frame to be encoded which is calculated by using the original scheme in VVC.

Comparison with BIM

Block importance mapping (BIM) is proposed in contributions JVET-V0057[4] and JVET-Y0077[5]. It selects individual QP values for each CTU based on its importance to future pictures. Here is a comparison between our method and BIM.

Similarity:

Function: Both methods aim for CTU-level QP adaptation.

Weights determination: Both are based on motion compensation to calculate the weights of different CTUs.

Difference:

Estimation avenue: BIM first determines the delta QP by calculating the motion compensation prediction error and then adjusts the Lagrange multiplier accordingly. Our method, on the other hand, starts with a defined calculation method for the Lagrange multiplier and then adjusts QP accordingly, offering finer granularity in adjusting the Lagrange multiplier.

Motion compensation algorithm: BIM derives its motion compensation through a Motion Compensation Temporal Filter (MCTF), while our method uses pre-encoding.

Modified Frames: Due to BIM’s reliance on MCTF, QP is only modified for frames where the temporal filter is active, i.e., when the Picture Order Count (POC) is divisible by eight. In contrast, our method’s pre-encoding approach allows modification for every frame.

In summary, while both methods share a similar purpose and computational basis, they differ significantly in their specific schemes and algorithms, leading to different implementation effects. Therefore, it is appropriate to regard these two methods as independent QP adaptation approaches.

Results

Performance of the proposed method

Based on the platform in VTM-23.0 [1], coding performance experiment was conducted following the common test conditions defined in [2]. For low-delay B configuration, experimental results are shown in Table 1. The proposed method can obtain -3.59%, -4.72%, and -3.78% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 1. Performance of the proposed method on top of VTM-23.0 (LDB)

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.75% | -3.11% | -3.23% | 103% | 101% |
| Class C | -4.63% | -7.29% | -4.81% | 102% | 100% |
| Class E | -3.59% | -3.99% | -3.32% | 106% | 97% |
| Overall | -3.59% | -4.72% | -3.78% | 103% | 100% |
| Class D | -0.98% | -2.45% | -1.02% | 102% | 101% |
| Class F | -4.73% | -6.52% | -4.11% | 108% | 97% |

For low-delay P configuration, experimental results are shown in Table 2. The proposed method can obtain -3.44%, -4.47%, and -3.05% BD-rate on average for Y, Cb, and Cr components, respectively.

Table 2. Performance of the proposed method on top of VTM-23.0 (LDP)

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.28% | -2.78% | -2.04% | 103% | 100% |
| Class C | -4.76% | -7.01% | -4.32% | 101% | 100% |
| Class E | -3.62% | -3.88% | -3.04% | 103% | 103% |
| Overall | -3.44% | -4.47% | -3.05% | 102% | 101% |
| Class D | -1.10% | -0.68% | -0.48% | 102% | 100% |
| Class F | -5.60% | -6.20% | -3.63% | 109% | 103% |

A bug of compatibility with the original VTM code is fixed. The observed encoding time of the proposed method demonstrates slight increase. The reason is that the pre-encoding exclusively employs integer-pixel inter-prediction utilizing a fixed size of 32×32. The time complexity associated with pre-encoding is negligible compared with the time-consuming full encoding.

Performance of BIM

The performance of BIM over VTM-23.0 is shown in Table 3 and 4. From the comparison, it can be observed that our method slightly increases the encoding time but achieves a more noticeable improvement in BD-rate.

Table 3. Performance of BIM on top of VTM-23.0 (LDB)

|  |  | Low delay B Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -3.21% | -1.35% | -1.65% | 100% | 100% |
| Class C | -3.22% | -0.07% | 0.42% | 99% | 99% |
| Class E | 0.15% | 2.50% | 0.49% | 102% | 101% |
| Overall | -2.37% | 0.04% | -0.42% | 100% | 100% |
| Class D | -1.57% | 1.41% | 1.37% | 100% | 99% |
| Class F | -2.71% | -2.36% | -0.99% | 101% | 100% |

Table 4. Performance of BIM on top of VTM-23.0 (LDP)

|  |  | Low delay P Main10 |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  |  | Over VTM-23.0 |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -2.90% | -1.42% | -1.50% | 99% | 100% |
| Class C | -2.94% | -0.03% | 0.43% | 100% | 100% |
| Class E | -0.41% | 1.65% | 0.83% | 101% | 101% |
| Overall | -2.29% | -0.19% | -0.28% | 100% | 100% |
| Class D | -1.38% | 1.15% | 1.73% | 100% | 100% |
| Class F | -2.90% | -2.27% | -1.10% | 101% | 100% |

Subjective Results

A subjective video quality test was conducted and no compression artifact was observed. Figure 1 provides the comparison of the subjective results using the original scheme, BIM and our method.

Figure 1 - Comparison of the subjective results (BasketballDrill.Q22.jvet10.lb 220)

Conclusion

It has been shown that CTU-level Lagrange multiplier and QP adaptation can provide coding performance benefits compared to the hierarchical QP setting at frame-level. The encoding time of the proposed method is not significantly increased. The proposed scheme can be implemented in the function "EncSlice::precompressSlice( )". It is recommended to adopt this proposed scheme into VTM under low-delay configuration.

References

- VTM-23.0, available at https://vcgit.hhi.fraunhofer.de/jvet/VVCSoftware_VTM/-/releases/VTM-23.0

- F. Bossen, J. Boyce, K. Suehring, X. Li, and V. Seregin, “VTM common test conditions and software reference configurations for SDR video”, JVET-T2010, Meeting by Teleconference, October 2020

- H. Guo, C. Zhu, L. Luo, et al, “AhG10: Lagrange multiplier and QP adaptation at CTU-level for VVC”, JVET-AF0089, Hannover Meeting, October 2023

- P. Wennersten, C. Hollmann, J. Ström, “[AHG10] Block importance mapping”. JVET-V0057, Meeting by Teleconference, April 2021

- P. Wennersten, J. Enhorn, C. Hollmann, J. Ström, “AHG10: Block importance mapping”. JVET-Y0077, Meeting by Teleconference, January 2022

Patent rights declaration(s)

University of Electronic Science and Technology of China may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Shenzhen Transsion Holdings Co., Ltd. (Transsion) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).