| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0059-v1 |
| --- | --- |

| Title: | EE2-1.15: Enhancements on CCP merge for chroma intra coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Hang Huang, Yue Yu, Haoping Yu, Dong Wang (OPPO) Zhipin Deng, Kai Zhang, Li Zhang (Bytedance Inc.) | Tel: Email: | huanghang@oppo.com {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | OPPO Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2-1.15a, 1.15b and 1.15c. These EEs were designed to test JVET-AF0083 [1] which proposes a fusion mode for the CCP merge mode, where the final prediction is generated by a weighted sum between the CCP-merge prediction and the MM-CCCM prediction or the DIMD prediction. A fusion type flag is signalled to indicate whether the MM-CCCM or DIMD is selected. Additionally, CCP merge mode is harmonized with LB-CCP by including the LB-CCP flag in the CCP merge candidate. Three tests are conducted in this EE. In EE2-1.15a, a CCP merge fusion operation with a mode flag is applied to the CCP merge mode, where the final chroma prediction combines the CCP merge prediction with the MM-CCCM prediction or the DIMD prediction. A fusion type flag is signalled to indicate whether the MM-CCCM or DIMD is selected. In EE2-1.15b, the local-boosting cross-component prediction (LBCCP) flag is inherited in the CCP merge mode. In EE2-1.15c, a combination of EE2-1.15a and EE2-1.15b is tested. The impact on coding efficiency and runtimes of the tests over ECM-11.0 is reported as follows {for Y, U, V, EncT, DecT}:

EE2-1.15a:

AI(Y/U/V): -0.02%/-0.55%/-0.61%, EncT 100.5%, DecT 100.7%

RA(Y/U/V):

EE2-1.15b:

AI(Y/U/V): -0.02%/-0.03%/-0.01%, EncT 99.9%, DecT 99.7%

RA(Y/U/V):

EE2-1.15c:

AI(Y/U/V): -0.04%/	-0.59%/	-0.63%, EncT, DecT

RA(Y/U/V):

# Introduction

The methods of cross-component merge mode (CCP merge) and local-boosting cross-component prediction (LB-CCP) have been adopted in ECM-9.0. In the CCP merge mode, a candidate list is created, which includes CCP models collected from the previous blocks coded by CCLM, MMLM, CCCM, GLM, chroma fusion, and CCP merge modes. Then one candidate in the list is selected as the CCP model for the current block. With LB-CCP, the CCP prediction can be filtered, indicated by a flag.

# Proposed Method

Aspect #1: In the proposed CCP-merge fusion method, a fused prediction is generated by a weighted sum of the CCP-merge prediction and either the MM-CCCM prediction or the DIMD prediction. A CCP-merge fusion flag is signalled conditionally under the CCP-merge flag, to indicate whether the fusion mode is applied. A CCP-merge fusion type flag is further signalled if the CCP-merge fusion flag is true, to indicate whether the MM-CCCM prediction or the DIMD prediction is selected and fused with the CCP-merge prediction.

Aspect #2: It is proposed to harmonize CCP merge mode and LB-CCP. The LB-CCP flag could be inherited from a CCP candidate in the CCP merge candidate list.

According to the EE2 description, each of the two aspects should be investigated individually. Specifically, EE2-1.15a is for Aspect #1, EE2-1.15b is for Aspect #2. A combination test of two aspects is listed as EE2-1.15c.

# Experimental results

The proposed methods are implemented on top of ECM-11.0 [2] and tested based on the common test conditions [3]. The simulation results of Test 1.15a-c are summarized in the following Tables.

Result for EE2-1.15a:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.98% | -1.08% | 99.8% | 101.8% | 100.3% | 99.8% |
| Class A2 | -0.08% | -0.56% | -0.60% | 101.1% | 100.8% | 100.5% | 99.8% |
| Class B | 0.00% | -0.66% | -0.76% | 100.6% | 101.2% | 100.8% | 100.0% |
| Class C | 0.01% | -0.29% | -0.28% | 101.2% | 100.3% | 99.6% | 100.0% |
| Class E | 0.00% | -0.29% | -0.33% | 99.8% | 99.4% | 100.0% | 100.0% |
| Overall | -0.02% | -0.55% | -0.61% | 100.5% | 100.7% | 100.3% | 100.0% |
| Class D | -0.03% | -0.20% | -0.30% | 100.6% | 102.5% | 99.9% | 100.0% |
| Class F | 0.02% | -0.41% | -0.23% | 99.7% | 98.8% | 100.0% | 100.0% |

Result for EE2-1.15b:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.12% | 0.02% | 99.5% | 99.5% | 99.9% | 100.0% |
| Class A2 | -0.02% | 0.01% | 0.07% | 99.9% | 100.2% | 99.5% | 100.1% |
| Class B | -0.01% | 0.04% | -0.06% | 99.8% | 99.8% | 100.0% | 100.0% |
| Class C | -0.02% | -0.05% | 0.05% | 100.0% | 99.6% | 100.0% | 100.0% |
| Class E | -0.02% | -0.09% | -0.02% | 99.9% | 99.6% | 100.0% | 100.0% |
| Overall | -0.02% | -0.03% | 0.01% | 99.9% | 99.7% | 99.9% | 100.0% |
| Class D | -0.05% | 0.05% | 0.10% | 99.6% | 97.8% | 100.0% | 100.0% |
| Class F | 0.00% | -0.02% | 0.02% | 98.9% | 99.9% | 100.0% | 100.0% |

Result for EE2-1.15c:

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -1.12% | -1.11% |  |  |  |  |
| Class A2 | -0.10% | -0.54% | -0.54% |  |  |  |  |
| Class B | -0.02% | -0.64% | -0.69% |  |  |  |  |
| Class C | -0.02% | -0.28% | -0.28% |  |  |  |  |
| Class E | -0.03% | -0.45% | -0.58% |  |  |  |  |
| Overall | -0.04% | -0.59% | -0.63% |  |  |  |  |
| Class D | -0.03% | -0.16% | -0.26% |  |  |  |  |
| Class F | 0.03% | -0.34% | -0.09% |  |  |  |  |

# Conclusions

This contribution reports the test results of EE2-1.15. This EE was designed to test the performance of the improvements proposed to the cross-component prediction merge mode. The results show that the gain of the CCP merge mode can be further improved. It is suggested to adopt the proposed methods of CCP merge fusion and LB-CCP flag inheritance to ECM.

# References

- H. Huang, Y. Yu, H. Yu, D. Wang (OPPO), Z. Deng, K. Zhang, L. Zhang (Bytedance), “Non-EE2: Enhancements on CCP merge for chroma intra coding”, JVET-AF0083, Oct. 2023.

- ECM11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AD2017, April, 2023.

# Patent rights declaration (s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).