| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0061-v2 |
| --- | --- |

| Title: | EE2-3.3: Utilizing LFNST/NSPT for inter coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Fan Wang Jonathan Gan Yue Yu Haoping Yu Dong Wang | Tel: Email: | wangfan6@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

This contribution reports the test results of EE2-3.3. In this test, the utilization of LFNST/NSPT for inter coding is tested. On top of ECM-11.0, simulation results are reported as follows:

RA {-0.32% Y, 0.23% U, 0.10% V, 102.1% EncT, 100.2% DecT }

LDB* {-0.87% Y, 0.53% U, 0.37% V, 103.8% EncT, 101.6% DecT }

*Note that in LDB anchor, the intra LFNST/NSPT is enabled.

Introduction

Utilizing LFNST/NSPT for inter coding was proposed in JVET-AF0082 [1]. The proposed method was further investigated in EE2.

In this test, the utilization of LFNST/NSPT for inter coding is tested. For inter coded block, an intra prediction mode is first derived according to the inter prediction block with a DIMD-like process. Then the derived intra prediction mode is used to select an LFNST/NSPT transform set and the transform can be processed with the selected LFNST/NSPT kernel, like in the intra coding process. The signaling of inter LFNST/NSPT index is different from that of intra LFNST/NSPT index.

The focus of this test is to reduce the encoding time while maintaining the coding gain.

The results for LDB configuration will be provided with intra LFNST/NSPT enabled as anchor.

Proposed method

To utilize LFNST/NSPT for inter coding, a virtual intra prediction mode (VIPM) is employed. Each VIPM represents a kind of feature, for example, a specific direction. The VIPM is derived by applying a DIMD-like process, which is also used in MIP and IntraTMP, to the inter prediction block. The horizontal and vertical gradients of pixels inside the prediction block are calculated with Sobel operator and the horizontal and vertical gradients of each pixel are used to derive a direction. The amplitudes of the gradient of each pixel are accumulated for the corresponding direction. The intra prediction mode corresponding to the strongest accumulation is selected as the VIPM. With the VIPM, the transform set of LFNST/NSPT can be derived. For inter coding, the kernel mapping method is the same as that for intra LFNST/NSPT.

The distribution of inter LFNST/NSPT index is different from that of intra LFNST/NSPT index. Therefore the signaling of inter LFNST/NSPT index is proposed to be different from that of intra LFNST/NSPT index. The intra LFNST/NSPT index binarization employs two context coded bins for each symbol, while it is proposed to use truncated unary code with different context models for inter LFNST/NSPT index coding.

In this EE test, encoding fast algorithm similar to inter MTS is applied to reduce the encoding time.

Test results

EE2-3.3 was implemented on top of the ECM-11.0 software [2].

It was tested based on the common test condition [3] in RA configuration.

It was tested based on the common test condition [3] except that LFNST/NSPT is enabled in anchor and test in LDB configuration.

The test results are summarized below.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.27% | 0.06% | 0.06% | 102.4% | 101.6% | 100.1% | 99.8% |
| Class A2 | -0.23% | 0.24% | 0.01% | 101.5% | 99.1% | 100.1% | 100.0% |
| Class B | -0.31% | 0.24% | 0.11% | 102.4% | 100.3% | 100.5% | 100.1% |
| Class C | -0.42% | 0.35% | 0.17% | 102.1% | 100.0% | 99.0% | 100.2% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.32% | 0.23% | 0.10% | 102.1% | 100.2% | 99.9% | 100.0% |
| Class D | -0.31% | 0.50% | 0.30% | 102.5% | 101.4% | 100.0% | 100.2% |
| Class F | -0.21% | 0.10% | -0.02% | 101.9% | 100.0% | 100.2% | 100.1% |
| Class TGM |  |  |  |  |  |  |  |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over anchor |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.76% | 0.83% | 0.48% | 103.7% | 102.5% | 100.2% | 100.1% |
| Class C | -0.98% | 0.34% | 0.68% | 103.7% | 101.5% | 101.0% | 100.3% |
| Class E | -0.91% | 0.28% | -0.24% | 104.0% | 100.1% | 101.5% | 100.0% |
| Overall | -0.87% | 0.53% | 0.37% | 103.8% | 101.6% | 100.8% | 100.1% |
| Class D | -0.65% | 1.80% | 1.43% | 103.2% | 102.8% | 100.8% | 100.2% |
| Class F | -0.56% | -0.17% | -0.05% | 102.9% | 103.5% | 100.2% | 100.4% |
| Class TGM |  |  |  |  |  |  |  |

Conclusions

From EE2-3.3, it is observed that utilizing LFNST/NSPT in inter coding brings -0.32% in RA and -0.87% in LDB BD-Rate reduction with reasonable encoding/decoding time. It is recommended to adopt EE2-3.3.

References

- F. Wang, J. Gan, Y. Yue, H. Yu, D. Wang, “Non-EE2: On LFNST/NSPT for inter coding”, JVET-AF0082, October 2023

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).