| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0063-v1 |
| --- | --- |

| Title: | EE2-1.2 related: AR-BVP for IntraTMP merge candidates |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lai Zhang Yue Yu Fan Wang Haoping Yu Dong Wang | Tel: Email: | zhanglai@oppo.com |
| Source: | OPPO |  |  |

_____________________________

- Abstract

IntraTMP with merge candidates is investigated in EE2-1.2. This contribution proposes to add auto relocated block vector prediction (AR-BVP) to the IntraTMP merge candidate list to further improve the coding efficiency. Specifically, the AR-BVP candidates are inserted after the spatial candidates during the construction of IntraTMP merge candidate list.

On top of ECM11.0, simulation results of the proposed method + EE2-1.2 are reported as below:

The proposed method + EE2-1.2 over ECM-11.0 :

AI: { -0.13% Y, -0.10% U, -0.09% V, xx% EncT, xx% DecT } for Overall

{ -0.39% Y, -0.47% U, -0.10% V, xx% EncT, xx% DecT } for class F

{ -1.07% Y, -1.02% U, -0.99% V, xx% EncT, xx% DecT } for class TGM

RA: { xx% Y, xx% U, xx% V, xx% EncT, xx% DecT } for Overall

{ xx% Y, xx% U, xx% V, xx% EncT, xx% DecT } for class F

{ xx% Y, xx% U, xx% V, xx% EncT, xx% DecT } for class TGM

Introduction

In JVET-AF0137 [1], it was proposed to augment the IntraTMP candidate list with predictors specified by BVs from the neighboring PU’s. Specifically, the block vectors obtained from the merge candidates of the previous blocks coded in IntraTMP or IBC mode are added to the IntraTMP block vectors candidate list (or IntraTMP merge candidates list). The new added block vectors are compared to the existing candidates from intraTMP according to the IntraTMP template matching cost. The candidate list is refined with a second template matching step, which is already used in IntraTMP.

In JVET-AF0129 [2], the auto relocated block vector prediction (AR-BVP) for IBC merge/AMVP candidate list is introduced. AR-BVP can be derived as a guiding BV plus a BV of reference block, allocated by the guiding BV. For example, a guiding block vector BV0, n associated with the current block B0 points to a reference block Bn. If Bn has a BV denoted as BVn, n+1 pointing to a reference block Bn+1, then BV0, n+1, given by BV0, n+1 = BV0, n + BVn, n+1, is defined as the AR-BVP, guided by BV0, n.

Proposal

This contribution proposes to add AR-BVP into the construction of IntraTMP merge candidate list. The AR-BVP candidates are inserted into the IntraTMP merge candidate list according to the following orders:

- Inherited from adjacent neighbors

- Inherited from non-adjacent neighbors

- Constructed AR-BVP from existing candidates

The existing BVs in the list and the constructed AR-BVPs will be used as the guiding block vectors.  For a certain guiding BV, five positions including top-left, top-right, center, bottom-left and bottom-right of reference block are checked to get a BV of reference block and construct AR-BVP. The construction process will end when no new AR-BVP is constructed or the list reaches the maximum size of fifty.

The IntraTMP merge candidate list is further sorted based on template matching cost and the best ten candidates are kept. After this, the merge list will be used in the subsequent template matching process, same as described in EE2-1.2.

Simulation Results

The proposed method was implemented on top of EMC-11.0 [3]. The simulations were performed following the common test conditions [4].

Table 1: Test result of the proposed method + EE2-1.2 over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02% | -0.09% | -0.03% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class A2 | -0.13% | -0.18% | -0.17% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | -0.18% | -0.09% | -0.18% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class C | -0.10% | -0.09% | 0.04% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class E | -0.21% | -0.05% | -0.11% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Overall | -0.13% | -0.10% | -0.09% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | -0.05% | -0.05% | 0.00% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class F | -0.39% | -0.47% | -0.10% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class TGM | -1.07% | -1.02% | -0.99% | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

Conclusions

This contribution proposes to add AR-BVP into the construction of IntraTMP merge candidate list. The results show that the proposed method can improve the coding efficiency of ECM with minor complexity increases. It is suggested to further study this method in EE2.

Reference

- K. Naser, et. al., “AHG12: IntraTMP with Merge Candidates” JVET-AF0137, October 2023

- N. Zhang, et. al., “Non-EE2: Auto relocated block vector prediction” JVET-AF0129, October 2023

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0

- M. Karczewicz, Y. Ye, “Common test conditions and evaluation procedures for enhanced compression tool testing” JVET-AF2017, October 2023

Patent rights declaration(s)

Guangdong OPPO Mobile Telecommunications Corp., Ltd. (OPPO) may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).