| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0069-v1 |
| --- | --- |

| Title: | AhG11: LOP with inputs transformed |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Du Liu, Jacob Ström, Mitra Damghanian, Per Wennersten Torshamnsgatan 23 164 83 Stockholm | Email: | {du.liu, jacob.strom, mitra.damghanian, per.wennersten} @ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

This contribution proposes to apply a 2x2 DCT-II transform and reshaping to the inputs of the LOP model and apply an inverse DCT-II and inverse reshaping to the output of the LOP model. An input of spatial size 144x144 is transformed and reshaped into 72x72x4 if it is luma (needed in Rec, Pred, BS, IPB). If chroma is included in the input, chroma samples are reshaped to 72x72x2 without transform and is concatenated with the transformed luma. If the input is QP, it is directly reshaped to a size of 72x72x1. The complexity in kMACs is reduced, since the spatial resolution inside the network is reduced by a factor of four while the final output still has the same number of pixels. To meet the LOP complexity, it is proposed to further increase the number of backbone blocks and channels. Regeneration of training data and training of the proposed model is needed.

It is stated that the proposed model has a complexity of 16.9 kMAC/pixel and 0.2M parameters (LOP.2: 17 kMAC/pixel 0.05M parameters). The performance (including NNIntra) compared to the NNVC-7.1 anchor (NNIntra+LOP) is reported to be:

Float: RA {-0.48%, -3.94%, -3.51%}, AI {-0.17%, -4.16%, -4.55%}, LDB {-0.21%, -0.03%, 0.53%}

Int16: RA {-0.44%, -3.37%, -3.42%}, AI {-0.18%, -4.07%, -4.57%}, LDB {-0.03%, 0.61%, 1.75%}

It is proposed to study the input-transformed model in the next EE.

# Introduction

At the last meeting, a unified LOP model has been studied and trained in JVET-AF0043 [1]. This model has become the LOP.2 model. The complexity of the LOP.2 model is 17.0 kMAC/pixel, and the number of parameters is 0.05M.

The architecture of the LOP.2 model is shown in Fig. 1 [1]. The inputs include reconstruction samples (Rec), prediction samples (Pred), boundary strength (BS), base QP (QPbase), slice QP (QPslice), and the block IPB information (IPB). The chroma input is upsampled by 2 to align to the spatial dimension of the luma input. There are two branches within the network, one for luma and one for chroma. At the output side, the luma branch has a Pixel Shuffle process to get back to the original spatial dimension of the luma samples to compensate for the downsampling happening in the head.

Fig.1. The architecture of the LOP.2 model.

# Proposed method

In this proposal, it is proposed to apply a 2x2 DCT-II transform and reshaping to the inputs and apply an inverse transform and inverse reshaping to the output. It is asserted that, under the complexity constraint, training the model to improve transform coefficients (rather than to improve luma samples directly) is beneficial to the performance of LOP.

Fig. 2 shows an example of applying the DCT transform to the reconstruction samples and reshaping. Fig. 2(a) shows the current dimension for the reconstruction input in LOP, and Fig. 2(b) shows the placement of DCT and the dimension of the input. In Fig. 2(b), the luma reconstruction input is transformed for each 2x2 subblocks, and each 2x2 subblock is reshaped into four channels corresponding to four DCT coefficients. The input luma of size 144x144x1 is therefore transformed and reshaped into 72x72x4, and 4 represents four DCT frequency channels. For chroma reconstruction samples, because of upsampling of 2, each 2x2 subblock is constant and there is no need to apply transform. Thus, it can be directly downsampled by 2 to get to 72x72x2. The upsampling of 2 in chroma is kept to retain compatibility with the current training, due to that in the training data, the chroma data is already upsampled and saved together with luma. In a more efficient implementation, both upsampling and downsampling can be skipped with identical results.

(a) Current dimension for the reconstruction input in LOP.

(b) The proposed placement of DCT-II and the input dimension.

Fig. 2. An example of applying the transform and reshaping to the reconstruction samples.

Similarly, the inputs “Pred” and “BS” follow the same transform and reshaping and obtain a dimension of 72x72x6. For “QPbase” and “QPslice”, since they are constant, there is no need to apply transform and they are directly reshaped to 72x72x1. For “IPB”, as there is only luma information, it is transformed and reshaped into 72x72x4.

A detailed dimension change through the model in shown in Fig. 3. The spatial dimension of the inputs is 72x72. After downsampling of 2 inside the network, the spatial dimension becomes 36x36. For the luma branch, the dimension is 36x36x16 right before Pixel Shuffle, and after Pixel Shuffle it becomes 72x72x4. Here the four channels represent four DCT frequency channels. Then, an inverse transform and inverse reshaping is applied to get back to the spatial domain of dimension 144x144x1. For the chroma branch, the dimension before the inverse DCT and inverse reshaping is 36x36x(4x2) and afterwards it becomes 72x72x2, where 2 represents the U and V channels.

Fig. 3. The input-transformed model and dimension change through the model.

Since the spatial dimension going through the network is only a quarter of the current LOP, the complexity in terms of kMAC/pixel is reduced to about a quarter as well. This is because the spatial resolution inside the network is reduced by a factor of four (so kMAC goes down) while the final output still has the same number of pixels (the denominator stays the same). The complexity is not exactly a quarter due to that the input channels have a slightly increased number to accommodate the number of DCT frequency channels.

As a result, taking the current LOP without modification except the input and output channels, the complexity is reduced from 17.0 kMAC/pixel to 4.6 kMAC/pixel. The number of parameters is slightly increased from 51248 to 54416, as the architecture has not been simplified.

To further adjust the network to meet the current LOP complexity, the number of channels and the number of backbone blocks for luma and chroma are modified. The proposed adjustment is shown below:

Head

- d1: 12  16

- d2: 8

- d3: 4

- d4: 2

- d5: 2

- d6: 24  48

Backbone

- NY: 14  18

- NUV: 4  7

- C: 16  32

- C1Y: 64  96

- C1UV:  32  48

- C21: 16  32

With this adjustment, the complexity is 16.9 kMAC/pixel and the number of parameters is 208107.

# Training and inference

The training of the input-transformed model is similar to the training of the unified HOP or LOP model, which includes three stages. Training data is generated using the input-transformed model based on NNVC-5.1. However, due to exploration, the models used in stage 2 data generation and stage 3 data generation are different, and they are also different from the final trained model. The following describes the three stages.

- At the first stage, the common DIV2K training data is used. This is the same dataset as stage1 training data for HOP and LOP. The model trained at stage 1 is based on HOP with inputs transformed, but without increased number of backbone blocks or inner channels. The training follows the HOP stage 1 training strategy, using learning rate 1e-4, batch size 64, and weights 6:1:1. The milestones are 30 and 34. The model complexity at stage 1 is 126 kMAC/pixel.

- At the second stage, the model from stage 1 is used for data generation of random-access training data. The model trained at stage 2 is based on HOP with inputs transformed, with reduced number of backbone blocks, branches, and channels. The training is similar to the HOP stage 2 training strategy, using learning rate 1e-4, batch size 64, and weights 6:1:1. The milestones are 15, 17, and 18. The model complexity at stage2 is 18 kMAC/pixel.

- At the third stage, the model from stage 2 is used for data generation of random-access training data. The model trained at stage3 is the proposed model, which is based on LOP with transformed inputs and increased number of backbone blocks and channels. The training is similar to LOP stage 3 training strategy, using learning rate 4e-4 and batch size 128. The milestones are 55, 57, and 58. The luma/chroma weighting is 6:1:1 instead of 12:1:1. The model complexity at stage3 is 16.9 kMAC/pixel.

| Table 1. Network Information for the input-transformed model in training. |  |  |
| --- | --- | --- |
| Network Information in Training Stage |  |  |
| Mandatory | GPU Type | GPU: NVIDIA A100-PCIE-40GB |
|  | Framework: | PyTorch v1.9 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | Stage1: 40 Stage2: 20 Stage3: 60 |
|  | Batch size: | Stage1/2: 64 Stage3: 128 |
|  | Loss function: | L1 and MSE |
|  | Training time: | Stage1: 6 days Stage2: 3 days Stage3: 6.5 days |
|  | Training data information: | DIV2K, BVI-DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | NNVC-5.1 using the input-transformed model |
| Optional |  |  |
|  | Number of iterations |  |
|  | Patch size | 144x144 |
|  | Learning rate: | Stage1: 1e-4, milestones [30, 34] Stage2: 1e-4, milestones [15, 17, 18] Stage3: 4e-4, milestones [55, 57, 58] |
|  | Optimizer: | ADAM |
|  | Preprocessing: |  |
|  | Mini-batch selection process: |  |
|  | Other information: |  |

| Table 4. Network Information for the input-transformed model in inference. |  |  |
| --- | --- | --- |
| Network Information in Inference Stage |  |  |
| Mandatory | HW environment: |  |
|  | GPU Type | N/A |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Total Parameter Number | 208107 |
|  | Parameter Precision (Bits) | fp32, int16 |
|  | Memory Parameter (MB) | fp32: 0.4MB, int16: 0.2MB |
|  | Multiplay Accumulate (MAC) | 16.9 kMAC/pixel |
| Optional |  |  |
|  | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: | 1 |
|  | Patch size | 128x128, 256x256 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

The number of parameters and complexity is consistent between the excel sheet attached in the contribution and the thop library [2].

# Results

The proposed model is tested on top of NNVC-7.1. The changes include the NN model and the corresponding transformed inputs and outputs. The parameter selection and residue scaling are kept unchanged. The results are in comparison to the NNVC CTC [3]. The int16 model is quantized with 12 bits.

## The proposed input-transformed model vs. the LOP anchor, float

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.16% | -6.06% | -5.97% | 106% | 163% |
| Class A2 | -0.85% | -3.88% | -1.87% | 102% | 166% |
| Class B | -0.31% | -3.84% | -4.19% | 105% | 162% |
| Class C | 0.08% | -2.51% | -2.05% | 106% | 163% |
| Class E |  |  |  |  |  |
| Overall | -0.48% | -3.94% | -3.51% | 105% | 164% |
| Class D | 0.60% | -1.42% | -1.19% | 105% | 160% |
| Class F | 0.00% | -1.39% | -0.20% | 109% | 166% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-7.1 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.60% | -5.04% | -6.75% | 100% | 151% |
| Class A2 | -0.30% | -4.28% | -2.34% | 97% | 147% |
| Class B | -0.12% | -4.20% | -5.16% | 97% | 143% |
| Class C | 0.19% | -2.63% | -2.81% | 96% | 133% |
| Class E | -0.15% | -5.15% | -5.83% | 96% | 145% |
| Overall | -0.17% | -4.16% | -4.55% | 97% | 143% |
| Class D | 0.45% | -2.01% | -1.06% | 96% | 129% |
| Class F | 0.04% | -1.18% | -1.85% | 94% | 140% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.46% | 1.34% | -0.72% | 101% | 160% |
| Class C | 0.01% | -1.39% | 2.18% | 105% | 164% |
| Class E | -0.09% | -0.52% | 0.40% | 114% | 163% |
| Overall | -0.21% | -0.03% | 0.53% | 105% | 162% |
| Class D | 0.33% | 1.02% | -0.46% | 101% | 157% |
| Class F | 0.04% | -1.20% | 2.69% | 106% | 152% |

The encoding time and decoding time is not reliable.

## The proposed input-transformed model vs. the LOP anchor, int16

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -1.24% | -5.29% | -5.77% | 94% | 72% |
| Class A2 | -0.76% | -2.93% | -1.67% | 90% | 75% |
| Class B | -0.26% | -3.38% | -4.14% | 90% | 72% |
| Class C | 0.16% | -2.27% | -2.06% | 90% | 73% |
| Class E |  |  |  |  |  |
| Overall | -0.44% | -3.37% | -3.42% | 91% | 73% |
| Class D | 0.61% | -1.21% | -1.29% | 91% | 75% |
| Class F | 0.01% | -1.24% | -0.05% | 89% | 76% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over NNVC-7.1 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.68% | -4.92% | -7.23% | 90% | 69% |
| Class A2 | -0.29% | -4.11% | -2.24% | 92% | 76% |
| Class B | -0.12% | -4.12% | -5.05% | 92% | 76% |
| Class C | 0.18% | -2.62% | -2.86% | 95% | 75% |
| Class E | -0.15% | -5.02% | -5.69% | 93% | 71% |
| Overall | -0.18% | -4.07% | -4.57% | 93% | 74% |
| Class D | 0.43% | -1.87% | -1.11% | 95% | 75% |
| Class F | 0.01% | -1.14% | -1.68% | 92% | 71% |

|  | Low delay B |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 anchor (NNIntra+LOP) |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B | -0.38% | 1.40% | 0.30% | 92% | 76% |
| Class C | 0.10% | -0.64% | 2.16% | 93% | 81% |
| Class E | 0.41% | 0.95% | 3.61% | 84% | 65% |
| Overall | -0.03% | 0.61% | 1.75% | 90% | 74% |
| Class D | 0.44% | 0.35% | 1.88% | 92% | 74% |
| Class F | 0.07% | -1.20% | 3.37% | 89% | 71% |

The encoding time and decoding time is not reliable.

## Supplementary tests

The following tests are provided for additional information. The supplementary tests are run on top of NNVC-7.1 with float models and are compared to the proposed float model (Test in Sec. 4.1).

- Sup. 1: The input-transformed model trained with weights 12:1:1 instead of 6:1:1 at stage 3.

- Sup. 2: The input-transformed model trained with weights 6:1:1 but using the LOP training data at stage 3.

- Sup. 3: The model has inputs and output reshaped as the input-transformed model at stage 3, with increased number of parameters, but without applying the DCT transform.

The encoding time and decoding time is not reliable in the tests.

### Sup. 1: The input-transformed model trained with weights 12:1:1.

It can be observed that using weights 12:1:1 is slightly worse than using 6:1:1 for both luma and chroma under RA.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Proposed float model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.12% | 2.90% | 3.29% | 101% | 108% |
| Class A2 | 0.19% | 1.76% | 1.86% | 105% | 108% |
| Class B | 0.02% | 1.81% | 3.03% | 103% | 108% |
| Class C | -0.01% | 1.82% | 3.00% | 101% | 110% |
| Class E |  |  |  |  |  |
| Overall | 0.07% | 2.02% | 2.84% | 103% | 109% |
| Class D | -0.08% | 1.29% | 2.68% | 102% | 111% |
| Class F | -0.10% | 1.40% | 1.83% | 103% | 102% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over Proposed float model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | -0.02% | 2.08% | 3.68% | 106% | 111% |
| Class A2 | -0.04% | 1.52% | 1.77% | 104% | 109% |
| Class B | -0.10% | 1.58% | 2.68% | 105% | 112% |
| Class C | -0.11% | 1.53% | 2.80% | 103% | 111% |
| Class E | -0.13% | 2.08% | 3.28% | 106% | 111% |
| Overall | -0.08% | 1.73% | 2.82% | 105% | 111% |
| Class D | -0.09% | 1.49% | 2.79% | 103% | 112% |
| Class F | -0.10% | 0.79% | 2.69% | 104% | 110% |

### Sup. 2: The input-transformed model trained with 611 but using the LOP training data.

It can be observed that using the LOP training data gives a loss of 0.29% for luma under RA configuration, which indicates that it is necessary to regenerate RA training data for the input-transformed model. The difference is minor under AI configuration, since the AI training data uses the same DIV2K dataset.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Proposed float model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.36% | 0.09% | -0.78% | 97% | 81% |
| Class A2 | 0.44% | 0.19% | 0.37% | 99% | 82% |
| Class B | 0.27% | 0.34% | 0.25% | 97% | 81% |
| Class C | 0.14% | 0.26% | 0.29% | 97% | 81% |
| Class E |  |  |  |  |  |
| Overall | 0.29% | 0.24% | 0.08% | 98% | 81% |
| Class D | 0.10% | -0.29% | 0.35% | 98% | 80% |
| Class F | 0.07% | 1.00% | 0.79% | 90% | 69% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over Proposed float model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.09% | -0.52% | -0.03% | 100% | 83% |
| Class A2 | 0.02% | -0.33% | -0.15% | 101% | 82% |
| Class B | 0.01% | -0.13% | -0.25% | 100% | 82% |
| Class C | 0.01% | -0.19% | -0.03% | 101% | 83% |
| Class E | -0.01% | -0.74% | -0.48% | 101% | 83% |
| Overall | 0.02% | -0.34% | -0.18% | 100% | 83% |
| Class D | -0.05% | -0.38% | -0.32% | 100% | 85% |
| Class F | 0.02% | 0.56% | 0.72% | 102% | 82% |

### Sup. 3: The input of the model is reshaped as the input-transformed model, with increased number of parameters, but without applying the DCT transform.

It can be observed that there is a loss of 0.16% for luma under RA if the DCT transform is omitted. The difference is minor under AI configuration, since the AI training data uses the same DIV2K dataset.

|  | Random access |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over Proposed float model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.27% | 0.09% | -0.75% | 98% | 81% |
| Class A2 | 0.22% | 0.19% | -0.44% | 101% | 81% |
| Class B | 0.13% | 0.07% | 0.67% | 100% | 84% |
| Class C | 0.09% | -0.07% | 0.00% | 100% | 85% |
| Class E |  |  |  |  |  |
| Overall | 0.16% | 0.06% | -0.01% | 100% | 83% |
| Class D | 0.06% | -0.74% | -0.29% | 102% | 85% |
| Class F | -0.02% | 0.33% | -0.10% | 98% | 82% |
|  | All Intra |  |  |  |  |
|  | BD-rate Over Proposed float model |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT |
| Class A1 | 0.02% | -0.21% | 0.38% | 102% | 84% |
| Class A2 | -0.04% | 0.09% | -0.15% | 106% | 86% |
| Class B | -0.03% | 0.13% | 0.29% | 105% | 87% |
| Class C | -0.01% | 0.06% | 0.15% | 105% | 88% |
| Class E | 0.00% | -0.03% | 0.66% | 105% | 86% |
| Overall | -0.01% | 0.02% | 0.26% | 104% | 86% |
| Class D | -0.01% | 0.11% | -0.07% | 104% | 90% |
| Class F | -0.09% | -0.08% | 0.14% | 106% | 86% |

# Conclusions

This contribution proposes to apply a 2x2 DCT-II transform and reshaping to the inputs of the LOP model and apply an inverse DCT-II and inverse reshaping to the output of the LOP model. This reduces the complexity in kMAC/pixel. In addition, it is also proposed to increase the number of backbone blocks and channels to reach 16.9 kMAC/pixel which is close to the current LOP complexity 17 kMAC/pixel. The number of parameters is increased from 0.05M to 0.2M. The performance compared to NNVC-7.1 anchor shows a BD-rate PSNR impact of -0.48% for luma float and -0.44% for luma int16 under RA configuration. It is proposed to study the method in the next EE.

# References

[1] AhG11/EE1: Status of the joint EE1-0 (LOP.2) training, JVET-AF0043, D. Rusanovskyy, Y.Li, M. Karczewicz, T. Shao, P. Yin, S. McCarthy, J. N. Shingala, A. Shyam, A. Suneja, S. P. Badya, J. Li, Y. Li, C. Lin, K. Zhang, L. Zhang, R. Chang, L. Wang, X. Xu, S. Liu, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[2] https://pypi.org/project/thop/

[3] Common Test Conditions and evaluation procedures for neural network-based video coding technology, E. Alshina, R.-L. Liao, S. Liu, A. Segall, JVET-AF2016, Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).