| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0077-v2 |
| --- | --- |

| Title: | AHG9: On Picture Modality Information |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingying Gao Han Boon Teo Chong Soon Lim Kiyofumi Abe Virginie Drugeon | Tel: Email: | +65-6550-5406 jingying.gao@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

This contribution proposes the following items on the picture modality type included in the technologies under consideration for future extensions of VSEI (version 2), JVET-AF2032.

- Common Aspects:

- Signal minimum and maximum wavelength which indicates the spectral band of optical radiation represented by the pictures in the CLVS, by adding the following syntax elements:

- Add vui_min_wavelength_mantissa and vui_max_wavelength_mantissa to specify the mantissa part of the min and max wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS.

- Add vui_min_wavelength_exponent_plus15 and vui_max_wavelength_exponent_plus15 to specify the exponent part of the max and min wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS.

- The unit of wavelength is specified in meters.

- Add vui_modality_type_extension_bits and vui_reserved_modality_type_extension for future extension of picture modality information.

- Change the type of picture modalities from thermal infrared picture to ultraviolet picture, when vui_modality_type is equal to 3, as in Table 4.

- Add a constraint on vui_colour_description_present_flag:

- When vui_modality_type is equal to 2 or 3, vui_colour_description_present_flag shall be equal to 0.

- Option 1 Aspect:

- Add vui_spectrum_range to specify the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS in a predefined spectrum range table. When vui_spectrum_range is equal to 1, the user is allowed to signal the min and max wavelength as specified in the proposed item 1.

- Add constraints on vui_spectrum_range:

- When vui_modality_type is equal to 1, vui_spectrum_range should be 0, 1, or 7.

- When vui_modality_type is equal to 2, vui_spectrum_range should be 0, 1, or in the range of 8 to 10.

- When vui_modality_type is equal to 3, vui_spectrum_range should be 0, 1, or in the range of 2 to 6.

- Option 2 Aspect:

- Add vui_spectrum_range_present_flag to enable signaling of the min and max wavelength directly, without a predefined spectrum range table. When mantissa is equal to 0 or is not present, it denotes that the wavelength is unspecified.

In this contribution, two syntax designs are proposed. It’s proposed to adopt one of the following options:

- Option 1 Aspect (items 5 and 6) + Common Aspects (items 1 to 4).

- Option 2 Aspect (item 7) + Common Aspects (items 1 to 4).

# 1	Introduction

Today's advanced image sensors, which operate across a variety of spectral ranges, are being integrated into cameras. These advanced image sensors are widely used in applications including autonomous vehicles, UAVs, industrial automation, and more. As illustrated in Figure 1, the picture can be captured from various ranges of spectrum, such as ultraviolet light, visible light, and infrared light. The source of light defines the picture modality type, and pictures of different modality types serve different purposes.

Figure 1: An example of the electromagnetic spectrum.

In the 32nd JVET meeting, signaling the picture modality type in VUI parameters proposed in JVET-AF0147 [1] was adopted into the technologies under consideration for future extensions of VSEI (version 2), with the document number of JVET-AF2032 [2]. It was commented that infrared imaging has many applications, and signaling the source properties at a high level in VVC would be interesting.

We propose to change the type of picture modalities from thermal infrared picture to ultraviolet picture, as shown in Table 4. The thermal infrared picture is captured based on the thermal radiation emitted by objects, which is indeed a type of infrared light. Ultraviolet, along with visible and infrared radiation, falls under optical radiation. This spectrum is widely utilized in imaging to record light that objects emit or reflect.

It was also suggested to signal the wavelength indicating the spectral band of optical radiation during the meeting. Table 1 illustrates the spectral bands for optics and photonics specified in ISO 20473:2007, which cover the spectral range from ultraviolet light to infrared light. In this proposal, the method of signaling wavelength indicating the spectral band of optical radiation represented is addressed, wherein the decimal floating points of wavelength are represented using mantissa and exponent.

For example, the minimum wavelength in meters is derived as follows:

MinWavelength = vui_min_wavelength_mantissa * 10 vui_min_wavelength_exponent_plus15-15

vui_min_wavelength_mantissa specifies the mantissa part of the minimum wavelength. Since the wavelength has up to 3 significant digits, 10 bits are allocated to represent the mantissa part of the minimum wavelength.

vui_min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength. 5 bits are allocated to represent the exponent part of the minimum wavelength. The units of wavelength are specified in meters in the following semantics. The actual exponent may be negative, hence, a bias of 15 is used for the exponent part. The actual exponent ranges from -15 to 16, which is sufficient to represent the entire spectrum range. The bias is calculated as 2(k-1)-1, wherein k represents the length in bits of the exponent.

Table 1: The spectral bands for optics and photonics specified in ISO 20473:2007.

| Designation of the radiation |  | Wavelength (nm) |
| --- | --- | --- |
| Ultraviolet radiation | Extreme UV | 1 to 100 |
|  | Vacuum UV | 100 to 190 |
|  | Deep UV | 190 to 280 |
|  | Mid UV | 280 to 315 |
|  | Near UV | 315 to 380 |
| Visible radiation, light |  | 380 to 780 |
| Infrared radiation | Near IR | 780 to 3 000 |
|  | Mid IR | 3 000 to 50 000 |
|  | Far IR | 50 000 to 106 |

We propose two syntax designs for signaling the spectrum range as follows:

- Option 1: Add vui_spectrum_range to specify the spectrum band, as described in Table 5. Table 5 includes the pre-defined spectrum range referring to standard ISO 20473:2007. When vui_spectrum_range is equal to 1, it allows the signaling of min and max of wavelength in the bitstream.

- Option 2: Add vui_spectrum_range_present_flag to enable signaling of the min and max of wavelength directly, without a predefined spectrum range table as in Table 5.

We also suggest having extension bits to the picture modality information, to allow additional signaling for the picture modality information in the future, by adding the following syntax elements:

- vui_modality_type_extension_bits equal to 0 specifies that vui_reserved_modality_type_extension is not present. vui_modality_type_extension_bits greater than 0 specifies the length, in bits, of vui_reserved_modality_type_extension.

- vui_reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of vui_reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of vui_reserved_modality_type_extension is equal to vui_modality_type_extension_bits.

VUI colour information allows the video decoder to adapt the video output to match the color range and properties of the display device. Since infrared pictures or ultraviolet pictures don't carry color information, VUI colour information is not necessary to be sent when the video is not captured from visible light. It’s proposed to add the following constraint on vui_colour_description_present_flag:

- When vui_modality_type is equal to 2 or 3, vui_colour_description_present_flag shall be equal to 0.

# 2	Proposal

## Option 1 Aspect + Common Aspects

### 7.2	VUI parameters syntax

| vui_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| vui_progressive_source_flag | u(1) |
| vui_interlaced_source_flag | u(1) |
| vui_non_packed_constraint_flag | u(1) |
| vui_non_projected_constraint_flag | u(1) |
| vui_aspect_ratio_info_present_flag | u(1) |
| if( vui_aspect_ratio_info_present_flag ) { |  |
| vui_aspect_ratio_constant_flag | u(1) |
| vui_aspect_ratio_idc | u(8) |
| if( vui_aspect_ratio_idc = = 255 ) { |  |
| vui_sar_width | u(16) |
| vui_sar_height | u(16) |
| } |  |
| } |  |
| vui_overscan_info_present_flag | u(1) |
| if( vui_overscan_info_present_flag ) |  |
| vui_overscan_appropriate_flag | u(1) |
| vui_colour_description_present_flag | u(1) |
| if( vui_colour_description_present_flag ) { |  |
| vui_colour_primaries | u(8) |
| vui_transfer_characteristics | u(8) |
| vui_matrix_coeffs | u(8) |
| vui_full_range_flag | u(1) |
| } |  |
| vui_chroma_loc_info_present_flag | u(1) |
| if( vui_chroma_loc_info_present_flag ) { |  |
| if( vui_progressive_source_flag && !vui_interlaced_source_flag ) |  |
| vui_chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| vui_chroma_sample_loc_type_top_field | ue(v) |
| vui_chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) { |  |
| vui_modality_info_present_flag | u(1) |
| if(vui_modality_info_present_flag) { |  |
| vui_modality_type | u(8) |
| vui_spectrum_range | u(8) |
| if( vui_spectrum_range = = 1 ) { |  |
| vui_min_wavelength_mantissa | u(10) |
| vui_max_wavelength_mantissa | u(10) |
| vui_min_wavelength_exponent_plus15 | u(5) |
| vui_max_wavelength_exponent_plus15 | u(5) |
| } |  |
| vui_modality_type_extension_bits | ue(v) |
| if(vui_modality_type_extension_bits > 0 ) |  |
| vui_reserved_modality_type_extension | u(v) |
| } |  |
| } |  |
| vui_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| vui_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

### 7.3	VUI parameters semantics

vui_colour_description_present_flag equal to 1 specifies that vui_colour_primaries, vui_transfer_characteristics, and vui_matrix_coeffs are present. vui_colour_description_present_flag equal to 0 specifies that vui_colour_primaries, vui_transfer_characteristics, and vui_matrix_coeffs are not present.

When vui_modality_type is equal to 2 or 3, vui_colour_description_present_flag shall be equal to 0.

vui_modality_info_present_flag equal to 1 specifies that vui_modality_type is present in the VUI parameters. vui_modality_info_present_flag equal to 0 specifies that vui_modality_type is not present in the VUI parameters.

vui_modality_type indicates the type of modality of the decoded picture as specified in Table 4. When not present, the value of vui_modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification.

Table 4– Mapping of vui_modality_type to the type of picture modalities

| vui_modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Thermal Infrared Picture Ultraviolet Picture |
| 4..255 | Reserved for future use |

vui_spectrum_range indicates the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS as specified in Table 5. When not present, the value of vui_spectrum_range is inferred to be equal to 0, denoting that the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification. The value of vui_spectrum_range shall be in the range of 0 to 10, inclusive, in bitstreams conforming to this version of this document. The values of 11 to 255, inclusive, for vui_spectrum_range are reserved for future use by ITU-T | ISO/IEC. Although the value of vui_spectrum_range is required to be in the range of 0 to 10, inclusive, in this version of this document, decoders shall also allow values of vui_spectrum_range in the range of 11 to 255, inclusive, to appear in the syntax. Decoders conforming to this version of this document shall ignore the bits that follow a value of vui_spectrum_range in the range of 11 to 255, inclusive, in the VUI parameters.

When vui_modality_type is equal to 1, vui_spectrum_range should be 0, 1, or 7.

When vui_modality_type is equal to 2, vui_spectrum_range should be 0, 1, or in the range of 8 to 10.

When vui_modality_type is equal to 3, vui_spectrum_range should be 0, 1, or in the range of 2 to 6.

Table 5– Definition of vui_spectrum_range

| Value | minimum wavelength (m) | maximum wavelength (m) | Informative remark |
| --- | --- | --- | --- |
| 0 | Unspecified |  | Image characteristics are unknown or are determined by the application. |
| 1 | User-defined |  | Image characteristics are defined by the user. |
| 2 | 1*10 -9 | 1*10 -7 | Extreme UV ISO 20473:2007 |
| 3 | 1*10 -7 | 19*10 -8 | Vacuum UV ISO 20473:2007 |
| 4 | 19*10 -8 | 28*10 -8 | Deep UV ISO 20473:2007 |
| 5 | 28*10 -8 | 315*10 -9 | Mid UV ISO 20473:2007 |
| 6 | 315*10 -9 | 38*10 -8 | Near UV ISO 20473:2007 |
| 7 | 38*10 -8 | 78*10 -8 | Visible radiation, light ISO 20473:2007 |
| 8 | 78*10 -8 | 3*10 -6 | Near IR ISO 20473:2007 |
| 9 | 3*10 -6 | 5*10 -5 | Mid IR ISO 20473:2007 |
| 10 | 5*10 -5 | 1*10 -3 | Far IR ISO 20473:2007 |
| 11..255 | For future use by ITU-T ¦ ISO/IEC |  |  |

vui_min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_min_wavelength_mantissa is equal to 0, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification

vui_max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_max_wavelength_mantissa is equal to 0, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification.

vui_min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_min_wavelength_mantissa is equal to 0, decoders shall ignore the value of vui_min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is derived as follows:

MinWavelength = vui_min_wavelength_mantissa * 10 vui_min_wavelength_exponent_plus15-15

vui_max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_max_wavelength_mantissa is equal to 0, decoders shall ignore the value of vui_max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is derived as follows:

MaxWavelength = vui_max_wavelength_mantissa * 10 vui_max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

vui_modality_type_extension_bits equal to 0 specifies that vui_reserved_modality_type_extension is not present. vui_modality_type_extension_bits greater than 0 specifies the length, in bits, of vui_reserved_modality_type_extension.

The value of vui_modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for vui_modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of vui_modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

vui_reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of vui_reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of vui_reserved_modality_type_extension is equal to vui_modality_type_extension_bits.

vui_payload_bit_equal_to_one shall be equal to 1.

vui_payload_bit_equal_to_zero shall be equal to 0.

## Option 2 Aspect + Common Aspects

### 7.2	VUI parameters syntax

| vui_parameters( payloadSize ) { | Descriptor |
| --- | --- |
| vui_progressive_source_flag | u(1) |
| vui_interlaced_source_flag | u(1) |
| vui_non_packed_constraint_flag | u(1) |
| vui_non_projected_constraint_flag | u(1) |
| vui_aspect_ratio_info_present_flag | u(1) |
| if( vui_aspect_ratio_info_present_flag ) { |  |
| vui_aspect_ratio_constant_flag | u(1) |
| vui_aspect_ratio_idc | u(8) |
| if( vui_aspect_ratio_idc = = 255 ) { |  |
| vui_sar_width | u(16) |
| vui_sar_height | u(16) |
| } |  |
| } |  |
| vui_overscan_info_present_flag | u(1) |
| if( vui_overscan_info_present_flag ) |  |
| vui_overscan_appropriate_flag | u(1) |
| vui_colour_description_present_flag | u(1) |
| if( vui_colour_description_present_flag ) { |  |
| vui_colour_primaries | u(8) |
| vui_transfer_characteristics | u(8) |
| vui_matrix_coeffs | u(8) |
| vui_full_range_flag | u(1) |
| } |  |
| vui_chroma_loc_info_present_flag | u(1) |
| if( vui_chroma_loc_info_present_flag ) { |  |
| if( vui_progressive_source_flag && !vui_interlaced_source_flag ) |  |
| vui_chroma_sample_loc_type_frame | ue(v) |
| else { |  |
| vui_chroma_sample_loc_type_top_field | ue(v) |
| vui_chroma_sample_loc_type_bottom_field | ue(v) |
| } |  |
| } |  |
| if( more_data_in_payload( ) ) { |  |
| if( payload_extension_present( ) ) { |  |
| vui_modality_info_present_flag | u(1) |
| if(vui_modality_info_present_flag) { |  |
| vui_modality_type | u(8) |
| vui_spectrum_range_present_flag | u(1) |
| if( vui_spectrum_range_present_flag = = 1 ) { |  |
| vui_min_wavelength_mantissa | u(10) |
| vui_max_wavelength_mantissa | u(10) |
| vui_min_wavelength_exponent_plus15 | u(5) |
| vui_max_wavelength_exponent_plus15 | u(5) |
| } |  |
| vui_modality_type_extension_bits | ue(v) |
| if(vui_modality_type_extension_bits > 0 ) |  |
| vui_reserved_modality_type_extension | u(v) |
| } |  |
| } |  |
| vui_payload_bit_equal_to_one /* equal to 1 */ | f(1) |
| while( !byte_aligned( ) ) |  |
| vui_payload_bit_equal_to_zero /* equal to 0 */ | f(1) |
| } |  |
| } |  |

### 7.3	VUI parameters semantics

vui_colour_description_present_flag equal to 1 specifies that vui_colour_primaries, vui_ _characteristics, and vui_matrix_coeffs are present. vui_colour_description_present_flag equal to 0 specifies that vui_colour_primaries, vui_transfer_characteristics, and vui_matrix_coeffs are not present.

When vui_modality_type is equal to 2 or 3, vui_colour_description_present_flag shall be equal to 0.

vui_modality_info_present_flag equal to 1 specifies that vui_modality_type is present in the VUI parameters. vui_modality_info_present_flag equal to 0 specifies that vui_modality_type is not present in the VUI parameters.

vui_modality_type indicates the type of modality of the decoded picture as specified in Table 4. When not present, the value of vui_modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification.

Table 4– Mapping of vui_modality_type to the type of picture modalities

| vui_modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Thermal Infrared Picture Ultraviolet Picture |
| 4..255 | Reserved for future use |

vui_spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS is present in the VUI parameters. vui_spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the CLVS is not present in the VUI parameters.

vui_min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification

vui_max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is unknown or unspecified or determined by other means not specified in this Specification.

vui_min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS. When vui_min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of vui_min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is derived as follows:

MinWavelength = vui_min_wavelength_mantissa * 10 vui_min_wavelength_exponent_plus15-15

vui_max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS.When vui_max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of vui_max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CLVS is derived as follows:

MaxWavelength = vui_max_wavelength_mantissa * 10 vui_max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

vui_modality_type_extension_bits equal to 0 specifies that vui_reserved_modality_type_extension is not present. vui_modality_type_extension_bits greater than 0 specifies the length, in bits, of vui_reserved_modality_type_extension.

The value of vui_modality_type_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for vui_modality_type_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of vui_modality_type_extension_bits in the range of 0 to 2 048, inclusive, to appear in the syntax.

vui_reserved_modality_type_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of vui_reserved_modality_type_extension in the syntax, but ignore the value. When present, the length, in bits, of vui_reserved_modality_type_extension is equal to vui_modality_type_extension_bits.

vui_payload_bit_equal_to_one shall be equal to 1.

vui_payload_bit_equal_to_zero shall be equal to 0.

# 3	Reference

[1] J. Gao, H. B. Teo, C. S. Lim, and K. Abe, “AHG8/AHG9: On Picture Modality Type”, JVET-AF00147, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[2] M. M. Hannuksela, J. Chen, S. Deshpande, Hendry, and S. McCarthy, “Technologies under consideration for future extensions of VSEI (version 2)”, JVET-AF2032, 32nd Meeting, Hannover, DE, 13–20 October 2023.

# 4	Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).