| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0079-v1 |
| --- | --- |

| Title: | AHG2/AHG9: VUI extension mechanism and picture modality information for AVC and HEVC |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jingying Gao Han Boon Teo Chong Soon Lim Kiyofumi Abe Virginie Drugeon | Tel: Email: | +65-6550-5406 jingying.gao@sg.panasonic.com |
| Source: | Panasonic |  |  |

_____________________________

# Abstract

In the 32nd JVET meeting, signaling the picture modality type in VUI parameters was adopted into the technologies under consideration for future extensions of VSEI (version 2), JVET-AF2032. In AVC and HEVC, the VUI parameters are signaled in the SPS without a specific VUI extension mechanism. This contribution proposes a VUI extension mechanism and picture modality information for AVC and HEVC.

The following items are proposed for the VUI extension mechanism in HEVC:

- Aspect 1:

- Modify sps_extension_4bits to sps_vui_extension_flag and sps_extension_3bits in the SPS RBSP syntax structure.

- Add sps_vui_extension( ) in the SPS RBSP syntax structure, as specified in Annex E.

- Aspect 2:

- Add vui_extension_bits and vui_reserved_extension in the sps_vui_extension( ) syntax structure to enable future extensions to VUI parameters.

The following items are proposed for the VUI extension mechanism in AVC:

- Aspect 3:

- Add sps_vui_extension( ) in the SPS extension RBSP syntax structure, as specified in Annex E.

- Aspect 4:

- Add additional_extension2_flag and additional_extension2_data_flag in the SPS extension RBSP syntax structure to enable future extensions to the SPS extension RBSP syntax structure.

- Aspect 5:

- Add vui_extension_bits and vui_reserved_extension in sps_vui_extension( ) syntax structure to enable future extensions to VUI parameters.

The picture modality information is added in the sps_vui_extension( )  syntax structure for both AVC and HEVC. The following items are proposed for adding picture modality information in AVC and HEVC:

- Aspect 6:

- Signal picture modality information such as picture modality type and spectrum range in the sps_vui_extension( ) syntax structure.

- Aspect 7:

- Add modality_type_extension_bits and reserved_modality_type_extension in the sps_vui_extension( ) syntax structure, for future extension of picture modality information.

It is proposed to adopt the proposed VUI extension mechanism and the proposed picture modality information into AVC and HEVC.

The proposed items for the VUI extension mechanism in AVC and HEVC are highlighted in yellow color.

The proposed items for the picture modality information in AVC and HEVC are highlighted in green color.

# Introduction

In the 32nd JVET meeting, signaling the picture modality type in VUI parameters proposed in JVET-AF0147 [1] was adopted into the technologies under consideration for future extensions of VSEI (version 2), with the document number of JVET-AF2032 [2]. In the follow-up discussion in the JVET plenary session, it was commented if this additional VUI signaling would also be beneficial to be included in the next versions of AVC and HEVC, and further study was suggested.

Unlike VSEI, the vui_parameters( ) syntax structure in AVC and HEVC is included in the SPS RBSP syntax without a specific VUI extension mechanism, as shown in the below syntax table. Hence, this contribution suggests ways of enabling the VUI extension mechanism to signal picture modality information for both AVC and HEVC.

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| … |  |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| … |  |
| } |  |

# Proposal for HEVC

## Proposed VUI extension mechanism for HEVC

Our approach to enable VUI extension is to make the following changes to 7.3.2.2.1 Sequence parameter set RBSP syntax and 7.4.3.2.1 Sequence parameter set RBSP semantics in HEVC specification.

In HEVC, there is no extension mechanism for the syntax of the VUI parameters, but the sequence parameter set RBSP syntax has an extension mechanism, 4 bits of sps_extension_4bits are reserved for future extension of SPS RBSP syntax. In the proposed method, sps_extension_4bits is modified to sps_vui_extension_flag and sps_extension_3bits. sps_vui_extension_flag indicates the presence of VUI extension data in the SPS RBSP syntax. Since the presence of VUI extension data should also depend on the presence of VUI parameters, VUI extension data is present if and only if (vui_parameters_present_flag = true) AND (sps_vui_extension_flag = true). This dual-flag mechanism ensures that the VUI extension data is only present when the VUI parameters are present and when there is an indication of the presence of VUI extension data. The VUI extension data is specified in the sps_vui_extension( ) syntax structure in Annex E of the HEVC specification.

### 7.3.2.2.1 General sequence parameter set RBSP syntax

| seq_parameter_set_rbsp( ) { | Descriptor |
| --- | --- |
| sps_video_parameter_set_id | u(4) |
| sps_max_sub_layers_minus1 | u(3) |
| sps_temporal_id_nesting_flag | u(1) |
| profile_tier_level( 1, sps_max_sub_layers_minus1 ) |  |
| sps_seq_parameter_set_id | ue(v) |
| chroma_format_idc | ue(v) |
| if( chroma_format_idc = = 3 ) |  |
| separate_colour_plane_flag | u(1) |
| pic_width_in_luma_samples | ue(v) |
| pic_height_in_luma_samples | ue(v) |
| conformance_window_flag | u(1) |
| if( conformance_window_flag ) { |  |
| conf_win_left_offset | ue(v) |
| conf_win_right_offset | ue(v) |
| conf_win_top_offset | ue(v) |
| conf_win_bottom_offset | ue(v) |
| } |  |
| bit_depth_luma_minus8 | ue(v) |
| bit_depth_chroma_minus8 | ue(v) |
| log2_max_pic_order_cnt_lsb_minus4 | ue(v) |
| sps_sub_layer_ordering_info_present_flag | u(1) |
| for( i = ( sps_sub_layer_ordering_info_present_flag ? 0 : sps_max_sub_layers_minus1 ); i <= sps_max_sub_layers_minus1; i++ ) { |  |
| sps_max_dec_pic_buffering_minus1[ i ] | ue(v) |
| sps_max_num_reorder_pics[ i ] | ue(v) |
| sps_max_latency_increase_plus1[ i ] | ue(v) |
| } |  |
| log2_min_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_luma_coding_block_size | ue(v) |
| log2_min_luma_transform_block_size_minus2 | ue(v) |
| log2_diff_max_min_luma_transform_block_size | ue(v) |
| max_transform_hierarchy_depth_inter | ue(v) |
| max_transform_hierarchy_depth_intra | ue(v) |
| scaling_list_enabled_flag | u(1) |
| if( scaling_list_enabled_flag ) { |  |
| sps_scaling_list_data_present_flag | u(1) |
| if( sps_scaling_list_data_present_flag ) |  |
| scaling_list_data( ) |  |
| } |  |
| amp_enabled_flag | u(1) |
| sample_adaptive_offset_enabled_flag | u(1) |
| pcm_enabled_flag | u(1) |
| if( pcm_enabled_flag ) { |  |
| pcm_sample_bit_depth_luma_minus1 | u(4) |
| pcm_sample_bit_depth_chroma_minus1 | u(4) |
| log2_min_pcm_luma_coding_block_size_minus3 | ue(v) |
| log2_diff_max_min_pcm_luma_coding_block_size | ue(v) |
| pcm_loop_filter_disabled_flag | u(1) |
| } |  |
| num_short_term_ref_pic_sets | ue(v) |
| for( i = 0; i < num_short_term_ref_pic_sets; i++) |  |
| st_ref_pic_set( i ) |  |
| long_term_ref_pics_present_flag | u(1) |
| if( long_term_ref_pics_present_flag ) { |  |
| num_long_term_ref_pics_sps | ue(v) |
| for( i = 0; i < num_long_term_ref_pics_sps; i++ ) { |  |
| lt_ref_pic_poc_lsb_sps[ i ] | u(v) |
| used_by_curr_pic_lt_sps_flag[ i ] | u(1) |
| } |  |
| } |  |
| sps_temporal_mvp_enabled_flag | u(1) |
| strong_intra_smoothing_enabled_flag | u(1) |
| vui_parameters_present_flag | u(1) |
| if( vui_parameters_present_flag ) |  |
| vui_parameters( ) |  |
| sps_extension_present_flag | u(1) |
| if( sps_extension_present_flag ) { |  |
| sps_range_extension_flag | u(1) |
| sps_multilayer_extension_flag | u(1) |
| sps_3d_extension_flag | u(1) |
| sps_scc_extension_flag | u(1) |
| sps_extension_4bits | u(4) |
| sps_vui_extension_flag | u(1) |
| sps_extension_3bits | u(3) |
| } |  |
| if( sps_range_extension_flag ) |  |
| sps_range_extension( ) |  |
| if( sps_multilayer_extension_flag ) |  |
| sps_multilayer_extension( ) /* specified in Annex F */ |  |
| if( sps_3d_extension_flag ) |  |
| sps_3d_extension( ) /* specified in Annex I */ |  |
| if( sps_scc_extension_flag ) |  |
| sps_scc_extension( ) |  |
| if( sps_vui_extension_flag && vui_parameters_present_flag ) |  |
| sps_vui_extension( ) /* specified in Annex E */ |  |
| if( sps_extension_4bits ) |  |
| if( sps_extension_3bits ) |  |
| while( more_rbsp_data( ) ) |  |
| sps_extension_data_flag | u(1) |
| rbsp_trailing_bits( ) |  |
| } |  |

### 7.4.3.2.1 General sequence parameter set RBSP semantics

sps_vui_extension_flag equal to 0 specifies that the sps_vui_extension( ) syntax structure (specified in Annex E) is not present in the SPS RBSP syntax structure. sps_vui_extension_flag equal to 1 specifies that this syntax structure might be present. When not present, the value of sps_vui_extension_flag is inferred to be equal to 0.

sps_extension_3bits sps_extension_4bits equal to 0 specifies that no sps_extension_data_flag syntax elements are present in the SPS RBSP syntax structure. When present, sps_extension_3bits sps_extension_4bits shall be equal to 0 in bitstreams conforming to this version of this Specification. Values of sps_extension_3bits sps_extension_4bits not equal to 0 are reserved for future use by ITU-T | ISO/IEC. Decoders shall allow the value of sps_extension_3bits sps_extension_4bits to be not equal to 0 and shall ignore all sps_extension_data_flag syntax elements in an SPS NAL unit. When not present, the value of sps_extension_3bits sps_extension_4bits is inferred to be equal to 0.

sps_extension_data_flag may have any value. Its presence and value do not affect the decoding process specified in this version of this Specification. Decoders conforming to this version of this Specification shall ignore all sps_extension_data_flag syntax elements.

## Proposed picture modality information for HEVC

The following items are proposed for adding picture modality information in HEVC, as specified in sps_vui_extension( ):

- Signal picture modality information such as modality type and spectrum range in the sps_vui_extension( ) syntax structure. It’s noted that the picture modality information is the same as option 2 of JVET-AG0077 [3] and the picture modality information will be updated according to the conclusion of the JVET-AG0077 discussion.

- Add modality_type_extension_bits and reserved_modality_type_extension in the sps_vui_extension( ) syntax structure, for future extension of picture modality information.

### E.2	VUI syntax

#### SPS VUI extension syntax:

| sps_vui_extension( ) { | C | Descriptor |
| --- | --- | --- |
| modality_info_present_flag | 0 | u(1) |
| if(modality_info_present_flag ) { |  |  |
| modality_type | 0 | u(8) |
| spectrum_range_present_flag | 0 | u(1) |
| if(spectrum_range_present_flag = = 1 ) { |  |  |
| min_wavelength_mantissa | 0 | u(10) |
| max_wavelength_mantissa | 0 | u(10) |
| min_wavelength_exponent_plus15 | 0 | u(5) |
| max_wavelength_exponent_plus15 | 0 | u(5) |
| } |  |  |
| modality_type_extension_bits | 0 | ue(v) |
| if(modality_type_extension_bits > 0) |  |  |
| reserved_modality_type_extension | 0 | u(v) |
| } |  |  |
| vui_extension_bits | 0 | ue(v) |
| if(vui_extension_bits > 0) |  |  |
| vui_reserved_extension | 0 | u(v) |
| } |  |  |

### E.3	VUI semantics

#### SPS VUI extension semantics:

modality_info_present_flag equal to 1 specifies that modality_type is present in the VUI extension. modality_info_present_flag equal to 0 specifies that modality_type is not present in the VUI extension.

modality_type indicates the type of modality of the decoded picture as specified in Table 4. When not present, the value of modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification.

Table 4– Mapping of modality_type to the type of picture modalities

| modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..255 | Reserved for future use |

spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the CVS is present in the VUI extension. spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the CVS is not present in the VUI extension.

min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS. When min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS is unknown or unspecified or determined by other means not specified in this Specification

max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS. When max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS is unknown or unspecified or determined by other means not specified in this Specification.

min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS. When min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS is derived as follows:

MinWavelength = min_wavelength_mantissa * 10 min_wavelength_exponent_plus15-15

max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS. When max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the CVS is derived as follows:

MaxWavelength = max_wavelength_mantissa * 10 max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

vui_extension_bits equal to 0 specifies that vui_reserved_extension is not present in the SPS VUI extension syntax structure. vui_ extension_bits greater than 0 specifies the length, in bits, of vui_reserved_extension.

The value of vui_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for vui_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of vui_extension_bits in the range of 0 to 2 048, inclusive.

vui_reserved_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of vui_reserved_extension in the syntax, but ignore the value. When present, the length, in bits, of vui_reserved_extension is equal to vui_extension_bits.

# Proposal for AVC

## Proposed VUI extension mechanism for AVC

Our approach to enable VUI extension is to make the following changes to 7.3.2.1.2 Sequence parameter set extension RBSP syntax and 7.4.2.1.2 Sequence parameter set extension RBSP semantics in AVC specification.

In AVC, there is no extension mechanism for the syntax of the VUI parameters, but the sequence parameter set extension RBSP syntax includes an extension mechanism, where additional_extension_flag allows adding additional extension data at the end of the SPS extension RBSP syntax. The presence of VUI extension data should also depend on the presence of VUI parameters. Same as HEVC, vui_parameters_present_flag is used as a conditional flag indicating the presence of VUI parameters. The VUI extension data is only present when the VUI parameters are present and when there is additional extension data at the end of the SPS extension RBSP syntax. The VUI extension data is specified in the sps_vui_extension( ) syntax structure in Annex E of the AVC specification.

Additionally, additional_extension2_flag and additional_extension2_data_flag are added to the SPS extension RBSP syntax structure, to enable future extensions to the SPS extension RBSP syntax.

### 7.3.2.1.2 Sequence parameter set extension RBSP syntax

| seq_parameter_set_extension_rbsp( ) { | C | Descriptor |
| --- | --- | --- |
| seq_parameter_set_id | 10 | ue(v) |
| aux_format_idc | 10 | ue(v) |
| if( aux_format_idc != 0 ) { |  |  |
| bit_depth_aux_minus8 | 10 | ue(v) |
| alpha_incr_flag | 10 | u(1) |
| alpha_opaque_value | 10 | u(v) |
| alpha_transparent_value | 10 | u(v) |
| } |  |  |
| additional_extension_flag | 10 | u(1) |
| if(additional_extension_flag = = 1 ) { |  |  |
| if(vui_parameters_present_flag = = 1) |  |  |
| sps_vui_extension( ) /* specified in Annex E */ | 10 |  |
| additional_extension2_flag | 10 | u(1) |
| if(additional_extension2_flag = = 1 ) |  |  |
| while( more_rbsp_data( ) ) |  |  |
| additional_extension2_data_flag | 10 | u(1) |
| } |  |  |
| rbsp_trailing_bits( ) | 10 |  |
| } |  |  |

### 7.4.2.1.2 Sequence parameter set extension RBSP semantics

additional_extension_flag equal to 0 indicates that no additional data follows within the sequence parameter set extension syntax structure prior to the RBSP trailing bits. The value of additional_extension_flag shall be equal to 0. The value of 1 for additional_extension_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for additional_extension_flag in a sequence parameter set extension NAL unit.

additional_extension_flag equal to 1 specifies that additional_exension2_flag is present in the SPS RBSP syntax structure. additional_extension_flag equal to 0 specifies that additional_exension2_flag is not present.

additional_extension2_flag equal to 0 indicates that no additional data follows within the sequence parameter set extension syntax structure prior to the RBSP trailing bits. The value of additional_extension2_flag shall be equal to 0. The value of 1 for additional_extension2_flag is reserved for future use by ITU-T | ISO/IEC. Decoders shall ignore all data that follows the value of 1 for additional_extension2_flag in a sequence parameter set extension NAL unit.

additional_extension2_data_flag may have any value. It shall not affect the conformance to profiles specified in Annex A, G, H, or I.

## Proposed picture modality information for AVC

The following items are proposed for adding picture modality information in AVC, as specified in the  sps_vui_extension( ) syntax structure:

- Signal picture modality information such as picture modality type and spectrum range in the sps_vui_extension( ) syntax structure. It’s noted that the picture modality information here is the same as option 2 of JVET-AG0077 and the picture modality information will be updated according to the conclusion of the JVET-AG0077 discussion.

- Add modality_type_extension_bits and reserved_modality_type_extension in the sps_vui_extension( ) syntax structure, for future extension of picture modality information.

### E.1	VUI syntax

#### SPS VUI extension syntax:

| sps_vui_extension( ) { | C | Descriptor |
| --- | --- | --- |
| modality_info_present_flag | 0 | u(1) |
| if(modality_info_present_flag ) { |  |  |
| modality_type | 0 | u(8) |
| spectrum_range_present_flag | 0 | u(1) |
| if(spectrum_range_present_flag = = 1 ) { |  |  |
| min_wavelength_mantissa | 0 | u(10) |
| max_wavelength_mantissa | 0 | u(10) |
| min_wavelength_exponent_plus15 | 0 | u(5) |
| max_wavelength_exponent_plus15 | 0 | u(5) |
| } |  |  |
| modality_type_extension_bits | 0 | ue(v) |
| if(modality_type_extension_bits > 0) |  |  |
| reserved_modality_type_extension | 0 | u(v) |
| } |  |  |
| vui_extension_bits | 0 | ue(v) |
| if(vui_extension_bits > 0) |  |  |
| vui_reserved_extension | 0 | u(v) |
| } |  |  |

### E.2	VUI semantics

#### SPS VUI extension semantics:

modality_info_present_flag equal to 1 specifies that modality_type is present in the VUI extension. modality_info_present_flag equal to 0 specifies that modality_type is not present in the VUI extension.

modality_type indicates the type of modality of the decoded picture as specified in Table 4. When not present, the value of modality_type is inferred to be equal to 0, denoting that the modality type of the picture is unknown or unspecified or determined by other means not specified in this Specification.

Table 4– Mapping of modality_type to the type of picture modalities

| modality_type | type of picture modality |
| --- | --- |
| 0 | Unspecified |
| 1 | Visible Picture |
| 2 | Infrared Picture |
| 3 | Ultraviolet Picture |
| 4..255 | Reserved for future use |

spectrum_range_present_flag equal to 1 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the coded video sequence is present in the VUI exntension. spectrum_range_present_flag equal to 0 specifies that the spectrum band of the optical radiation wavelength represented by the pictures in the coded video sequence is not present in the VUI extension.

min_wavelength_mantissa specifies the mantissa part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When min_wavelength_mantissa is equal to 0 or is not present, the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is unknown or unspecified or determined by other means not specified in this Specification

max_wavelength_mantissa specifies the mantissa part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When max_wavelength_mantissa is equal to 0 or is not present, the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is unknown or unspecified or determined by other means not specified in this Specification.

min_wavelength_exponent_plus15 minus 15 specifies the exponent part of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When min_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of min_wavelength_exponent_plus15.

The value of the minimum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is derived as follows:

MinWavelength = min_wavelength_mantissa * 10 min_wavelength_exponent_plus15-15

max_wavelength_exponent_plus15 minus 15 specifies the exponent part of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence. When max_wavelength_mantissa is not present or is equal to 0, decoders shall ignore the value of max_wavelength_exponent_plus15.

The value of the maximum wavelength indicating the spectral band of optical radiation represented by the pictures in the coded video sequence is derived as follows:

MaxWavelength = max_wavelength_mantissa * 10 max_wavelength_exponent_plus15-15

MinWavelength and MaxWavelength are in units of meters as specified in ISO/IEC 80000.

vui_extension_bits equal to 0 specifies that vui_reserved_extension is not present in the SPS VUI extension syntax structure. vui_ extension_bits greater than 0 specifies the length, in bits, of vui_reserved_extension.

The value of vui_extension_bits shall be in the range of 0 to 2 048, inclusive. Values in the range of 1 to 2 048, inclusive, for vui_extension_bits are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this document. Decoders conforming to this version of this document shall allow any value of vui_extension_bits in the range of 0 to 2 048, inclusive.

vui_reserved_extension shall not be present in bitstreams conforming to this version of this document. However, decoders conforming to this version of this document shall allow the presence of vui_reserved_extension in the syntax, but ignore the value. When present, the length, in bits, of vui_reserved_extension is equal to vui_extension_bits.

# Reference

[1] J. Gao, H. B. Teo, C. S. Lim, and K. Abe, “AHG8/AHG9: On Picture Modality Type”, JVET-AF0147, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[2] M. M. Hannuksela, J. Chen, S. Deshpande, Hendry, and S. McCarthy, “Technologies under consideration for future extensions of VSEI (version 2)”, JVET-AF2032, 32nd Meeting, Hannover, DE, 13–20 October 2023.

[3] J. Gao, H. B. Teo, C. S. Lim, K. Abe, V. Drugeon, “AHG9: On Picture Modality Information”, JVET-AG0077, 33rd Meeting, Online, 17–26 January 2024.

# Patent rights declaration(s)

Panasonic may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).