| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0081-v1 |
| --- | --- |

| Title: | AHG9: On signalling privacy protection information in SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

Abstract

This contribution proposes to have a mechanism to convey privacy protection information (PPI) in an SEI message. Privacy protection information for coded video denotes measures that may be applied to protect personal identifying information within the coded picture in the video. The protection can be in the form of removal and / or alteration of data. It is asserted that such action can have an impact on the handling or use of the coded video at decoder side.

Two options are proposed to convey PPI privacy protection information in an SEI message as follows:

- Option 1: Define privacy protection information as a new optimization type in Encoder Optimization Information (EOI) SEI message.

- Option 2: Define a new SEI message to carry the privacy protection information.

1. Introduction

Collection, distribution, and usage of image/video data containing personal information are strictly regulated. For example, regulations such as California Consumer Privacy Act (CCPA) in the United States, Personal Information Protection Law (PIPL) in China, and General Data Protection Regulation (GDPR) in the EU rigorously control the use of data containing personal information. This poses a significant challenge in the development and application that uses coded video. For example, videos collected from from surveillance which may be used for security / intelligence application need to have privacy personal information protected. Organizations are adopting technologies for de-identification and anonymization of personal information to develop and utilize coded video while adhering to strict standards. Table 1 below show non-exhausted list of organizations providing privacy protection technologies and types of their technologies. Blurring and masking techniques remove information to make personal identification difficult. The replacing technique substitutes faces or license plates with different / generic ones, making it difficult to identify the individuals in the original video.

Table 1 – Examples of picture modality types

| Organization | technology |
| --- | --- |
| Brighter AI [1] | Blurring, replacing |
| secureredact [2] | Blurring |
| Gallio PRO[3] | Blurrung |
| Celantur[4] | Blurring |
| AXIS[5] | Masking, blurring |

2. Problem description

Removing or modifying personal information from videos to comply with data protection laws is a common practice. However, such protection may have impact to the handling and / or use of the video. For example, if specific information is removed or altered for a coded video and the video is used for a machine analysis task, the result that be different from what it is supposed to be and to avoid that some adjustment may be applied. Therefore, providing information in the bitstream about any changes (e.g., removal or alteration of personal information) in the coded video may be valuable.

It is asserted that, currently, there is no mechanism to convey privacy protection information (PPI) defined for coded video. The absent of this feature may cause the following issues when privacy data protection is applied:

- The reliability of perception. When personal information is removed or altered from a video, it can affect the perception of both human and machine. For example, if face or license plate numbers are altered, both machines and humans will perceive different information from the original video. In other words, it may be recognized as another person or another vehicle.

- The unnecessary perception. There may be cases where altered personal information has an impact and cases where it does not. For example, when facial blurring is applied, face recognition results may be affected, but human action recognition may not be affected.

PPI can tell which picture's personal information has modified. It can help prevent perceoption from a modified picture or indicate that the perception result is from a modified picture. This can improve the reliability of cognitive results and reduce unnecessary perception.

3. Proposal

3.1 Option 1: Privacy protection information as a new optimization type in encoder optimization information (EOI) SEI message

Privacy protection information can be conveyed from encoder to decoder as one of the optimization types in the current Encoder Optimization Information (EOI) SEI message that is included in JVET-AF2032. Further detail information may also be signalled for such optimization type. The changes to the current syntax and semantics of EOI SEI message is shown below.

Syntax:

| encoder_optimization_info(payloadSize ) { | Descriptor |
| --- | --- |
| eoi_cancel_flag | u(1) |
| if( !eoi_cancel_flag ) { |  |
| eoi_persistence_flag | u(1) |
| eoi_for_human_viewing_flag | u(1) |
| eoi_for_machine_analysis_flag | u(1) |
| eoi_type | u(16) |
| if( EoiObjectBasedFlag ) |  |
| eoi_object_based_idc | ue(v) |
| if( EoiTemporalResamplingFlag ) { |  |
| eoi_temporal_resampling_type_flag | u(1) |
| eoi_num_int_pics | ue(v) |
| } |  |
| if( EoiPrivacyProtectionFlag ) { |  |
| privacy_protection_type_idc | u(4) |
| protected_info_type | u(8) |
| } |  |
| } |  |
| } |  |

Semantic:

eoi_type indicates the types of optimization method as specified in Table x1 where ( eoi_type & bitMask ) not equal to 0 indicates that the optimization type with the bitMask value in Table x1 has been applied. When eoi_type is greater than 0 and ( eoi_type & bitMask ) is equal to 0, the optimization type with the bitMask value has not been applied. When eoi_type is equal to 0, optimization as determined by the application has been used.

Table x1 – Definition of eoi_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization in a manner that quality fluctuates temporally |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | With personal information protection optimization; the pictures for which this SEI message persists have been pre-processed or encoded to protect personal information. (e.g. removal or replacing of personal identifiable information, pseudonymization, anonymization) |

The variables EoiObjectBasedFlag, EoiTemporalResamplingFlag, EoiSpatialResamplingFlag, EoiTemporalQualityFlag, and EoiSpatialQualityFlag, EoiPrivacyProtectionFlag specifying whether eoi_type indicates the type of the optimization to include object-based optimization, temporal resampling optimization, spatial resampling optimization, temporal quality optimization ,spatial quality optimization and personal information protection optimization, respectively, are derived as follows:

EoiObjectBasedFlag = ( ( eoi_type & 0x01 ) > 0 ) ? 1 : 0
EoiTemporalResamplingFlag = ( ( eoi_type & 0x02 ) > 0 ) ? 1 : 0
EoiSpatialResamplingFlag = ( ( eoi_type & 0x04 ) > 0 ) ? 1 : 0	(xx)
EoiTemporalQualityFlag = ( ( eoi_type & 0x08 ) > 0 ) ? 1 : 0
EoiSpatialQualityFlag = ( ( eoi_type & 0x10 ) > 0 ) ? 1 : 0
EoiPrivacyProtectionFlag = ( ( eoi_type & 0x20 ) > 0 ) ? 1 : 0

privacy_protection_type_idc indicates the types of privacy protection method as specified in Table 2.

Table 2 – Definition  of  privacy_protection_type_idc

| privacy_protection_type_idc | Interpretation |
| --- | --- |
| 0 | Unknown. |
| 1 | With blurring; personal information is blurred to make it unidentifiable. |
| 2 | With replacing; personal information is replaced with something different from the original to make it unidentifiable. |
| 3 | With masking; personal information is masked so that it cannot be identified. |
| 4..15 | Reserved for future use |

protected_info_type indicates the types of protected information as specified in Table 3 where ( protected_info_type & bitMask ) not equal to 0 indicates that the information type with the bitMask value in Table 3 has been protected.

Table 3 – Definition of protected_info_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Information that identifies a person (such as their face) is protected. |
| 0x02 | Information that can identify vehicles(e.g. cars, license plates) is protected. |
| 0x04 | Information that can infer locations(e.g. text or images on signs) is protected |

3.2 Option 2: Define a new SEI to convey privacy protection information

It is also possible to convey PPI in a new SEI message as shown below.

Syntax:

| privacy_protection_info(payloadSize ) { | Descriptor |
| --- | --- |
| privacy_protection_cancel_flag | u(1) |
| if(!privacy_protection_cancel_flag){ |  |
| privacy_protection_persistence_flag | u(1) |
| privacy_protection_type_idc | u(4) |
| protected_info_type | u(8) |
| } |  |
| } |  |

Semantic:

privacy_protection_cancel_flag equal to 1 specifies that the persistence of the previous applied privacy protection is cancelled. privacy_protection_cancel_flag equal to 0 indicates that privacy_protection_persistence_flag, privacy_protection_type_idc and protected_info_type follow.

privacy_protection_persistence_flag specifies the persistence of the privacy protection the current layer. privacy_protection_persistence_flag equal to 0 specifies that the privacy protection applies for the current picture only. privacy_protection_persistence_flag equal to 1 specifies that the privacy protection applies for the current picture and all subsequent pictures of the current layer in output order until one or more of the following conditions are true:

–	A new CLVS of the current layer begins.

–	The bitstream ends.

–	A picture in the current layer associated with an privacy_protection_info SEI message is output that follows the current picture in output order.

privacy_protection_type_idc indicates the types of privacy protection method as specified in Table 4.

Table 4 – Definition of privacy_protection_type_idc

| privacy_protection_type_idc | Interpretation |
| --- | --- |
| 0 | Unknown. |
| 1 | With blurring; personal information is blurred to make it unidentifiable. |
| 2 | With replacing; personal information is replaced with something different from the original to make it unidentifiable. |
| 3 | With masking; personal information is masked so that it cannot be identified. |
| 4..15 | Reserved for future use |

protected_info_type indicates the types of protected information as specified in Table 5 where ( protected_info_type & bitMask ) not equal to 0 indicates that the information type with the bitMask value in Table 5 has been protected.

Table 5 – Definition of protection_info_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Information that identifies a person (such as their face) is protected. |
| 0x02 | Information that can identify vehicles(e.g. cars, license plates) is protected. |
| 0x04 | Information that can infer locations(e.g. text or images on signs) is protected |

4. Reference

- https://brighter.ai/

- https://www.secureredact.ai/

- https://gallio.pro/

- https://www.celantur.com/automotive-data-anonymization/

- https://www.axis.com/products/axis-live-privacy-shield

- Miska M. Hannuksela, Jie Chen, Sachin Deshpande, Hendry, Sean McCarthy “Technologies under consideration for future extensions of VSEI (version 2)”, JVET-AF2032, 32nd Meeting, Hannover, DE, 13–20 October 2023

5. Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).