| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0082-v1 |
| --- | --- |

| Title: | AHG9: On Source Picture Timing Information SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jonatan Samuelsson-Allendes Sachin Deshpande | Tel: Email: | +46721882169 samuelssonj@sharplabs.com |
| Source: | Sharp Labs of America, Inc. |  |  |

_____________________________

# Abstract

This input document relates to the Source Picture Timing Information (SPTI) SEI message included in the Technology under Consideration for future extensions of VSEI (draft 2). It is proposed to modify how TemporalId is used to derive SourcePictureInterval. It is asserted that the proposed modification makes the SPTI signalling more robust and able to handle more usage scenarios.

# Introduction

The source picture timing information SEI message offers content providers the means for providing receivers with information about the source picture timing, which may be different from the output picture timing. In the current version of the SPTI SEI message draft the source picture interval is defined for “consecutive pictures” and mentions “the previous decoded output picture”. We assert that it might not always be clear to the receiver which picture is meant by “the previous decoded output picture”. In particular in cases that involves temporal scalability, step-wise temporal access and/or drop of individual pictures due to congestion.

# Proposal

We believe the most reliable and robust reference is the previous picture in temporal layer zero. It is therefore proposed to make the following modifications (marked in red):

spti_sublayer_interval_scale_factor[ i ], when present, specifies a scale factor used in determining the source picture interval of corresponding consecutive pictures in output order in the CLVS having TemporalId less than or equal to i, relative to the previous output picture with TemporalId equal to 0. The value 0 may be used to indicate that the source picture corresponding to the current decoded output picture is identical to the source picture corresponding to the previous decoded output picture with TemporalId equal to 0.

The indicated source picture interval associated with an output pictures having TemporalId less than or equal to i, relative to the previous output picture with TemporalId equal to 0, denoted by the variable SourcePictureInterval[ i ], in units of seconds, is derived as follows:

SourcePictureInterval[ i ] = ElementalSourcePictureInterval * spti_sublayer_interval_scale_factor[ i ] * ( 1 – 2 * temporalReversalFlag )				(8-X)

The variable temporalReversalFlag is equal to ( spti_source_type & 0x10 )? 1 : 0.

# Patent rights declaration(s)

Sharp Labs of America, Inc., may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Acknowledgement
These research results were obtained from the commissioned research (No.05101) by National Institute of Information and Communications Technology (NICT), Japan.