| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0083-v1 |
| --- | --- |

| Title: | AHG9: On feature-based optimization type |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chul Keun Kim Hendry Jaehyun Lim Seung hwan Kim | Email: | chulkeun.kim@lge.com |
| Source: | LG Electronics |  |  |

_____________________________

Abstract

This contribution proposes to add feature-based optimization type in Encoder Optimization Information (EOI) SEI message. The feature-based optimization type indicates that the pictures have been pre-processed or encoded based on the quality of features. This helps clarify the criteria and properties of the encoder optimization.

1. Introduction

Traditional video codecs aim to minimize pixel-level distortions in order to optimize for human viewing. However, minimizing pixel-level distortions may not be optimal for machine analysis. This is because machine analysis relies on features rather than pixel-level details. A feature can refer to information about various aspects of the input data, such as edges, texture, shape, or higher level semantic information, and a feature can also be the output of a neural network that that represent specific feature of the input picture. Research on feature-based optimization for machine analysis is reported as follows:

- Feature-based RDO. This method optimizes ratio-distortion by calculating distortion using extracted features. To extract features, pre-trained models are used [1][2] or machine perception aware metric model can be defined through the training[3].

- Feature-based pre-processing. This method minimizes loss of important features and reduces unnecessary information for machine analysis[4][5]. This makes the input picture more suitable for compression and reduce the bitrate without negative impact on machine analysis.

2. Problem description

Encoder may apply feature-based optimization to improve quality for areas with important features over areas without less important features in the decoded picture. The presence of important features can vary depending on the method of feature extraction. Feature extraction method can be defined based on the type and purpose of the task which may have an impact on both human and machine perception. Therefore, providing information in the bitstream about feature-based optimization in the coded video may be valuable.

3. Proposal

We propose to add a new value to eoi_type to indicate feature-based optimization in the Encoder Optimization Information (EOI) SEI messages that is included in JVET-AF2032[6].

3.1 Text implementation for the proposal

The changes to the current semantics of EOI SEI message is shown below.

Semantic:

eoi_type indicates the types of optimization method as specified in Table x1 where ( eoi_type & bitMask ) not equal to 0 indicates that the optimization type with the bitMask value in Table x1 has been applied. When eoi_type is greater than 0 and ( eoi_type & bitMask ) is equal to 0, the optimization type with the bitMask value has not been applied. When eoi_type is equal to 0, optimization as determined by the application has been used.

Table x1 – Definition of eoi_type

| bitMask | Interpretation |
| --- | --- |
| 0x01 | Object-based optimization; the pictures for which this SEI message persists have been pre-processed or encoded so that detected objects in the pictures are optimized with respect to other parts of the pictures for the indicated optimization purposes |
| 0x02 | Temporal resampling optimization |
| 0x04 | Spatial resampling optimization |
| 0x08 | Temporal quality optimization in a manner that quality fluctuates temporally |
| 0x10 | Spatial quality optimization; the pictures for which this SEI message persists have been pre-processed or encoded to reduce unnecessary information or improve the quality of necessary information.(e.g to reduce the amount of noise and remove speckles at the picture-level) |
| 0x20 | Feature based optimization; the pictures for which this SEI message persists have been pre-processed or encoded based on the quality of features. (e.g to minimize loss of important features or optimize by calculating distortion using extracted features) |

The variables EoiObjectBasedFlag, EoiTemporalResamplingFlag, EoiSpatialResamplingFlag, EoiTemporalQualityFlag, and EoiSpatialQualityFlag, EoiFeatureBasedFlag specifying whether eoi_type indicates the type of the optimization to include object-based optimization, temporal resampling optimization, spatial resampling optimization, temporal quality optimization ,spatial quality optimization and feature-based optimization, respectively, are derived as follows:

EoiObjectBasedFlag = ( ( eoi_type & 0x01 ) > 0 ) ? 1 : 0
EoiTemporalResamplingFlag = ( ( eoi_type & 0x02 ) > 0 ) ? 1 : 0
EoiSpatialResamplingFlag = ( ( eoi_type & 0x04 ) > 0 ) ? 1 : 0	(xx)
EoiTemporalQualityFlag = ( ( eoi_type & 0x08 ) > 0 ) ? 1 : 0
EoiSpatialQualityFlag = ( ( eoi_type & 0x10 ) > 0 ) ? 1 : 0
EoiFeatureBasedFlag = ( ( eoi_type & 0x20 ) > 0 ) ? 1 : 0

4. Reference

- K. Fischer, F. Brand, C. Herglotz, and A. Kaup, “Video coding for machines with feature-based rate-distortion optimization,” IEEE 22nd International Workshop on Multimedia Signal Processing, p. 6, September 2020.

- A. Gou, H. Sun, X. Zeng and Y. Fan, "Fast VVC Intra Encoding for Video Coding for Machines," 2023 IEEE International Symposium on Circuits and Systems (ISCAS), Monterey, CA, USA, 2023, pp. 1-5, doi: 10.1109/ISCAS46773.2023.10181507.

- A. Marie, K. Desnos, C. Fu, J. Zhou, L. Morin and L. Zhang, "Towards Machine Perception Aware Image Quality Assessment," 2023 IEEE 25th International Workshop on Multimedia Signal Processing (MMSP), Poitiers, France, 2023, pp. 1-6, doi: 10.1109/MMSP59012.2023.10337677.

- G. Lu, X. Ge, T. Zhong, J. Geng and Q. Hu, “Preprocessing enhanced image compression for machine vision,”, 2022, arXiv:2206.05650

- S. Kwak, J. Yun, H. Choo and M. Kim, “Feature-Guided Machine-Centric Image Coding for Downstream Tasks,” 2023 IEEE International Conference on Multimedia and Expo Workshops, doi: 10.1109/ICMEW59549.2023.00037

- Miska M. Hannuksela, Jie Chen, Sachin Deshpande, Hendry, Sean McCarthy, “Technologies under consideration for future extensions of VSEI (version 2)”, JVET-AF2032, 32nd Meeting, Hannover, DE, 13–20 October 2023

5. Patent rights declaration(s)

LG Electronics Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).