| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0084-v1 |
| --- | --- |

| Title: | AHG12: DIMD Merge List |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Médéric Blestel, Pierre Andrivon | Email: | mblestel@ofinno.com pandrivon@ofinno.com |
| Source: | Ofinno |  |  |

_____________________________

# Abstract

Contribution JVET-AG0084 proposes to extend the DIMD merge process that has been introduced in JVET-AE0071, refined in JVET-AF0120 and JVET-AF0106, by introducing the generation of a DIMD merge candidates list. The DIMD merge list is built reusing the Decoder-side Intra Mode Derivation (DIMD) information and merged DIMD information (as described in JVET-AF0120) from spatial adjacent and non-adjacent neighboring blocks. On top of ECM-11.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

- AI: -0.12%, 0.00%, 0.05%  {XXX%, XXX%}

- RA: XXX%, XXX%, XXX%  {XXX%, XXX%}

# Background

## DIMD process

ECM includes Decoder-side Intra Mode Derivation (DIMD), which is composed of up to five intra-prediction mode directions, derived by analyzing the directionality of the content in the surrounding of the current block. A Histogram of Gradients (HoG), computed on a three-sample wide/high L-shaped template formed of already reconstructed samples, is built by accumulating the magnitudes of all gradients for all the samples within the gray region of Figure 1. Up to five intra predictions directions are identified thanks to the cumulative magnitude of all the gradients, and these intra predictions are then blended with the Planar mode to form the final DIMD prediction. Uniform or spatial blending is used using weights that depend on the relative magnitudes of the modes in the histogram of gradients.

Figure 1 - the DIMD template to form the HoG.

## DIMD Merge Mode

DIMD merge mode has been introduced in [1] and [2]. It consists in merging the HoG of neighboring blocks to derive DIMD information. Up to 13 CUs’ DIMD information in the surroundings are used to derive the merged HoG (MHoG). The MHoG is used to derive intra prediction modes and weights, as in conventional DIMD. The directional modes and the respective weights corresponding to the five highest amplitudes in the MHoG are selected, and the corresponding predictors are blended as in conventional DIMD. In [2], a reduced storage version of the DIMD merge is proposed where the five highest amplitudes of the histograms are stored and averaged.

In [3], the DIMD merge mode is extended to include up to 31 surrounding CUs to derive the MHoG. Indeed, on top of the original 13 CUs used in [2], non-adjacent spatial candidates are considered as in Figure 2.

Figure 2: non-adjacent spatial neighboring candidates for DIMD merge.

A new flag is introduced and is signaled just after the DIMD flag if the latter is true.

# DIMD Merge Mode list

## Proposed approach

This contribution proposes to create a DIMD merge list from neighboring blocks’ DIMD information.
It includes:

- DIMD information from spatial neighbors (as in Figure 3),

- DIMD information from non-adjacent neighbors (as in Figure 2),

- DIMD information derived from the MHoG (as described in [3]).

Figure 3: neighboring blocks

Two redundancy checks (pruning stage) are applied: one comparing with DIMD merge candidates, and one comparing with DIMD information derived from the current block. Thus, a DIMD merge candidate is added to the DIMD merge list when the associated DIMD information is different from every information from DIMD merge candidates already present in the list and the current block DIMD information.

Two additional flags are added conditioned to the DIMD flag, i.e., the DIMD Merge is considered as a sub-mode of DIMD.

The two flags are:

- The DIMD merge mode flag (like in [1])

- The DIMD merge mode index representative of the DIMD merge candidate.

## DIMD merge candidates’ evaluation

DIMD merge is only available as an option if the current block has at least one neighbor coded with DIMD or DIMD merge modes using the same method as proposed in [1]. Besides, the neighboring is extended to include part of the non-adjacent candidates.

When the DIMD merge is available, the DIMD merge list candidate is derived both at the encoder and decoder sides. On the encoder side, each candidate is evaluated using an Hadamard pass, then in the RDO loop if the associated Hadamard-based cost is competitive.

A DIMD merge candidate may be discarded if the associated cost is not competitive compared to other modes.

# Experimental results

The tables below summarize the coding efficiency and runtime impact of the proposed method in ECM 11.0 using the JVET common test conditions.

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 | -0,17% | -0,07% | -0,07% |  |  |
| Class A2 | -0,17% | 0,05% | 0,10% |  |  |
| Class B | -0,11% | 0,00% | 0,05% |  |  |
| Class C | -0,10% | -0,03% | 0,05% |  |  |
| Class E | -0,07% | 0,07% | 0,11% |  |  |
| Overall | -0,12% | 0,00% | 0,05% |  |  |
| Class D | -0,09% | -0,04% | 0,09% |  |  |
| Class F |  |  |  |  |  |
|  | Random Access Main 10 |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |
|  | Y | U | V | EncT | DecT |
| Class A1 |  |  |  |  |  |
| Class A2 |  |  |  |  |  |
| Class B |  |  |  |  |  |
| Class C |  |  |  |  |  |
| Class E |  |  |  |  |  |
| Overall |  |  |  |  |  |
| Class D |  |  |  |  |  |
| Class F |  |  |  |  |  |

# References

- S. Blasi, I. Zupancic, J. Lainema, “AHG12 - Decoder-side Intra Mode Derivation Merge,” JVET-AE0071, July 2023.

- S. Blasi, I. Zupancic, J. Lainema, “EE2-2.1 DIMD merge,” JVET-AF0120, October 2023.

- J. Huo, J. Fan, Z. Zhang, Y. Ma, F. Yang, M. Li, “EE2-related: Non-adjacent spatial candidates for DIMD merge,” JVET-AF0106, October 2023.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).