| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0089-v1 |
| --- | --- |

| Title: | AHG9: Temporal extrapolation purpose for the neural-network post-filter characteristics SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Miska M. Hannuksela, Francesco Cricri, Honglei Zhang Nokia Technologies, Tampere, Finland | Email: | {miska.hannuksela, francesco.cricri, honglei.1.zhang}@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

The neural-network post-filter characteristics (NNPFC) SEI message includes the nnpfc_purpose syntax element, which enables to signal any number of purposes to which the defined neural-network post-filter (NNPF) is intended. For example, the purposes include picture rate upsampling through interpolating pictures between one or more pairs of input pictures.

It is asserted that there are use cases for temporal extrapolation, i.e., generating one or more pictures following all the input pictures in output order. Hence, this contribution proposes the addition of temporal extrapolation as one of the purposes of nnpfc_purpose. This addition includes the following aspects:

- A new bit in nnpfc_purpose is dedicated for temporal extrapolation.

- When nnpfc_purpose indicates temporal extrapolation, nnpfc_extrapolated_pics_minus1 plus 1 indicates the number of extrapolated pictures generated by the NNPF.

- The constraints to avoid conflicting or ambiguous output order are updated to concern extrapolated pictures in addition to interpolated pictures.

# Introduction

A temporal extrapolation NN, which is also known as a video prediction NN, generates one or more pictures in the future given one or more past pictures as input.

Use cases for temporal extrapolation include:

- Very low delay computer vision for domains like robotics and autonomous driving, where extrapolated future pictures facilitate anticipatory decision making. [1]

- Increase of the rendered picture rate in very low-latency applications, such as cloud gaming, relative to the decoded picture rate. [2]

This contribution proposes adding temporal extrapolation as one of the purposes of nnpfc_purpose.

# Overview of the proposed changes

This contribution proposes the addition of temporal extrapolation as one of the purposes of nnpfc_purpose. This addition includes the following aspects:

- A new bit in nnpfc_purpose is dedicated for temporal extrapolation.

- When nnpfc_purpose indicates temporal extrapolation, nnpfc_extrapolated_pics_minus1 plus 1 indicates the number of extrapolated pictures generated by the NNPF.

- The constraints to avoid conflicting or ambiguous output order are updated to concern extrapolated pictures in addition to interpolated pictures.

# Proposed specification text changes

## NNPFC SEI message syntax

| nn_post_filter_characteristics( payloadSize ) { | Descriptor |
| --- | --- |
| … |  |
| if( PictureRateUpsamplingFlag ) |  |
| for( i = 0; i < nnpfc_num_input_pics_minus1; i++ ) |  |
| nnpfc_interpolated_pics[ i ] | ue(v) |
| if( TemporalExtrapolationFlag ) |  |
| nnpfc_extrapolated_pics_minus1 | ue(v) |
| … |  |

## NNPFC SEI message semantics

…

The value of nnpfc_purpose shall be in the range of 0 to 63127, inclusive, in bitstreams conforming to this edition of this document. Values of 64128 to 65 535, inclusive, for nnpfc_purpose are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this edition of this document. Decoders conforming to this edition of this document shall ignore NNPFC SEI messages with nnpfc_purpose in the range of 64128 to 65 535, inclusive.

Table 20 – Definition of nnpfc_purpose

| bitMask | Interpretation |
| --- | --- |
| 0x01 | General visual quality improvement |
| 0x02 | Chroma upsampling (from the 4:2:0 chroma format to the 4:2:2 or 4:4:4 chroma format, or from the 4:2:2 chroma format to the 4:4:4 chroma format) |
| 0x04 | Resolution resampling (increasing or decreasing the width or height) |
| 0x08 | Picture rate upsampling |
| 0x10 | Bit depth upsampling (increasing the luma bit depth or the chroma bit depth) |
| 0x20 | Colourization |
| 0x40 | Temporal extrapolation (i.e., generating one or more future pictures) |

The variables ChromaUpsamplingFlag, ResolutionResamplingFlag, PictureRateUpsamplingFlag, BitDepthUpsamplingFlag, and ColourizationFlag, and TemporalExtrapolationFlag, specifying whether nnpfc_purpose indicates the purpose of the NNPF to include chroma upsampling, resolution resampling, picture rate upsampling, bit depth upsampling, and colourization, and temporal extrapolation, respectively, are derived as follows:

ChromaUpsamplingFlag = ( ( nnpfc_purpose & 0x02 ) > 0 ) ? 1 : 0
ResolutionResamplingFlag = ( ( nnpfc_purpose & 0x04 ) > 0 ) ? 1 : 0
PictureRateUpsamplingFlag = ( ( nnpfc_purpose & 0x08 ) > 0 ) ? 1 : 0	(75)
BitDepthUpsamplingFlag = ( ( nnpfc_purpose & 0x10 ) > 0 ) ? 1 : 0
ColourizationFlag = ( ( nnpfc_purpose & 0x20 ) > 0 ) ? 1 : 0
TemporalExtrapolationFlag = ( ( nnpfc_purpose & 0x40 ) > 0 ) ? 1 : 0

NOTE 2 – When a reserved value of nnpfc_purpose is taken into use in the future by ITU-T | ISO/IEC, the syntax of this SEI message could be extended with syntax elements whose presence is conditioned by nnpfc_purpose being equal to that value or any one of a set of values including that value.

When ChromaFormatIdc is equal to 3, ChromaUpsamplingFlag shall be equal to 0.

When ChromaUpsamplingFlag is equal to 1, ColourizationFlag shall be equal to 0.

When PictureRateUpsamplingFlag or TemporalExtrapolationFlag is equal to 1 and the input picture with index 0 is associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5, all input pictures are associated with a frame packing arrangement SEI message with fp_arrangement_type equal to 5 and the same value of fp_current_frame_is_frame0_flag.

When TemporalExtrapolationFlag is equal to 1, the extrapolated pictures generated by the NNPF follow all input pictures of the NNPF in output order. When TemporalExtrapolationFlag is equal to 1 and there is a decoded output picture that follows, in output order, the current picture for which the NNPF is activated, the extrapolated pictures generated by the NNPF precede that decoded output picture in output order.

…

nnpfc_interpolated_pics[ i ] specifies the number of interpolated pictures generated by the NNPF between the i-th and the ( i + 1 )-th input picture for the NNPF. The value of nnpfc_interpolated_pics[ i ] shall be in the range of 0 to 63, inclusive. When the nnpfc_interpolated_pics[ i ] syntax elements are present, the value of nnpfc_interpolated_pics[ i ] shall be greater than 0 for at least one value of i in the range of 0 to nnpfc_num_input_pics_minus1 − 1, inclusive.

NOTE 4 – When PictureRateUpsamplingFlag is equal to 1 for an NNPF and the NNPFA SEI message that activated this NNPF has nnpfa_persistence_flag equal to 1, only for a single value of i in the range of 0 to numInputPics − 1, inclusive, the value of nnpfc_interpolated_pics[ i ] is greater than 0.

nnpfc_extrapolated_pics_minus1 plus 1 specifies the number of extrapolated pictures generated by the NNPF subsequent to all input pictures for the NNPF in output order. The value of nnpfc_extrapolated_pics_minus1 shall be in the range of 0 to 62, inclusive.

The variables NumInpPicsInOutputTensor, specifying the number of pictures that have a corresponding input picture and are present in the output tensor of the NNPF, InpIdx[ idx ], specifying the input picture index, to the list of input pictures in reverse output order, of the idx-th picture that is present in the output tensor of the NNPF and has a corresponding input picture, and numPicsInOutputTensor, specifying the total number of pictures present in the output tensor of the NNPF, are derived as follows:

for( i = 0, numPicsInOutputTensor = 0; i < numInputPics; i++ )
	if( nnpfc_input_pic_filtering_flag[ i ] ) {
		InpIdx[ numPicsInOutputTensor ] = i
		numPicsInOutputTensor++
	}						(79)
NumInpPicsInOutputTensor = numPicsInOutputTensor
if( PictureRateUpsamplingFlag )
	for( i = 0; i  <=  numInputPics − 2; i++ )
		numPicsInOutputTensor  +=  nnpfc_interpolated_pics[ i ]
if( TemporalExtrapolationFlag )
	numPicsInOutputTensor  +=  nnpfc_extrapolated_pics + 1

## Constraint to avoid output order conflicts

### 8.28.1	General post-processing filtering process using NNPFs

#### 8.28.1.1	General

…

For any particular pair of pictures inputPicA and inputPicB consecutive in output order in CroppedDecodedPictures, when there are one or more pictures interpolatedPicSetA intermediatePicSetA in ListNnpfOutputPics between inputPicA and inputPicB in output order, one and only one of the following shall apply:

–	tThe pictures in interpolatedPicSetA intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with PictureRateUpsamplingFlag equal to 1 when a particular picture currPicA in CroppedDecodedPictures was the current picture.

–	The pictures in intermediatePicSetA shall be among the pictures that were output by applying a particular NNPF nnpfA with TemporalExtrapolationFlag equal to 1 when a particular picture currPicA in CroppedDecodedPictures was the current picture.

The application of any other NNPF that was used in the filtering process for one picture when currPicA was the current picture or the application of any NNPF (including nnpfA) that was used in the filtering process for one picture when any other picture currPicB in CroppedDecodedPictures was the current picture shall not output any picture between the inputPicA and inputPicB in output order.

# References

- S. Oprea et al., "A review of deep learning techniques for video prediction", IEEE Trans. on Pattern Analysis and Machine Intelligence, vol. 44, no. 6, Jun. 2022. https://ieeexplore.ieee.org/document/9294028

- J. Guo et al., "ExtraNet: real-time extrapolated rendering for low-latency temporal supersampling", ACM Trans. on Graphics. vol. 40, no. 6, Dec. 2021. https://dl.acm.org/doi/abs/10.1145/3478513.3480531

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).