| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0092-v2 |
| --- | --- |

| Title: | EE2-1.20: TIMD fusion with non-angular predictor |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon, Médéric Blestel | Email: | pandrivon@ofinno.com mblestel@ofinno.com |
| Source: | Ofinno |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2 Test 1.20a and Test 1.20b proposed in JVET-AF0053. The method proposes a non-angular intra predictor (among DC or Planar) in the TIMD fusion process, additionally to the two selected TIMD modes. Two tests are conducted in this EE. In Test 1.20 a, a non-angular predictor is introduced in the TIMD fusion process additionally to the other selected TIMD modes. In Test 1.20b, additionally to Test 1.20a, TIMD predictors are blended using a location-dependent sample-based fusion process similarly to DIMD (JVET-AC0046). On top of ECM-11.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

Test 1.20a

- AI: -0.04%, 0.01%, -0.03%  {100.3%, 100.3%}

- RA: 0.00%, -0.04%, -0.04%  {99.6%, 99.8%}

Test 1.20b

- AI: -0.06%, -0.04%, -0.03%  {100.2%, 100.1%}

- RA: 0.01%, 0.00%, -0.04%  {99.8%, 98.9%}

# Introduction

The Template-based Intra Mode Derivation (TIMD) process [1] is based on a template cost analysis in which several intra modes -- from a candidates list comprising MPMs, wide-angular, DC, horizontal, vertical, and extended modes -- are tested. The two intra modes (IPM1 and IPM2) providing the smallest SATD costs are selected and blended in a fusion process according to their respective template cost under the condition that SATDIPM2<2.SATD(IPM1).

# Proposed methods

JVET-AF0053 [2] introduces a third non-angular intra prediction mode (IPMNA) in the TIMD fusion process.

## Test 1.20a – TIMD fusion with non-angular predictor

The non-angular intra mode IPMNA (among DC and Planar) with the lowest SATD cost is determined first and added to the TIMD fusion process if it is different from the two selected TIMD intra modes (IPM1 and IPM2) and SATDIPMNA<k.SATDIPM1 with k=1.5. Thus, the logic behind the non-angular prediction activation condition is based on already computed SATD costs.

The weights wi used during the TIMD fusion process are computed from SATD cost:

wi=sumSATD-SATD(IPMi)2×sumSATD

with

sumSATD=j=13SATD(IPMj)

## Test 1.20b - TIMD fusion with non-angular predictor and sample-based fusion

In addition to Test 1.20a, location-dependent sample-based blending of the DIMD fusion process [1] is re-used but the location-dependent criterion applying to amplitudes of the selected predictors is replaced by a SATD cost-based criterion. The location-dependent criterion is determined from a ratio of the normalized SATD of the selected TIMD predictors computed in ABOVE and LEFT template area, as follows:

If norSATDA<12 .norSATDL, locDepi=1 (vertical)

Else if norSATDL<12 .norSATDA, locDepi=2 (horizontal)

Otherwise, locDepi=0 (diagonal)

where:

- locDepi is the location-dependent parameter value associated with the i-th selected TIMD mode (e.g. i belongs to [0;2]). A value of 0 indicates no location-dependency, a value of 1 indicates vertical location-dependency of the i-th selected TIMD mode, a value of 2 indicates horizontal location-dependency of the i-th selected TIMD mode,

- norSATDA is the normalized SATD cost associated to a selected TIMD candidate and computed in ABOVE template:

norSATDA=1hT×wCU

- norSATDL is the normalized SATD cost associated to a selected TIMD candidate and computed in LEFT template:

norSATDL=1wT×hCU

with:

- hT, the height of the (ABOVE) template area,

- wT, the width of the (LEFT) template area,

- hCU, the height of the current CU,

- wCU, the width of the current CU.

Furthermore, CU size adaptations are undertaken. The maximum weight deviation of the sample-based blending (range ∆i) becomes CU-size dependant where bigger CUs (greater or equal to 128 samples) have less permitted deviations (maximum deviation is equal to 8 rather than 10).

Eventually, if one of the two templates is not available, the location-dependency state is set accordingly to the present template. In case of LEFT template presence only, the location-dependency is set to 2 (horizontal), otherwise in case of ABOVE template presence only, the location-dependency is set to 1 (vertical).

# Experimental results

The tables below summarize the coding efficiency, runtime impact, and peak memory consumption of EE2 Test 1.20a and Test 1.20b in ECM 11.0 using the JVET common test conditions [3].

## Test 1.20a

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06% | -0.07% | -0.06% | 101.0% | 101.2% | 98.3% | 100.2% |
| Class A2 | -0.05% | 0.00% | 0.04% | 100.5% | 101.0% | 99.7% | 100.3% |
| Class B | -0.04% | -0.01% | -0.12% | 99.4% | 100.1% | 102.4% | 100.1% |
| Class C | -0.04% | -0.05% | 0.03% | 100.9% | 100.0% | 100.6% | 100.0% |
| Class E | -0.03% | 0.21% | 0.02% | 100.3% | 99.4% | 102.9% | 100.0% |
| Overall | -0.04% | 0.01% | -0.03% | 100.3% | 100.3% | 101.0% | 100.1% |
| Class D | -0.06% | -0.08% | -0.09% | 100.0% | 100.3% | 100.1% | 100.0% |
| Class F | 0.01% | -0.10% | 0.11% | 100.3% | 100.4% | 100.0% | 100.0% |

Table 1. Performance of the proposed method on top of ECM-11.0 (AI)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.08% | 0.01% | 98.7% | 99.6% | 100.1% | 99.7% |
| Class A2 | 0.04% | 0.04% | 0.05% | 98.8% | 99.4% | 100.3% | 100.1% |
| Class B | 0.00% | -0.06% | -0.13% | 100.5% | 100.5% | 99.9% | 99.8% |
| Class C | 0.00% | -0.06% | -0.03% | 99.8% | 99.5% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | -0.04% | -0.04% | 99.6% | 99.8% | 100.1% | 99.9% |
| Class D | -0.02% | 0.09% | -0.01% | 99.3% | 100.2% | 99.8% | 100.0% |
| Class F | -0.07% | -0.09% | -0.07% | 99.1% | 99.4% | 100.0% | 100.2% |

| Random Access Main 10. Over ECM-11.0. Y; U; V; EncT; DecT; EncVmPeak; DecVmPeak. Class A1; -0.03%; -0.08%; 0.01%; 98.7%; 99.6%; 100.1%; 99.7%. Class A2; 0.04%; 0.04%; 0.05%; 98.8%; 99.4%; 100.3%; 100.1%. Class B; 0.00%; -0.06%; -0.13%; 100.5%; 100.5%; 99.9%; 99.8%. Class C; 0.00%; -0.06%; -0.03%; 99.8%; 99.5%; 100.0%; 100.0%. Class E. Overall; 0.00%; -0.04%; -0.04%; 99.6%; 99.8%; 100.1%; 99.9%. Class D; -0.02%; 0.09%; -0.01%; 99.3%; 100.2%; 99.8%; 100.0%. Class F; -0.07%; -0.09%; -0.07%; 99.1%; 99.4%; 100.0%; 100.2%. Random Access Main 10. Over ECM-11.0. Y; U; V; EncT; DecT; EncVmPeak; DecVmPeak. Class A1; -0.03%; -0.08%; 0.01%; 98.7%; 99.6%; 100.1%; 99.7%. Class A2; 0.04%; 0.04%; 0.05%; 98.8%; 99.4%; 100.3%; 100.1%. Class B; 0.00%; -0.06%; -0.13%; 100.5%; 100.5%; 99.9%; 99.8%. Class C; 0.00%; -0.06%; -0.03%; 99.8%; 99.5%; 100.0%; 100.0%. Class E. Overall; 0.00%; -0.04%; -0.04%; 99.6%; 99.8%; 100.1%; 99.9%. Class D; -0.02%; 0.09%; -0.01%; 99.3%; 100.2%; 99.8%; 100.0%. Class F; -0.07%; -0.09%; -0.07%; 99.1%; 99.4%; 100.0%; 100.2%. |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |

Table 2. Performance of the proposed method on top of ECM-11.0 (RA)

## Test 1.20b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.07% | -0.13% | -0.05% | 100.6% | 101.1% | 99.8% | 100.4% |
| Class A2 | -0.05% | -0.03% | 0.05% | 100.3% | 99.6% | 100.5% | 100.0% |
| Class B | -0.06% | -0.05% | -0.14% | 99.5% | 99.6% | 99.9% | 100.0% |
| Class C | -0.07% | -0.01% | 0.04% | 100.5% | 100.6% | 100.6% | 100.0% |
| Class E | -0.05% | 0.02% | -0.01% | 100.3% | 100.0% | 102.1% | 100.0% |
| Overall | -0.06% | -0.04% | -0.03% | 100.2% | 100.1% | 100.5% | 100.1% |
| Class D | -0.04% | -0.18% | -0.08% | 100.2% | 99.3% | 100.0% | 100.0% |
| Class F | 0.05% | 0.03% | 0.02% | 100.4% | 100.6% | 100.0% | 100.0% |

Table 3. Performance of the proposed method on top of ECM-11.0 (AI)

|  | Random Access Main 10 |  |  |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |  |  |  |  |  |  |
|  | Y | U |  | V |  | EncT |  | DecT |  | EncVmPeak |  | DecVmPeak |  |
| Class A1 | -0.02% | 0.01% |  | 0.04% |  | 99.0% |  | 99.7% |  | 100.3% |  | 100.0% |  |
| Class A2 | 0.03% | 0.03% |  | 0.09% |  | 98.7% |  | 99.0% |  | 100.2% |  | 100.2% |  |
| Class B | 0.02% | 0.07% |  | -0.17% |  | 100.6% |  | 98.7% |  | 100.2% |  | 100.0% |  |
| Class C | 0.00% | -0.12% |  | -0.03% |  | 100.1% |  | 98.6% |  | 100.0% |  | 100.0% |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall | 0.01% | 0.00% |  | -0.04% |  | 99.8% |  | 98.9% |  | 100.2% |  | 100.1% |  |
| Class D | 0.00% | -0.20% |  | -0.39% |  | 99.7% |  | 98.2% |  | 99.7% |  | 100.0% |  |
| Class F | -0.05% | 0.17% |  | 0.10% |  | 99.5% |  | 98.1% |  | 100.2% |  | 100.4% |  |

Table 4. Performance of the proposed method on top of ECM-11.0 (RA)

# References

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 11 (ECM 11),” JVET-AF2025, October 2023.

- P. Andrivon, M. Blestel, “EE2-related: Test 2.5a and TIMD fusion improvement”, JVET-AF0053, October 2023.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, October 2023.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).