| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0095-v1 |
| --- | --- |

| Title: | EE2-1.22c: Combination of EE2-1.22b and EE2-1.4 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Pierre Andrivon Médéric Blestel, Chuan Zhou, Zhuoyi Lv, K. Naser, F. Le Léannec | Email: | pandrivon@ofinno.com chuan.zhou@vivo.com karam.naser@interdigital.com |
| Source: | Ofinno, vivo, InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results for EE2 Test 1.22c. On top of ECM-11.0, the reported PSNR-Y, Cb, Cr, BD-rate and {EncT, DecT} results are as follows:

- AI: -0.11%, -0.09%, -0.03%  {100.1%, 100.3%}

- RA: X%, X%, X%  {X%, X%}

# Introduction

In Test 1.22c, a combination of Test 1.22b [1] and Test 1.4 [2] is tested. This test proposes the following modifications to the TIMD and DIMD fusion processes:

- TIMD/DIMD intra prediction modes may select IntraTMP/IBC Block Vector (Test 1.22b + Test 1.4)

- a non-angular intra prediction mode (among DC, Planar or the best IntraTMP/IBC Block Vector for TIMD/DIMD modes) is conditionally introduced (Test 1.20a + Test 1.4)

- location sample-based blending for TIMD is used similarly to DIMD (Test 1.20b and Test 1.22b)

- reference lines for TIMD are adjusted (Test 1.21 and Test 1.22b)

# Experimental results

The tables below summarize the coding efficiency, runtime impact, and peak memory consumption of EE2 Test 1.22c in ECM 11.0 using the JVET common test conditions [3].

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.09% | -0.04% | 0.02% | 100.7% | 99.8% | 98.7% | 100.4% |
| Class A2 | -0.12% | -0.09% | 0.00% | 100.7% | 100.3% | 100.2% | 100.2% |
| Class B | -0.11% | -0.05% | -0.08% | 99.6% | 100.3% | 97.4% | 100.2% |
| Class C | -0.10% | -0.14% | -0.03% | 100.0% | 101.0% | 99.7% | 100.1% |
| Class E | -0.14% | -0.12% | 0.01% | 100.1% | 100.0% | 99.0% | 100.0% |
| Overall | -0.11% | -0.09% | -0.03% | 100.1% | 100.3% | 98.9% | 100.2% |
| Class D | -0.10% | -0.15% | -0.02% | 98.9% | 100.2% | 100.4% | 100.0% |
| Class F | -0.06% | -0.15% | 0.10% | 100.3% | 100.6% | 100.1% | 100.1% |

Table 1. Performance of the proposed method on top of ECM-11.0 (AI)

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #DIV/0! |
| Class A2 | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class B | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #DIV/0! |
| Class C | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #DIV/0! |
| Class E |  |  |  |  |  |  |  |
| Overall | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
| Class D | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #DIV/0! |
| Class F | #VALUE! | #VALUE! | #VALUE! | #NUM! | #NUM! | #NUM! | #DIV/0! |

Table 2. Performance of the proposed method on top of ECM-11.0 (RA)

# References

- P. Andrivon, M. Blestel, C. Zhou, Z. Lv, “EE2-1.22b: Combination of EE2-1.20b and EE2-1.21”, JVET-AG0094, January 2024.

- F. Le Léannec, K. Naser, G. Rath, T. Dumas, P. Bordes, Y. Chen, “EE2-1.4: IntraTMP extension to DIMD”, JVET-AGxxxx, January 2024.

- M. Coban, R.-L. Liao, K. Naser, J. Ström, L. Zhang, “Algorithm description of Enhanced Compression Model 11 (ECM 11)”, JVET-AF2025, October 2023.

# Patent rights declaration(s)

Ofinno LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).