| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0101-v1 |
| --- | --- |

| Title: | AHG9: Film grain adaptive SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ying Gao Shaowei Xie Yaxian Bai Menghu Jia Cheng Huang Ping Wu | Tel: Email: | gao.ying@zte.com.cn ping.wu@zte.com.cn |
| Source: | ZTE |  |  |

_____________________________

- Abstract

It is proposed to define a new film grain adaptive SEI message to specify the regions that the film grain applies, wherein:

- these regions may include the external regions or/and the internal regions.

- external regions refer to the existing available regions defined through different type of SEI messages or other approach.

- internal regions refer to the regions defined in the film grain adaptive SEI message.

Introduction

In the 32nd JVET meeting, the contribution JVET-AF0142 [1] proposes to address the film grain synthesis applied locally on the picture by using the annotated regions as a mask and the contribution JVET-AF0144 [2] proposes to address the film grain synthesis applied locally on a per image basis by using the auxiliary data as a mask. Furthermore, the conclusions recorded in the meeting notes [3] confirm that the functionality as such was assessed to be useful and suggest to study the possibility of defining a new SEI message rather than changing an existing one.

Since it has been recognized that film grain may not necessarily be applicable to the entire picture but only in some parts of the picture, such as one or more annotated regions, forefront objects, etc., it would be a simple approach if these existing available regions can be used in the newly defined SEI message.

On the other hand, these existing available regions may not meet all the requirements, so it is also necessary to provide a way to describe the film grain region in the newly defined SEI message.

Proposed Solution

It is proposed to define a new film grain adaptive SEI message to specify the regions that the film grain applies, wherein:

- these regions may include the external regions or/and the internal regions.

- external regions refer to the existing available regions defined through different type of SEI messages or other approach.

- internal regions refer to the regions defined in the film grain adaptive SEI message.

Proposal

Proposed film grain adaptive SEI message syntax

| film_grain_adaptive( payloadSize ) { | Descriptor |
| --- | --- |
| fga_region_external_flag | u(1) |
| fga_region_internal_flag | u(1) |
| if(fga_region_external_flag){ |  |
| fga_num_region_external_minus1 | ue(v) |
| for(i=0; i<=fga_num_regions_external_minus1;i++){ |  |
| fga_region_source[i] | u(2) |
| fga_region_idx_present_flag | u(1) |
| if(fga_region_idx_present_flag) |  |
| fga_region_external_idx[i] | ue(v) |
| } |  |
| } |  |
| if (fga_region_internal_flag){ |  |
| fga_num_region_internal_minus1 | ue(v) |
| for(i=0; i<=fga_num_regions_internal_minus1;i++){ |  |
| fga_region_internal_top[i] | u(16) |
| fga_region_internal_left[i] | u(16) |
| fga_region_internal_width[i] | u(16) |
| fga_region_internal_height[i] | u(16) |
| } |  |
| } |  |
| } |  |

Proposed film grain adaptive SEI message semantics

The Film Grain Adaptive (FGA) SEI message carries parameters that identify the regions where the film grain synthesized based on the film grain characteristics SEI message should applicable to:

fga_region_external_flag equal to 1 indicates that the regions include external regions which are existing available regions defined through different type of SEI messages or other approach. fga_region_external_flag equal to 0 indicates that the regions do not include external regions.

fga_region_internal_flag equal to 1 indicates that the regions include internal regions which are defined in the film grain adaptive SEI message. fga_region_internal_flag equal to 0 indicates that the regions do not include internal regions.

fga_num_region_external_minus1 plus 1 specifies the number of external regions.

fga_region_source[i] indicates the source used to identify the i-th external region as specified in Table 1. The value of fga_region_source[i] shall be in the range of 0 to 4, inclusive. The values of 2 and 3 for fga_region_source[i] are reversed for future use.

Table 1 - fga_region_source[i] values

| Value | Description |
| --- | --- |
| 0 | Annotated region |
| 1 | Alpha channel information |
| 2...3 | Reserved |

When fga_region_source[i] is equal to 0, the decoder could determine the range of the i-th external region based on the Annotated Region SEI message.

When fga_region_source[i] is equal to 1, the decoder could determine the range of the i-th external region based on the Alpha Channel Information SEI message.

fga_region_idx_present_flag equal to 1 indicates that the fga_region_external_idx[i] syntax element is present. fga_region_idx_present_flag equal to 0 indicates that the fga_region_external_idx[i] syntax element is not present.

fga_region_external_idx[i](when present) specifies the index of the i-th external region.

When fga_region_source[i] is equal to 0, the following constraints should be present:

- ar_object_idx[ j ] in the Annotated Region SEI message is equal to fga_region_external_idx[i].

- The bounding box of the ar_object_idx[ j ] specified by the ar_bounding_box_top[ ar_object_idx[ j ] ], ar_bounding_box_left[ ar_object_idx[ j ] ], ar_bounding_box_width[ ar_object_idx[ j ] ], and ar_bounding_box_height[ ar_object_idx[j ] ] is the i-th external region.

fga_num_region_internal_minus1 plus 1 specifies the number of internal regions.

fga_region_internal_top[i], fga_region_internal_left[i], fga_region_internal_width[i], and fga_region_internal_height[i] specify the coordinates of the top-left corner and the width and height, respectively, of the i-th internal region

Proposed additions to the VVC or VSEI specification

It is proposed to add the following highlighted to the VVC or VSEI specification [4]:

Payload type

In subclause D.2.1of VVC specification, add the following table row:

| sei_payload( payloadType, payloadSize ) { | Descriptor |
| --- | --- |
| if( nal_unit_type = = PREFIX_SEI_NUT ) |  |
| … |  |
| else if( payloadType = = *** ) /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| film_grain_adaptive ( payloadSize ) |  |
| else /* Specified in Rec. ITU-T H.274 ¦ ISO/IEC 23002-7 */ |  |
| reserved_message( payloadSize ) |  |
| ... |  |
| } |  |

Persistence scope

In subclause D.2.2 of VVC specification, add the following table row in Table D-1:

Table D.1 – Persistence scope of SEI messages (informative)

| SEI message | Persistence scope |
| --- | --- |
| ... |  |
| film grain adaptive | The PU containing the SEI message |

Film grain characteristics SEI message

In subclause 8.5.2of VSEI specification( Film grain characteristics SEI message semantics ), add the following text:

8.5.2 Film grain characteristics SEI message semantics

This SEI message provides the decoder with a parameterized model for film grain synthesis.

NOTE 1–For example, an encoder could use the film grain characteristics SEI message to characterize film grain that was present in the original source video material and was removed by pre-processing filtering techniques. Synthesis of simulated film grain on the decoded images for the display process is optional and does not need to exactly follow the specified semantics of the film grain characteristics SEI message. When synthesis of simulated film grain on the decoded images for the display process is performed, there is no requirement that the method by which the synthesis is performed be the same as the parameterized model for the film grain as provided in the film grain characteristics SEI message.

NOTE 2–The display process is not specified in this Specification.

NOTE 3–SMPTE RDD 5 specifies a film grain simulator based on the information provided in the film grain characteristics SEI message.

NOTE 4–If the current PU contains a film grain adaptive (FGA) SEI message, then the simulated film grain values are only applied to those regions specified in the film grain adaptive (FGA) SEI message.

...

fg_blending_mode_id identifies the blending mode used to blend the simulated film grain with the input images as specified in Table 6. fg_blending_mode_id shall be in the range of 0 to 1, inclusive. The values of 2 and 3 for fg_blending_mode_id are reserved for future use by ITU-T | ISO/IEC and shall not be present in bitstreams conforming to this version of this Specification. Decoders shall ignore film grain characteristic SEI messages with fg_blending_mode_id equal to 2 or 3.

Table 6 – fg_blending_mode_id values

| Value | Description |
| --- | --- |
| 0 | Additive |
| 1 | Multiplicative |

Depending on the value of fg_blending_mode_id, the blending mode is specified as follows:

–	If fg_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] + G[ c ][ x ][ y ] )	(21)

–	Otherwise (fg_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] +	(22)
			Round( ( Î[ c ][ x ][ y ] * G[ c ][ x ][ y ] ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

where Î[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the input image Î, G[ c ][ x ][ y ] is the simulated film grain value at the same position and colour component, and fgBitDepth[ c ] is the number of bits used for each sample in a fixed-length unsigned binary representation of the arrays Igrain[ c ][ x ][ y ], Î[ c ][ x ][ y ], and G[ c ][ x ][ y ], where c = 0..2, x = 0..PicWidthInLumaSamples − 1, and y = 0..PicHeightInLumaSamples − 1.

If the current PU contains a film grain adaptive (FGA) SEI message with fga_region_source equal to 1, the simulated film grain value shall be weighted by the auxiliary data value as follow:

–	If fg_blending_mode_id is equal to 0, the blending mode is additive as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] + G[ c ][ x ][ y ]* Iaux[ c ][ x ][ y ])

–	Otherwise (fg_blending_mode_id is equal to 1), the blending mode is multiplicative as specified by:

Igrain[ c ][ x ][ y ] = Clip3( 0, ( 1  <<  fgBitDepth[ c ] ) − 1, Î[ c ][ x ][ y ] +
			Round( ( Î[ c ][ x ][ y ] * G[ c ][ x ][ y ]* Iaux[ c ][ x ][ y ]) ) ÷ ( ( 1  <<  fgBitDepth[ c ] ) − 1 ) ) )

Where Iaux[ c ][ x ][ y ] represents the sample value at coordinates x, y of the colour component c of the decoded auxiliary picture based on the ACI SEI message associated with the current PU.

Reference

- G. Teniou, S. Wenger, [AHG9] Local Film Grain Synthesis using Annotated Regions SEI, JVET-AF0142, Oct 2023.

- G. Teniou, S. Wenger “[AHG9] Adaptive Film grain synthesis using Alpha Channel Information SEI message” JVET-AF0144, Oct 2023.

- J.-R. Ohm, Meeting Report of the 32nd JVET Meeting, AF1000, Nov 2023.

- Recommendation ITU-T H.274 (V2) Versatile supplemental enhancement information messages for coded video bitstream, May 2022.

Patent rights declaration(s)

ZTE Corporation may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).