| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0108-v1 |
| --- | --- |

| Title: | AhG12 Entropy coding extension |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Galpin, F. Lo Bianco C. Salmon-Legagneur M. Balcilar | Author(s) or Contact(s): | franck.galpin@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution proposes to add more flexibility to the bins coding in current ECM. First, some bins coded in EP are transformed to be coded with an associated context. Second, an additional coding type is introduced in between the CABAC and the EP coding where the encoding uses a fixed probability without adaptation. The method is tested on some bins which were coded in EP. It is reported that on top of ECM-11.0, the overall coding performance impact for {Y, U, V} is {0.00% -0.11% -0.15%} {-0.08% -0.22% -0.23%} in AI and RA configurations respectively.

# Introduction

In current ECM, slice data can be coded either using CABAC coding, where a context is associated to the bin to encode and EP coding.

In some cases, the fixed equa-probability of EP coding might not be optimal. It is proposed to add a mode where the probability depends on the syntax element to encode (Non Equi-Probable).

In order to choose when to use this mode, EP coded bins are changed in order to get an associated context. The change does not impact the encoding and decoding of current ECM-11.0 but allows to alter choose which bins can use CABAC, EP or NEP.

# Simulations

In the simulations, only bins originally in EP were retrained. In total, 366 new contexts are added. After training, 64 contexts are still using EP coding while 302 uses the NEP coding.

|  | All Intra Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | 0.00% | -0.16% | -0.27% | 103.4% | 99.9% | 102.0% |
| Class A2 | 0.01% | -0.19% | -0.11% | 101.8% | 96.5% | 101.4% |
| Class B | 0.00% | -0.06% | -0.22% | 102.0% | 98.4% | 96.1% |
| Class C | 0.00% | -0.18% | -0.12% | 110.3% | 94.1% | 101.7% |
| Class E | -0.02% | 0.02% | 0.03% | 101.6% | 100.0% | 101.6% |
| Overall | 0.00% | -0.11% | -0.15% | 103.9% | 97.6% | 100.1% |
| Class D | -0.02% | -0.21% | -0.13% | 102.3% | 99.6% | 99.6% |
| Class F | 0.06% | -0.08% | 0.13% | 103.8% | 101.3% | 97.2% |
| Class TGM | 0.13% | 0.12% | 0.05% | 103.4% | 100.1% | 99.8% |

|  | Random Access Main 10 |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | VmPeak |
| Class A1 | -0.07% | -0.32% | -0.10% | 92.2% | 85.1% | 101.5% |
| Class A2 | -0.04% | -0.22% | -0.21% | 93.4% | 92.6% | 102.2% |
| Class B | -0.10% | -0.17% | -0.35% | 94.6% | 91.2% | 100.1% |
| Class C | -0.10% | -0.20% | -0.19% | 100.5% | 96.3% | 100.0% |
| Class E |  |  |  |  |  |  |
| Overall | -0.08% | -0.22% | -0.23% | 95.4% | 91.5% | 100.8% |
| Class D | -0.06% | -0.33% | -0.59% | 99.5% | 97.0% | 99.2% |
| Class F | -0.03% | -0.11% | -0.08% | 99.0% | 97.1% | 100.5% |
| Class TGM | -0.07% | -0.21% | -0.07% | 97.7% | 95.3% | 101.7% |

# Conclusion

This contribution proposes to add more flexibility to the current bin coding in ECM. First a context is associated with EP bins in order to allow potential retraining of these particular context. Second, a new mode, less complex than CABAC is introduced. The changes allow a gain of coding performance of {-0.08% -0.22% -0.23%} for {Y, U, V} in RA configuration. It is proposed to study this contribution for next EE stage.

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).