| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17-26 January 2024 | Document: JVET-AG0114-v2 |
| --- | --- |

| Title: | AHG11: On new input and backbone enhancement of model for super-resolution |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jiedong Ye, Xu Li, Yiqing Zhu, Qiong Liu (HUST) Chuan Zhou, M. Rafie, Zhuoyi Lv (vivo) | Email: | ye_jd@hust.edu.cn chuan.zhou@vivo.com |
| Source: | Huazhong University of Science and Technology vivo Mobile Communication Co., Ltd |  |  |

_____________________________

- Abstract

In this contribution, we propose 2 methods to further improve the coding performance of the super-resolution (SR) model. And we design our SR network with reference to the unified model HOP of in-loop filter, which was defined in JVET-AD0380 [1]. First, the SR models in NNVC use reconstruction and multiple side information including prediction, slice QP, base QP and slice type as the inputs of the network. We add reference picture resampling (RPR) picture as a new input to the neural network. Second, to preprocess the shallow features and improve the diversity of the features fed into the backbone, we add enhancement modules before the backbone part.

Considering both flexibility and performance in the use of the model, we conducted experiments under two conditions.

- We train an unified model using both I- and B-frame training data, and the SR of both I- and B-frames in the inference phase is performed by this model. We call this model, which is applied to both I-frame B-frame, IB_model.

- We train two separated models using I- and B-frame training data separately, and SR of I- and B-frames in the inference phase is performed by the respective models. We call them I_model and B_model.

For IB_model, compared with NNVC-7.0-vtm, the results with SADL implementation in float are shown {-5.88%, -3.99%, -2.82%} and {-3.48%, -10.39%, -7.20%} BD-rate savings under AI and RA, respectively.

For I_model and B_model, compared with NNVC-7.0-vtm, the results with SADL implementation in float are shown {-5.96%, -4.15%, -3.07%} and {-3.80%, -10.45%, -7.02%} BD-rate savings under AI and RA, respectively.

The complexity of IB_model is 452kMAC/pixel compared to 469kMAC/pixel of the SR model in NNVC-7.0.

The complexity of I_model and B_model are 447 and 452kMAC/pixel, respectively, compared to 469kMAC/pixel of the SR model in NNVC-7.0.

Introduction

The current SR model in NNVC-7.0 [2] uses reconstruction and multiple side information including prediction, slice QP, base QP and slice type as input. The network learns the difference between the RPR image and the real high-resolution image. In this contribution, to improve the performance of current SR model, we add the RPR image as a new input.

In the original SR model, the shallow feature extraction part uses only one layer of 3×3 convolution and PReLU. In order to provide more diverse shallow features for the deep feature extraction backbone network, we add shallow feature enhancement module (SEM) after the original shallow feature extraction part.

Unlike the current SR model in NNVC, the input interface of our models are aligned to the unified in-loop filter, in which components are unified.

Proposed method

Main idea

Our main idea has two points:

- Introduce RPR as a new input to the SR model;

- Introduce SEM module into the shallow feature extraction part.

We implemented this idea to both IB_model and I_model/B_model.

Network architecture of IB_model

In original Luma_Intra model, reconstruction Ylrrec, prediction Ylrpre and slice QP are fed into network. The RPR image is only used at the end of the network as a baseline for upsampling results.

Figure 1 IB_model network architecture

The unified SR network architecture is shown in Figure 1. The red numbers indicate the number of channels in the branch. We believe that the content of RPR images can also provide valid information for SR networks. Therefore, we feed it as a new additional input to the network, similar to Ylrrecand Ylrpre, also extract its shallow features using 1-layer 3×3 convolution and PReLU. Considering the resolution of the RPR image, the stride of its convolution is set to 2. Then all the shallow features are concatenated together and fed into the deep feature extraction network.

The SEM aims to fully explore high-frequency texture information of input images. Inspired by the mechanism of multi-head attention, the input images are fed into several different number of convolution layers to focus on different size of targets in the image. Meanwhile these different branches of features are learned and processed independently by the network, which can improve the feature diversity. In addition to this we have removed base QP.

Network architecture of I_model/B_model

Figure 2 I_model/B_model network architecture

In the I_model and B_model, we remove the slice type which is used to distinguish I and B frames, otherwise the network structure is basically the same as the unified model. The difference is that the I model has more channels in the prediction branch and less channels in the rpr branch. I_model and B_model will be trained using I-frame and B-frame data respectively.

Training stage information

Table 1 Network Information for one unified model

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1100 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~7 days |
|  | Training data information: | TVD_4K, BVI-DVC_4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0_nnvc-6.0, QP{17, 22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 72*72 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy | Reduced 3 times in the later part of training |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flip, rotation |
|  | Mini-batch selection process: | random cropping |
|  | Training data update strategy: |  |
|  | Other information: |  |

Table 2 Network Information for two models

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: GeForce RTX 3090 |
|  | Framework: | PyTorch v1.9.0 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | ~1100 |
|  | Batch size: | 64 |
|  | Loss function: | L1 |
|  | Training time (for 1 model): | ~7 days |
|  | Training data information: | TVD_4K, BVI-DVC_4K |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | VTM-11.0_nnvc-6.0, QP{17, 22, 27, 32, 37, 42} |
| Optional | Number of iterations |  |
|  | Patch size | 72*72 |
|  | Learning rate: | 1e-4 |
|  | Learning rate update strategy | Reduced 3 times in the later part of training |
|  | Optimizer: | ADAM |
|  | Preprocessing: | flip, rotation |
|  | Mini-batch selection process: | random cropping |
|  | Training data update strategy: |  |
|  | Other information: |  |

Inference stage information

Table 3 Inference information for IB_model

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | Not used |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.816M |
|  | Total Number of Parameters (All Models) | 1.816M |
|  | Parameter Precision (Bits) | float32 |
|  | Memory Parameter (MB) | 6.97 MB |
|  | Multiply Accumulate (MAC/pixel) | 452.603K |
|  | Calculation Method | thop |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128*128 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Table 4 Inference information for two models

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: |  |
|  | GPU Type | Not used |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.795M(I model), 1.815M(B model) |
|  | Total Number of Parameters (All Models) | 3.61M |
|  | Parameter Precision (Bits) | float32 |
|  | Memory Parameter (MB) | 6.89 MB(I model), 6.96 MB(B model) |
|  | Multiply Accumulate (MAC/pixel) | 447.443K(I model), 452.339K(B model) |
|  | Calculation Method | thop |
| Optional | Total Conv. Layers |  |
|  | Total FC Layers |  |
|  | Total Memory (MB) |  |
|  | Batch size: |  |
|  | Patch size | 128*128 |
|  | Changes to network configuration or weights required to generate rate points |  |
|  | Peak Memory Usage (Total) |  |
|  | Peak Memory Usage (per Model) |  |
|  | Border handling |  |
|  | Other information: |  |

Experiment results

This section will show the coding performance of our two sets of experiments in the AI and RA. The proposed CNN-based up-sampling filter is tested on top of NNVC 7.0, according to the common test conditions [3]. The tested QP values are {22, 27, 32, 37, 42}.

IB_model

Complexity information:

Number of Parameters: 1.816M; Multiply Accumulate: 452.603kMAC/pixel.

Table 5 Results of IB_model in RA configurations

| Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BD-rate Over VTM NNVC-7.0_vtm_anchor |  |  |  |  |
| Test Sequence |  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | Tango2 | -12.12% | -23.53% | -15.28% |
|  | FoodMarket4 | -8.80% | -6.81% | -8.41% |
|  | Campfire | -3.64% | 0.11% | -2.89% |
| Class A2 | CatRobot | -3.85% | -5.75% | 0.19% |
|  | DaylightRoad2 | -1.88% | -1.20% | 0.55% |
|  | ParkRunning3 | -4.99% | 13.23% | 8.94% |
| Average of A1 |  | -8.19% | -10.07% | -8.86% |
| Average of A2 |  | -3.57% | 2.10% | 3.23% |
| Average of A1 & A2 |  | -5.88% | -3.99% | -2.82% |

Table 6 Results of IB_model in AI configurations

| All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BD-rate Over VTM NNVC-7.0_vtm_anchor |  |  |  |  |
| Test Sequence |  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | Tango2 | -9.86% | -21.88% | -17.32% |
|  | FoodMarket4 | -7.34% | -4.90% | -6.56% |
|  | Campfire | -0.09% | 0.11% | -0.06% |
| Class A2 | CatRobot | -2.78% | -15.44% | -8.94% |
|  | DaylightRoad2 | 6.57% | -30.26% | -16.78% |
|  | ParkRunning3 | -7.36% | 10.03% | 6.44% |
| Average of A1 |  | -5.77% | -8.89% | -7.98% |
| Average of A2 |  | -1.19% | -11.89% | -6.43% |
| Average of A1 & A2 |  | -3.48% | -10.39% | -7.20% |

I_model and B_model

Complexity information:

I_model: Number of Parameters: 1.795M; Multiply Accumulate: 447.443kMAC/pixel.

B_model: Number of Parameters: 1.815M; Multiply Accumulate: 452.339kMAC/pixel.

Table 7 Results of I_model/B_model in RA configurations

| Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BD-rate Over VTM NNVC-7.0_vtm_anchor |  |  |  |  |
| Test Sequence |  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | Tango2 | -12.48% | -24.18% | -16.37% |
|  | FoodMarket4 | -9.03% | -7.01% | -8.08% |
|  | Campfire | -3.58% | 0.12% | -3.16% |
| Class A2 | CatRobot | -3.82% | -6.20% | -0.36% |
|  | DaylightRoad2 | -1.88% | -1.03% | 0.46% |
|  | ParkRunning3 | -4.99% | 13.41% | 9.09% |
| Average of A1 |  | -8.36% | -10.36% | -9.20% |
| Average of A2 |  | -3.56% | 2.06% | 3.06% |
| Average of A1 & A2 |  | -5.96% | -4.15% | -3.07% |

Table 8 Results of I_model/B_model in AI configurations

| All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BD-rate Over VTM NNVC-7.0_vtm_anchor |  |  |  |  |
| Test Sequence |  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | Tango2 | -10.17% | -21.74% | -18.03% |
|  | FoodMarket4 | -7.62% | -4.90% | -4.95% |
|  | Campfire | -0.09% | 0.10% | -0.08% |
| Class A2 | CatRobot | -3.37% | -15.83% | -8.80% |
|  | DaylightRoad2 | 5.93% | -30.45% | -17.12% |
|  | ParkRunning3 | -7.45% | 10.14% | 6.89% |
| Average of A1 |  | -5.96% | -8.85% | -7.69% |
| Average of A2 |  | -1.63% | -12.05% | -6.35% |
| Average of A1 & A2 |  | -3.80% | -10.45% | -7.02% |

NNVC-7.0 SR model

Complexity information:

Number of Parameters: luma-I: 1.49 M, luma-B: 1.49M, chroma-IB: 1.50M;

Multiply Accumulate: 469kMAC/pixel.

Table 9: Results of the NNVC-7.0 SR model in RA configurations

| Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BD-rate Over VTM NNVC-7.0_vtm_anchor |  |  |  |  |
| Test Sequence |  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | Tango2 | -11.64% | -22.13% | -14.84% |
|  | FoodMarket4 | -7.81% | -3.19% | -6.68% |
|  | Campfire | -2.92% | 0.24% | -2.67% |
| Class A2 | CatRobot | -3.15% | -1.91% | 3.75% |
|  | DaylightRoad2 | -1.63% | -1.64% | 0.64% |
|  | ParkRunning3 | -4.87% | 13.16% | 6.76% |
| Average of A1 |  | -7.45% | -8.36% | -8.06% |
| Average of A2 |  | -3.22% | 3.20% | 3.71% |
| Average of A1 & A2 |  | -5.34% | -2.58% | -2.17% |

Table 10: Results of the NNVC-7.0 SR model in AI configurations

| All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- |
| BD-rate Over VTM NNVC-7.0_vtm_anchor |  |  |  |  |
| Test Sequence |  | Y-PSNR | U-PSNR | V-PSNR |
| Class A1 | Tango2 | -9.54% | -20.39% | -16.97% |
|  | FoodMarket4 | -7.01% | -1.30% | -4.28% |
|  | Campfire | -0.08% | 0.11% | -0.06% |
| Class A2 | CatRobot | -2.94% | -8.91% | 1.57% |
|  | DaylightRoad2 | 6.64% | -29.47% | -15.83% |
|  | ParkRunning3 | -7.48% | 10.46% | 4.96% |
| Average of A1 |  | -5.54% | -7.19% | -7.10% |
| Average of A2 |  | -1.26% | -9.31% | -3.10% |
| Average of A1 & A2 |  | -3.40% | -8.25% | -5.10% |

Summary

For unified IB_model, the performance has improved {-0.54%, -1.41%, -0.65%} and {-0.08%, -2.14%, -2.1%} in {Y, U, V} component under RA and AI configuration compared with NNVC-7.0 SR model.

For separated I_model and B_model, The performance has improved {-0.62%, -1.56%, -0.90%} and {-0.4%, -2.2%, -1.92%} in {Y, U, V} component under RA and AI configuration compared with NNVC-7.0 SR model.

In both RA and AI configurations, the performance of our model is improved in YUV components compared to the SR model in NNVC-7.0. The improvement is especially more significant in the UV component, up to -2.2%.

Conclusion

In this contribution, we propose 2 methods to further improve the coding performance of the super-resolution (SR) model. First we use RPR as a new input to the SR network and second we introduce the SEM module to extract more diverse shallow features.

The results show promising coding performance of the proposed methods under two experimental settings. It is recommended to adopt the proposed methods in NNVC.

Reference

[1] E. Alshina, F. Galpin, “BoG report on NN-filter design unification”, JVET-AD0380

[2] F. Galpin, Y. Li, D. Rusanovskyy, J. Ström, L. Wang, “Algorithm Description for Neural Network-based Video Coding (NNVC-7.0)” JVET-AF2019.

[3] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016.

Patent rights declaration(s)

vivo Mobile Communication Co., Ltd. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).