| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0117-v1 |
| --- | --- |

| Title: | EE2-5.3: Spatial CABAC tuning |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Jani Lainema Alireza Aminlou Pekka Astola Döne Bugdayci Sansli | Tel: Email: | - first.lastname@nokia.com |
| Source: | Nokia |  |  |

_____________________________

# Abstract

This contribution reports results of EE2-5.3a and EE2-5.3b exploration experiments investigating spatial CABAC tuning approach originally proposed in JVET-AE0058 and later in JVET-AF0109. Test 5.3a includes the proposed spatial CABAC tuning approach implemented on top of unmodified ECM-11.0 software, while Test 5.3b provides results of the spatial CABAC tuning approach with the retrained context initialization of Test 5.1. The summarized results are reported as follows.

For test EE2-5.3a the impact on coding efficiency and runtimes over ECM-11.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak , DecVmPeak }:

AI {-0.03%, -0.04%, -0.04%, 99.1%, 100.3%, 99.8%, 99.9%}
RA {-0.04%, -0.14%, -0.14%, 100.0%, 99.6%, 99.8%, 99.9%}
LB {-0.08%, 0.20%, -0.21%, 99.3%, 99.0%, 99.4%, 99.8%}

For test EE2-5.3b the impact on coding efficiency and runtimes over ECM-10.0 are reportedly {for Y, U, V, EncT, DecT, EncVmPeak , DecVmPeak }:

AI {-0.07%, 0.02%, -0.02%, 99.9%, 100.0%, 99.9%, 100.0%}
RA {-0.18%, 0.21%, 0.03%, 99.5%, 100.1%, 100.0%, 100.0%}
LB {-0.14%, 0.44%, 0.32%, 100.2%, 99.2%, 100.1%, 100.1%}

It is further reported that the worst-case memory requirement for the proposed method under CTC conditions is 4587 bytes for both Tests 5.3a and 5.3b.

# Proposed method

## Introduction

It is asserted that the current CABAC context update mechanism may lead to “overfitting” the probability estimates to the statistical properties of the CTU or couple of CTUs that are preceding the current CTU in scan order.

Figure 1 illustrates the processing order of the blocks and related syntax elements in ECM. CTUs are scanned in raster scan order, while CUs inside CTUs are scanned in nested tree order. When starting to process the current CTU (CTUC) the processing “jumps” from the bottom-right CU (yellow CU 8) of CTUL to the top-left CU of the current CTU (blue CU 0 of CTUC). At that stage, the context parameters of the CABAC engine can be expected to be finetuned for the bottom-right area of the previous CTU instead of the ideal case where the context parameters would be finetuned for the actual next CU to be processed.

It is proposed to use the bins of the syntax elements of the bottom CUs of the above CTU (the green CUs of CTUA in the example of Fig. 1) to tune the CABAC contexts when starting to process a new CTU. It was found that tuning the state parameters of the shorter window probability estimator is typically enough and only minor additional impact is achieved by tuning both probability estimators.

Figure 1: Example of processing order of the CUs in the left and above CTUs

## Proposed changes to ECM

It is proposed to buffer bins relating to syntax elements of bottom CUs of CTUs. The number of contexts whose bins may be stored for a single CTU is limited to a given number (128 was selected for these tests). Furthermore, the number of bin values that can be stored for each of those contexts is limited to 4 in this experiment. Thus, a maximum of 128 * 4 bin values equalling to 512 bin values may be stored for each CTU in one CTU row.

When starting to encode or decode a new CTU, the stored bins from the above CTU are retrieved and fed into the context update mechanism of the shorter window probability estimator (with “window 0”). The bins are fed to context update process in the reverse order with respect to their coding order to simulate encoding or decoding that approaches the top-left corner of the new CTU from the direction of bottom-right corner of the CTU above.

# Memory analysis

Memory required for enabling the proposed operation consists of two components.

Firstly, we need to buffer a set of decoded bins for each CTU on one CTU line. An element of such buffer should hold a10-bit context ID (for the current ECM where the number of contexts is below 1024), a 4-bit bin storage containing the bin values for that context and a 2-bit counter indicating how many bins are stored for that context (holding a value between 1 and 4, inclusive – there is no need to store zero value as that can be indicated by other means, like using “1023” as the context number).

The number of contexts for which bins are stored is limited to 128 for each CTU in Test 5.3. Following table summarizes the memory requirement per CTU for the bin buffers.

| Test | Bits / element | Number of elements | Total bits / CTU | Total bytes / CTU |
| --- | --- | --- | --- | --- |
| EE2-5.3 | 10+4+2 = 16 | 128 | 2048 | 256 |

Secondly, to make processing faster, there may be extra memory allocated for the active CTU, for example, to keep track on which contexts have been encountered and included in the bin storage buffer. Such table could have an entry for each context (853 in ECM-11.0), each entry containing a 7-bit index to the bin storage array (as maximum of 128 contexts can be stored for one CTU in these experiments). Thus, the total size of such table is 853*128 bits or 747 bytes in ECM-11.0. It should be noted that this kind of a table is not strictly required if memory is considered a major cost element in the design.

The total memory requirement then depends on the number of CTUs in a CTU row. Considering the CTC settings for different classes of content and the memory elements outlined above, the total amount of memory needed for the Tests 5.3a and 5.3b are as follows:

| Class | CTUs / | Bytes / | Bytes / | Bytes for | Bytes |
| --- | --- | --- | --- | --- | --- |
|  | row | CTU | CTU row | active CTU | Total |
| A1 | 15 | 256 | 3840 | 747 | 4587 |
| A2 | 15 | 256 | 3840 | 747 | 4587 |
| B | 15 | 256 | 3840 | 747 | 4587 |
| C | 7 | 256 | 1792 | 747 | 2539 |
| D | 4 | 256 | 1024 | 747 | 1771 |
| E | 10 | 256 | 2560 | 747 | 3307 |
| F | 15 | 256 | 3840 | 747 | 4587 |
| TGM | 15 | 256 | 3840 | 747 | 4587 |

# Coding efficiency analysis

## Test 5.3a on spatial CABAC tuning

Following tables summarize the coding efficiency results for Test 5.3a. Full results are available in the accompanied Excel sheets.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.04 % | -0.05 % | -0.07 % | 98.7% | 99.9% | 100.0% | 100.0% |
| Class A2 | -0.02 % | 0.08 % | 0.04 % | 99.3% | 99.5% | 100.0% | 100.0% |
| Class B | -0.04 % | -0.03 % | -0.08 % | 98.9% | 99.9% | 99.2% | 99.8% |
| Class C | -0.01 % | -0.05 % | 0.05 % | 99.5% | 102.1% | 100.0% | 100.0% |
| Class E | -0.03 % | -0.16 % | -0.12 % | 99.5% | 100.0% | 100.0% | 100.0% |
| Overall | -0.03 % | -0.04 % | -0.04 % | 99.1% | 100.3% | 99.8% | 99.9% |
| Class D | -0.02 % | 0.05 % | 0.12 % | 98.2% | 105.1% | 99.9% | 100.0% |
| Class F | -0.03 % | -0.14 % | 0.23 % | 98.9% | 100.2% | 100.0% | 100.0% |
| ClassTGM | -0.07 % | -0.05 % | -0.05 % | 98.3% | 99.3% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.02 % | -0.25 % | -0.26 % | 100.0% | 99.0% | 100.0% | 99.9% |
| Class A2 | -0.04 % | -0.06 % | 0.02 % | 99.7% | 99.0% | 100.0% | 99.8% |
| Class B | -0.06 % | -0.04 % | -0.17 % | 100.0% | 100.1% | 99.7% | 100.0% |
| Class C | -0.04 % | -0.22 % | -0.13 % | 99.9% | 100.1% | 99.6% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.04 % | -0.14 % | -0.14 % | 100.0% | 99.6% | 99.8% | 99.9% |
| Class D | 0.03 % | 0.02 % | -0.13 % | 100.4% | 100.1% | 99.8% | 100.0% |
| Class F | -0.09 % | -0.16 % | 0.00 % | 100.0% | 101.0% | 99.9% | 100.0% |
| ClassTGM | -0.06 % | -0.12 % | -0.03 % | 100.2% | 100.0% | 100.7% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.12 % | 0.48 % | -0.05 % | 99.8% | 100.2% | 100.0% | 100.0% |
| Class C | 0.02 % | -0.02 % | -0.36 % | 99.2% | 97.6% | 99.7% | 99.8% |
| Class E | -0.14 % | 0.04 % | -0.26 % | 98.8% | 99.1% | 97.9% | 99.3% |
| Overall | -0.08 % | 0.20 % | -0.21 % | 99.3% | 99.0% | 99.4% | 99.8% |
| Class D | 0.02 % | -0.07 % | -0.73 % | 99.8% | 94.8% | 99.3% | 100.0% |
| Class F | -0.33 % | 0.72 % | 0.40 % | 99.8% | 97.2% | 99.9% | 99.9% |
| ClassTGM | -0.14 % | -0.14 % | -0.01 % | 99.4% | 100.3% | 100.0% | 99.1% |

## Test 5.3b on spatial CABAC tuning with retrained context initialization (of EE2-5.1)

Following tables summarize the coding efficiency results for Test 5.3b. Full results are available in the accompanied Excel sheets.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.06 % | -0.05 % | -0.14 % | 99.9% | 100.1% | 100.0% | 99.9% |
| Class A2 | -0.02 % | -0.02 % | -0.03 % | 100.1% | 99.9% | 100.0% | 100.0% |
| Class B | -0.06 % | 0.04 % | -0.07 % | 99.8% | 99.8% | 99.2% | 100.0% |
| Class C | -0.10 % | 0.10 % | 0.06 % | 99.6% | 100.4% | 100.4% | 100.0% |
| Class E | -0.11 % | -0.02 % | 0.08 % | 100.2% | 100.1% | 100.0% | 100.0% |
| Overall | -0.07 % | 0.02 % | -0.02 % | 99.9% | 100.0% | 99.9% | 100.0% |
| Class D | -0.12 % | 0.08 % | 0.16 % | 99.5% | 102.6% | 99.9% | 100.0% |
| Class F | -0.08 % | -0.04 % | -0.01 % | 100.2% | 100.6% | 100.0% | 100.0% |
| ClassTGM | -0.23 % | -0.16 % | -0.31 % | 99.9% | 101.0% | 100.0% | 100.1% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.19 % | 0.28 % | 0.15 % | 100.1% | 99.9% | 100.1% | 100.2% |
| Class A2 | -0.10 % | 0.10 % | -0.04 % | 100.0% | 99.7% | 100.1% | 99.7% |
| Class B | -0.19 % | 0.29 % | -0.09 % | 99.4% | 100.4% | 100.2% | 100.0% |
| Class C | -0.23 % | 0.14 % | 0.15 % | 98.8% | 100.3% | 99.8% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.18 % | 0.21 % | 0.03 % | 99.5% | 100.1% | 100.0% | 100.0% |
| Class D | -0.30 % | 0.38 % | 0.01 % | 100.4% | 101.6% | 99.8% | 100.0% |
| Class F | -0.28 % | -0.29 % | -0.01 % | 99.8% | 100.4% | 99.9% | 100.0% |
| ClassTGM | -0.37 % | -0.38 % | -0.32 % | 99.7% | 98.6% | 100.5% | 100.2% |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.20 % | 0.89 % | 0.66 % | 100.2% | 98.9% | 100.0% | 100.0% |
| Class C | -0.05 % | 0.12 % | -0.29 % | 100.5% | 99.5% | 100.1% | 100.5% |
| Class E | -0.15 % | 0.12 % | 0.57 % | 99.8% | 99.3% | 100.1% | 100.0% |
| Overall | -0.14 % | 0.44 % | 0.32 % | 100.2% | 99.2% | 100.1% | 100.1% |
| Class D | -0.09 % | 0.59 % | -0.99 % | 99.3% | 98.9% | 100.2% | 100.1% |
| Class F | -0.08 % | 0.73 % | 0.56 % | 99.6% | 99.4% | 98.8% | 100.8% |
| ClassTGM | -0.32 % | -0.25 % | -0.20 % | 101.4% | 97.3% | 99.3% | 98.8% |

# Patent rights declaration(s)

Nokia may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).