| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0119-v1 |
| --- | --- |

| Title: | Cross-check of EE2-Test 5.1 and 5.3 (JVET-AG0117 Spatial CABAC tuning in combination with retrained context initialization for inter slices) |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K. Andersson | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson AB |  |  |

_____________________________

# Abstract

The cross-check of the combination test of Test 5.1 (retrained context initialization) and 5.3 (spatial CABAC tuning) has been performed successfully. Encoder and decoder match and the PSNR and bitrates match with the proponents. The software was checked and it is judged to be aligned with what is proposed.

# Introduction

In JVET-AF0109 it is asserted that the CABAC may get overfitted to the previous CTU or a number of previous CTUs. The approach in JVET-AF0109 is to make a tuning of the probabilities based on the above CTU in backwards order compared to encoding order to end up just above the first CU the current CTU. To limit memory overhead there is a maximum number of bins that are stored per CTU and there is also a limit on the number of bins per context element.

# Results

The results have been obtained by compiling the software on linux with the following settings:

#define JVET_AF0151_CABAC_RETRAIN_NEW_CONTEXTS            1 // Test 5.1: use retrained contexts for the anchor

#define JVET_AF0202_WINDOWS_OFFSETS_SLICETYPE             0 // Test 5.2: window offsets per slice type

#define JVET_AF0109_CABAC_SPATIAL_TUNING                  1 // Test 5.3: spatial CABAC tuning

The encoding times are not accurate. Decoding time has been derived by averaging of 10 decoding’s on same machine without writing reconstruction and MD5.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,06% | -0,05% | -0,14% | 93,5% | 100,1% | 100,8% | 99,9% |
| Class A2 | -0,02% | -0,02% | -0,03% | 96,2% | 100,3% | 100,0% | 100,1% |
| Class B | -0,06% | 0,04% | -0,07% | 99,7% | 100,0% | 100,0% | 99,9% |
| Class C | -0,10% | 0,10% | 0,06% | 100,4% | 100,0% | 99,6% | 100,0% |
| Class E | -0,11% | -0,02% | 0,08% | 99,8% | 100,7% | 100,0% | 100,0% |
| Overall | -0,07% | 0,02% | -0,02% | 98,2% | 100,2% | 100,1% | 100,0% |
| Class D | -0,12% | 0,08% | 0,16% | 99,1% | 100,2% | 99,9% | 100,0% |
| Class F | -0,08% | -0,04% | -0,01% | 94,5% | 100,5% | 99,4% | 100,0% |

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,19% | 0,28% | 0,15% | 95,2% | 100,5% | 100,0% | 100,0% |
| Class A2 | -0,10% | 0,10% | -0,04% | 95,6% | 100,8% | 99,6% | 100,1% |
| Class B | -0,19% | 0,29% | -0,09% | 97,2% | 100,6% | 99,4% | 100,1% |
| Class C | -0,23% | 0,14% | 0,15% | 93,5% | 100,5% | 99,6% | 100,0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,18% | 0,21% | 0,03% | 95,5% | 100,6% | 99,6% | 100,1% |
| Class D | -0,30% | 0,38% | 0,01% | 92,0% | 101,4% | 99,8% | 100,0% |
| Class F | -0,28% | -0,29% | -0,01% | 94,3% | 99,8% | 99,8% | 100,3% |

|  | Low delay B Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0,20% | 0,89% | 0,66% | 92,9% | 100,3% | 100,3% | 99,8% |
| Class C | -0,05% | 0,12% | -0,29% | 98,1% | 100,4% | 101,4% | 100,2% |
| Class E | -0,15% | 0,12% | 0,57% | 89,3% | 101,1% | 100,2% | 101,0% |
| Overall | -0,14% | 0,44% | 0,32% | 93,6% | 100,5% | 100,7% | 100,2% |
| Class D | -0,09% | 0,59% | -0,99% | 98,8% | 99,9% | 100,2% | 100,1% |
| Class F | -0,08% | 0,73% | 0,56% | 96,1% | 100,9% | 99,0% | 102,0% |

# References

[1] J. Lainema, A. Aminlou, P. Astola, D. B. Sansli, “EE2-5.3: Spatial CABAC tuning”, JVET-AG0117, Online, January 2024.

[2] V. Seregin, et.al, “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AF2024, Hannover, Germany, October 2023.

[3] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Hannover, Germany, October 2023.

[4] ECM-11.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

# Patent rights declaration(s)

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).