| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0120-v1 |
| --- | --- |

| Title: | Non-EE2: On line buffer restriction |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Zhipin Deng1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9 18 Zhongguancun Nansijie Haidian District, 100190, Beijing, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | {zhipin.deng, zhangkai.video, lizhang.idm}@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

In ECM-11.0, many coding tools can access more than one line buffer, but a few coding tools still follow the limitation of one line buffer restriction as in VVC. This contribution proposes to remove the line buffer restriction for all coding tools in ECM. The impact on coding efficiency and runtimes of the proposed test over ECM-11.0 are reportedly for {Y, U, V, EncT, DecT}:

AI { -0.05%, 0.06%, 0.02%, 100.4%	, 100.3%}; RA { -0.01%, -0.02%, -0.05%, 100.4%, 100.0%}

# Introduction

In VVC, the line buffer for intra prediction is limited to one line of samples, thus intra prediction including CCLM are limited not to access neighboring reconstruction samples beyond one row above the current CTU. In ECM, several new coding methods have been adopted, and some of them break the line buffer restriction in VVC, as listed in Table 1.

Table 1. The line buffer requirement of coding tools in ECM-11.0

| Coding methods in ECM-11.0 | Require more than one line buffer? |
| --- | --- |
| CCCM/NS-CCCM/GL-CCCM/MDF-CCCM | Yes |
| GLM with gradients and downsampled luma | Yes |
| DIMD template | Yes |
| TIMD template | Yes |
| Template usage such as interTM merge | Yes |
| Intra MPM: non-adjacent spatial candidates | Yes |
| Inter/IBC/affine motion: non-adjacent spatial candidates | Yes |
| Intra/Inter CCP merge: non-adjacent spatial candidates | Yes |
| intraTMP/IBC search region | Yes |
| Extended MRL | No |
| Template based MRL | No |
| Intra luma fusion | No |

# Proposed method

In this contribution, it is proposed to remove the line buffer restriction for all coding tools in ECM.

# Experimental results

The proposed method is implemented on top of ECM-11.0 [1] and tested following the common test conditions [2]. The simulation results of the proposed method compared to ECM-11.0 are summarized in Table 2.

Table 2: Coding performance of the proposed test

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | 0.00% | 0.04% | 100.5% | 100.3% | 99.9% | 99.9% |
| Class A2 | -0.01% | 0.11% | 0.02% | 100.1% | 99.6% | 100.0% | 100.0% |
| Class B | -0.05% | 0.12% | -0.09% | 100.5% | 100.0% | 100.0% | 100.0% |
| Class C | -0.06% | -0.02% | 0.13% | 100.3% | 100.9% | 99.6% | 100.0% |
| Class E | -0.07% | 0.10% | 0.00% | 100.4% | 100.6% | 100.0% | 100.0% |
| Overall | -0.05% | 0.06% | 0.02% | 100.4% | 100.3% | 99.9% | 100.0% |
| Class D | -0.03% | -0.04% | -0.07% | 100.1% | 100.0% | 100.0% | 100.0% |
| Class F | -0.04% | -0.15% | 0.17% | 99.9% | 100.1% | 100.0% | 100.0% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.03% | -0.05% | -0.04% | 100.0% | 99.7% | 100.4% | 100.3% |
| Class A2 | 0.00% | -0.02% | 0.01% | 100.3% | 100.5% | 99.8% | 100.0% |
| Class B | 0.00% | 0.05% | -0.03% | 100.4% | 100.2% | 99.4% | 100.1% |
| Class C | -0.01% | -0.08% | -0.13% | 100.7% | 99.8% | 100.0% | 100.1% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.01% | -0.02% | -0.05% | 100.4% | 100.0% | 99.8% | 100.1% |
| Class D | 0.00% | 0.11% | 0.16% | 100.7% | 99.4% | 99.9% | 100.0% |
| Class F | -0.05% | -0.13% | -0.21% | 100.6% | 100.9% | 99.6% | 99.9% |

# Conclusions

The proposed method can achieve decent BD-rate gains with negligible runtime change. It is recommended to include the proposed methods in the next EE2 for a further investigation.

# References

- ECM-11.0, https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tree/ECM-11.0.

- M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Oct. 2023.

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).