| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0125-v1 |
| --- | --- |

| Title: | AHG12: Parallel friendly use of boundary distortion for DMVR |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | K.Andersson, R. Yu | Tel: Email: | +46730371390 kenneth.r.andersson@ericsson.com |
| Source: | Ericsson |  |  |

_____________________________

# Abstract

In JVET-AF0057 a normative subjective enhancement of DMVR was proposed, wherein a subblock boundary distortion metric was introduced in the subblock bilateral matching search. Concerns were raised regarding parallel computation of subblock motions due to introduction of dependency to neighbouring subblock boundary samples.

In the present contribution, an improved method over the JVET-AF0057 method is proposed. The subblock boundary distortion calculations are moved to a refinement step after the subblock bilateral matching search. In the refinement step, the initial motion vector pair and the best bilateral matching candidate motion vector pair are compared with respect to boundary distortion. The motion vector pair that has least sum of boundary distortion, motion vector cost and bilateral matching distortion is selected as the refined motion vector pair.

Objective performance under ECM-11.0 CTC is reported to be:

Random access (Y/U/V): -0.01%/0.02%/-0.01% Dec time: 100.3 %

It is asserted that the proposed method is more parallel friendly compared to the JVET-AF0057 method while the subjective improvement is maintained. It is proposed to include the method to ECM.

# Introduction

DMVR can introduce visual artifacts and to combat that encoder methods have been included to both the VVC reference software and ECM software. Since ECM is exploratory work and not a final standard it is believed to be more appropriate to deal with the issue on both encoder and decoder side.

# Proposal

To have a design which inherently aims to make the motion vector determination of DMVR more robust, it is proposed to introduce a refinement step as a final step after bilateral matching search. The refinement step selects a motion vector pair from the initial motion vector pair and the best bilateral matching candidate based on a boundary distortion metric. The approach is used for blocks with size 16x32, 32x16, 32xN and Nx32 where N is equal to 32 or greater, as the DMVR visual artifacts are more likely to appear for those blocks.

The boundary distortion is computed by comparison of gradients in neighbouring subblock samples left of the current subblock and above the current subblock to the corresponding gradients on the left most and top most subblock samples of the current subblock. The motion vector pair which gives least sum of boundary distortion (bd) and block matching distortion (bm) is selected for each subblock.

The boundary distortion at the top subblock boundary of the current subblock is defined as:

h = h1+h2+h3+h4,

h1 = i=1(M-2)/2abs((Bi*2,N-1 – Bi*2-1,N-1) – (Ai*2,0 – Ai*2-1,0)),

h2 = i=1M-22-1abs((Bi*2+1,N-1 – Bi*2-1,N-1) – (Ai*2+1,0 – Ai*2-1,0)),

h3 =  i=1M/4abs((Bi*4-1,N-1 – Bi*4-4,N-1) – (Ai*4-1,0 – Ai*4-4,0)),

h4 =  i=1M/8abs((Bi*8-1,N-1 – Bi*8-8,N-1) – (Ai*8-1,0 – Ai*8-8,0)),

where B is the subblock above the current subblock and the dimension of the subblocks are MxN (16x16), and A is a set of candidate motion compensated prediction sample values that were generated using the candidate motion vector pair.

The boundary distortion at the left subblock boundary of the current subblock is defined as:

v = v1+v2+v3+v4,

v1 = i=1(N-2)/2abs((DM-1,i*2 – DM-1,i*2-1) – (A0,i*2 – A0,i*2-1)),

v2 = i=1N-22-1abs((DM-1,i*2+1 – DM-1,i*2-1) – (A0,i*2+1 – A0,i*2-1)),

v3 = i=1N/4abs((DM-1,i*4-1 – DM-1,i*4-4) – (A0,i*4-1 – A0,i*4-4)),

v4 =  i=1N/8abs((DM-1,i*8-1 – DM-1,i*8-8) – (A0,i*8-1 – A0,i*8-8)),

where D is the subblock to the left of the current subblock.

When both subblock boundaries are internal (not at block boundary):

bd = (v + h) << 4.

When only top subblock boundary is internal (not at block boundary):

bd = h << 2.

When only left subblock boundary is internal (not at block boundary):

bd = v << 2.

When no subblock boundary is internal (top-left subblock):

bd = 0.

The distortion is defined as the sum of the block matching distortion, boundary distortion and motion vector cost:

d = bm + bd + mvCost.

Where mvCost is related to the sum of absolute horizontal respectively absolute vertical difference between the initial motion vector and a candidate motion vector and is something already used in ECM.

When spatial activity is low it can be difficult to get a reliable bilateral matching candidate motion. Subblocks closest to the left and/or the top block boundary do not have neighbouring subblock samples available on two sides and can thus not be aligned both vertically and horizontally using the boundary distortion. Therefore, spatial activity is computed for subblocks at the left and/or the top block boundary. If the spatial activity is below a threshold for a bilateral matching candidate, the block matching distortion is increased by a factor of 8 for that candidate. Furthermore, for subblocks at the block boundary, the bilateral matching candidates also have their block matching distortion increased by a factor of 2 when a candidate motion vector pair has one motion vector with one motion vector component greater than 7 (in integer pixel).

The candidate motion vector pair with least total distortion is selected as the refined motion vector pair for the subblock. This is asserted to make the motion selection at the block boundary more robust.

# Results

Decoder timings are accurate, average over several runs without MD5 and without writing reconstruction to disk. Encoder timings are not accurate but expected to be similar as decoder timings since DMVR is used on the decoder side.

|  | Random Access Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0,02% | 0,05% | 0,09% | 101,8% | 100,4% | 100,1% | 100,4% |
| Class A2 | -0,02% | 0,01% | -0,08% | 99,0% | 100,1% | 99,6% | 100,0% |
| Class B | 0,00% | 0,05% | -0,03% | 93,7% | 100,3% | 99,4% | 100,0% |
| Class C | -0,04% | -0,02% | -0,03% | 97,4% | 100,3% | 100,0% | 99,9% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,01% | 0,02% | -0,01% | 97,3% | 100,3% | 99,7% | 100,1% |
| Class D | -0,05% | 0,04% | 0,02% | 94,3% | 100,2% | 99,7% | 100,0% |

# Conclusion

It is proposed to include the simplified normative method to ECM to make sure that DMVR is robust.

# References

[1] K. Andersson, R. Yu, “EE2-3.5: DMVR with robust MV derivation”, JVET-AF0057, Hannover, Germany, October 2023.

[2] M. Karczewice, Y. Yan, “Common Test Conditions and evaluation procedures for enhanced compression tool testing”, JVET-AF2017, Hannover, Germany, October 2023.

[3] ECM-11.0, available at https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

# Patent rights declaration

Ericsson may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).