| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0130-v2 |
| --- | --- |

| Title: | EE1-4.1: Unified CNN-based super resolution for resampling-based video coding |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Chaoyi Lin1, Yue Li2, Junru Li2, Kai Zhang2, Li Zhang2 1Bafangcheng 9th building F14 Yuhang District, 311100, Hangzhou City, Zhejiang Province, China 28910 University Center Lane San Diego, CA 92122, USA | Tel: Email: | linchaoyi.cy@bytedance.com yue.li@bytedance.com lijunru@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Bytedance Inc. |  |  |

_____________________________

# Abstract

This contribution reports the EE1-4.1 test results, which are based on JVET-AF0143. In JVET-AF0143, a unified neural network-based (NN-based) super resolution architecture is proposed for resampling-based video coding. The experimental results are reported as:

Test 4.1.1.a: the LOP SR reportedly shows on average {-1.49%, -0.60%, 0.14%} and {-1.22%, -0.22%, 0.63%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Test 4.1.1.b: the HOP SR reportedly shows on average {-2.20%, -0.56%, 0.26%} and {-1.17%, -2.68%, -1.54%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Test 4.1.2.a: the combined LOP model (LOP SR + LOP LF + NN intra) reportedly shows on average {} and {} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Test 4.1.2.b: the combined HOP model (HOP SR + HOP LF + NN intra) reportedly shows on average {} and {} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

# Introduction

JVET-AF0143 [1] presents a unified architecture for High Operation Point (HOP) and Low Operation Point (LOP) super resolution (SR), respectively. The complexity of proposed unified LOP SR and HOP SR are designed to be 20.3 and 469 kMAC/pixel, respectively, compared to 469 kMAC/pixel of the current SR model in NNVC 7.1.

In this contribution, the following test results are reported: unified LOP SR (test 4.1.1.a), unified HOP SR (test 4.1.1.b), combination test of LOP SR, LOP NNLF, and NN intra (test 4.1.2.a), and combination test of HOP SR, HOP NNLF, and NN intra (test 4.1.2.b).

# Proposed Unified SR Architecture

## LOP SR Architecture

The proposed unified LOP SR architecture and hyperparameters are illustrated in Figure 1. The input interface of the filter is aligned to the unified in-loop filter. The chroma components are upsampled to the same size of luma components before feeding into the network. Then a 1×1 convolution is used to fuse the concatenated features. Then, split of luma and chroma with different complexity is designed in a same way to the LOP in-loop filter. The chroma branch includes a smaller number of channels and basic backbone blocks compared to luma branch.

The encoder determines whether to apply the resampling-based method adaptively as that in NNVC-7.1. For LOP SR , the encoder enables resampling-based method only if QP≥32.

Figure 1 The unified LOP SR architecture.

The network information in the inference stage is provided in Table 1.

Table 1 Network information for the proposed LOP SR model in inference stage.

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 0.08 M |
|  | Total Parameter Number | 0.08 M |
|  | Parameter Precision (Bits) | Int 16 |
|  | Memory Parameter (MB) | 0.16 MB |
|  | Multiply Accumulate (MAC) | 20.3 kMAC/pixel |

The network information in the training stage is provided in Table 2.

Table 2 Network information for the proposed LOP SR model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 45 |
|  | Batch size: | 16 |
|  | Training time: | 120h/model |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn off all the NN tools |
|  | Loss function: | L1, L2 |

## HOP SR Architecture

The proposed unified HOP SR architecture and hyperparameters are illustrated in Figure 2. The input interface of the filter is aligned to the unified in-loop filter. The chroma components are upsampled to the same size of luma components before feeding into the network. Then a 1×1 convolution is used to fuse the concatenated features. The architecture of backbone blocks is the same as HOP In-loop filter. The RPR upsampled reconstruction will be added with output of network to generate the final upsampled reconstruction.

Figure 2 The unified HOP SR architecture.

The network information in the inference stage is provided in Table 3.

Table 3 Network information for the proposed HOP SR model in inference stage

| Network Information in Inference Stage |  |  |
| --- | --- | --- |
| Mandatory | HW environment: | Intel(R) Xeon(R) Platinum 8260 CPU @ 2.40GHz |
|  | Framework: | SADL |
|  | Number of GPUs per Task | 0 |
|  | Number of Parameters (Each Model) | 1.9 M |
|  | Total Parameter Number | 1.9 M |
|  | Parameter Precision (Bits) | Int 16 |
|  | Memory Parameter (MB) | 3.8 MB |
|  | Multiply Accumulate (MAC) | 469 kMAC/pixel |

The network information in the training stage is provided in Table 4.

Table 4 Network information for the proposed HOP SR model in training stage

| Network Information in Training Stage |  |  |
| --- | --- | --- |
| Mandatory | GPU Type | GPU: Tesla-V100-SXM2-32GB |
|  | Framework: | PyTorch v1.11 |
|  | Number of GPUs per Task | 1 |
|  | Epoch: | 45 |
|  | Batch size: | 32 |
|  | Training time: | 240h/model |
|  | Training data information: | DIV2K, BVI_DVC, TVD |
|  | Training configurations for generating compressed training data (if different to VTM CTC): | Turn off all the NN tools |
|  | Loss function: | L1, L2 |

# Experimental Results

The proposed CNN-based up-sampling filter is tested on top of NNVC 7.1, according to the common test conditions [2]. The tested QP values are {22, 27, 32, 37, 42}.

## Test 4.1.1.a

The LOP SR reportedly shows on average {-1.49%, -0.60%, 0.14%} and {-1.22%, -0.22%, 0.63%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 5 test results of EE1-4.1.1.a for integer model

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.17% | -6.60% | -4.30% | 75% | 43% |
| Class A2 | -2.26% | 3.60% | 4.99% | 90% | 65% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Avg. A1&A2 | -3.72% | -1.50% | 0.34% | 82% | 52% |
| Average | -1.49% | -0.60% | 0.14% | 136% | 80% |

Table 6 test results of EE1-4.1.1.a for integer model

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -3.20% | -1.29% | -0.11% | 112% | 49% |
| Class A2 | -4.11% | -0.03% | 3.90% | 84% | 47% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Avg. A1&A2 | -3.65% | -0.66% | 1.89% | 97% | 48% |
| Average | -1.22% | -0.22% | 0.63% | 100% | 81% |

## Test 4.1.1.b

The HOP SR reportedly shows on average {-2.20%, -0.56%, 0.26%} and {-1.17%, -2.68%, -1.54%} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

Table 7 test results of EE1-4.1.1.b for integer model

|  | Random access Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -7.49% | -6.05% | -3.19% | 231% | 43% |
| Class A2 | -3.52% | 3.25% | 4.46% | 172% | 64% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E |  |  |  |  |  |
| Avg. A1&A2 | -5.51% | -1.40% | 0.64% | 199% | 52% |
| Average | -2.20% | -0.56% | 0.26% | 136% | 80% |

Table 8 test results of EE1-4.1.1.b for integer model

|  | All Intra Main10 |  |  |  |  |
| --- | --- | --- | --- | --- | --- |
|  | BD-rate Over NNVC-7.1 VTM anchor |  |  |  |  |
|  | Y-PSNR | U-PSNR | V-PSNR | EncT | DecT CPU |
| Class A1 | -5.30% | -5.12% | -4.70% | 317% | 48% |
| Class A2 | -1.74% | -10.95% | -4.52% | 241% | 48% |
| Class B | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class C | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Class E | 0.00% | 0.00% | 0.00% | 100% | 100% |
| Avg. A1&A2 | -3.52% | -8.04% | -4.61% | 276% | 48% |
| Average | -1.17% | -2.68% | -1.54% | 142% | 81% |

## Test 4.1.2.a

The combined LOP model (LOP SR + LOP LF + NN intra) reportedly shows on average {} and {} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

## Test 4.1.2.b

The combined HOP model (HOP SR + HOP LF + NN intra) reportedly shows on average {} and {} BD-rate changes for {Y, U, V} under RA and AI configurations, respectively.

# Conclusion

This contribution reports the test results of EE1-4.1. The unified LOP SR show a good trade-off between complexity and coding performance. Moreover, the unified HOP SR show a promising coding performance. It is recommended that the unified LOP and HOP SR to be adopted into NNVC software.

# Reference

[1] C. Lin, Y. Li, J. Li, K. Zhang, L. Zhang, “AHG11: Unified CNN super resolution for resampling-based video coding”, JVET-AF0143

[2] E. Alshina, R.-L. Liao, S. Liu, A. Segall, “Common test conditions and evaluation procedures for neural network-based video coding technology”, JVET-AF2016

# Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).