| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0131-v2 |
| --- | --- |

| Title: | EE2-1.11: IntraTMP search range extension with adaptive sampling |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | D. Ruiz Coll, Karam Naser Philippe Bordes Fabrice Le Léannec Franck Galpin | Tel: Email: | duizcoll@ofinno.com, karam.naser@interdigital.com |
| Source: | Ofinno, InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the experiment results of EE2-1.11 tests. It aims at harmonizing the IBC and IntraTMP search areas using an adaptive sampling rate during the sparse stage. In the existing ECM design, the IBC search area is delineated by a fixed number of CTU rows. Conversely, the IntraTMP search area is dynamically determined based on the dimensions of the current block, typically smaller than the IBC search area. IntraTMP searches for the most similar template that matches the current block template. To mitigate the computational complexity associated with template matching, a consistent sampling factor of 3 is uniformly applied in both horizontal and vertical search directions within IntraTMP. However, extending the IntraTMP search area to align with the IBC dimension leads to a notable increase in computational load.

This contribution proposes to use an adaptive sampling factor rate determined as a combination of the searching distance from the current CU and the dimensions of each respective search region when extending the IntraTMP search region to align with the IBC search region. The proposed methods have been implemented on top of ECM-11.0, and the subsequent test results are as follows {for Y, U, V, EncT, DecT, EncVmPeak, DecVmPeak}:

Test-1.11a: Sampling rate based on the search distance.

AI: ClassF {-0.32%, -0.38%, -0.18%, 101.2%, 104.0%, 102.6%, 100.0%};

AI: ClassTGM {-0.49%, -0.50%, -0.52%, 101.6%, 105.6%, 98.9%, 100.0%};

AI: Overall {-0.02%, 0.00%, -0.04%, 102.3%, 104.4%, 100.2%, 100.0%}.

RA: ClassF{-0.20%, -0.27%, 0.00%, 97.7%, 99.7%, 100.4%, 100.9%};

RA: ClassTGM {-0.45%, -0.64%, -0.46%, 96.4%, 98.6%, 101.3%, 100.4% };

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%, 10x.x%, 10x.x%}.

Test-1.11b: Sampling rate based on the IntraTMP regions dimension.

AI: ClassF {-0.28%, -0.40%, -0.31%, 101.8%, 105.3%, 100.0%, 100.0%};

AI: ClassTGM {-0.54%, -0.52%, -0.57%, 101.9%, 106.6%, 99.9%, 99.8%};

AI: Overall {0.00%, -0.02%, -0.02%, 101.8%, 102.6%, 100.1%, 100.0%};

RA: ClassF{-0.13%, -0.24%, -0.29%, 97.9%, 98.6%, 100.5%, 101.0%};

RA: ClassTGM {-0.41%, -0.53%, -0.45%, 96.5%, 97.8%, 100.7%, 100.2%};

RA: Overall {-0.xx%, -0.xx%, -0.xx%, 10x.x%, 10x.x%, 10x.x%, 10x.x%}.

Test-1.11c: Combination of Test-1.11a and Test-1.11b.

AI: ClassF {-0.22%, -0.25%, 0.06%, 100.5%, 100.8%, 102.6%, 99.9%};

AI: ClassTGM {-0.35%, -0.34%, -0.40%, 100.7%, 100.8%, 98.9%, 99.8%};

AI: Overall {0.01%, 0.05%, 0.02%, 101.5%, 101.9%, 99.6%, 100.0%};

RA: ClassF{-0.15%,  -0.04%, 0.02%, 98.0%, 99.1%, 100.2%, 100.0%};

RA: ClassTGM {-0.32%, -0.49%, -0.46%, 96.2%, 98.8%, 99.1%, 100.1%};

RA: Overall* {0.02%, 0.00%, -0.06%, 99.6%, 99.1%, 100.2%, 100.0%}.

# Introduction

The IBC search region comprises the reconstructed samples of a fixed number of CTUs based on the CTU dimension. This region encompasses all left neighbor CTUs within the same row as the current CTU and extends to two CTU rows above the current CTU, and the topmost CTU row initiates two CTUs to the left of the current CTU [1]. In the specific case of a CTU size set at 256, the search region is restricted to only one CTU row above the current CTU, starting one CTU to the left of the current CTU.

Figure 1. IBC Search area compressing the reconstructed samples of the CTUs inside the area marked by the red line (CTU size of 128)

In the context of  IntraTMP [2], the search area is IntraTMP search area is dynamically configured in proportion to the width (W) and height (H) dimensions of the current block. This adjustment is achieved through a multifactor parameter denoted as 'a,' which governs the trade-off between gain and complexity. For block sizes of 4 and 8 samples, the intraTMP search region dimension (SearchRangeWidth, SearchRegionHeight) is constrained to 64 [3]in the following manner:

SearchRangeWidth= max⁡(a * W,64)

SearchRangeHeight =max⁡( a * H, 64)

The IntraTMP search area is bounded in the horizontal and vertical dimensions by the SearchRangeWidth and SearchRegionHeight parameters, and the area is partitioned into six rectangular regions, as depicted in Fig. 2:

Figure 2. IntraTMP search area divided into six search regions in ECM-11.0.

The IntraTMP candidate BV search is performed in a two-step process. First, the spare searching is conducted across all regions utilizing a SamplingFactor of 3, and it selects the 30 BV candidates exhibiting the lowest SAD template cost. Second, a refinement searching stage is applied to the sparse candidates, further refined in a window of 3x3 pixels, and the 19 candidates with the lowest SAD template cost are selected for a "short IntraTMP list."

# Proposal

This contribution reports the results of the contribution JVET-AF0161 [4] adopted in the 32nd Meeting for further study in the context of the Exploration Experiments 2 [5].

JVET-AF0161 investigated the extension of the IntraTMP search region to align with the dimensions of the IBC search region, allowing the use of the unique reconstructed buffer for both tools. This feature is particularly desirable from the hardware point of view, considering that the reconstructed buffer uses a high-cost on-chip memory.

To cover the whole IBC reference region, the number of IntraTMP regions is increased by one compared to ECM-11, and exclusively, the boundaries of the farthest original regions (R1, R2, and R3) have been modified to cover the whole IBC search region, as it is depicted in Fig. 3:

Figure 3. The IntraTMP search area isadjusted to the  IBC search area. The extended search area (non-dashed area) uses an adaptive sampling rate (SR2).

The ‘regular search area’ maintains the exact dimensions as in ECM-11 constrained by the SearchRangeWidth and SearchRangeHeight parameters, and the same sample rate (SR1) of 3.

The searching process in the further away regions (R1 and R7) is subject to pruning if the normalized TMP cost of the last BV candidate in the sparse list is less than a threshold, which depends on the current block's horizontal and vertical size (W, H). Furthermore, the search regions R1 and R6 are scanned in the inverse direction, from left to right, checking before the most probable candidates, and the scanning pattern is shifted by one pixel per row. Lastly, for block sizes of 64 samples, the multifactor parameter ‘a’ is set to 4, preventing the regular search region from exceeding the IBC search region.

This contribution studies two methods of adapting the sampling rate beyond the regular search area to reduce the complexity of the search extension. These methods propose to use an adaptive sampling rate (SR2)  calculated according to specific parameters, as described below.

Test-11a: Sampling rate based on the search distance.

In the context of the screen content, the probability distributions of the horizontal and vertical components of the BVs within IntraTMP closely approximate a normal distribution N(µx, σx) and N(µy, σy) respectively, which standard deviation is correlated with the current block dimensions.

Fig. 4 illustrates an example of the probability density distribution (PDF) for the horizontal component of the BVs within IntraTMP obtained for block sizes of 16x16 using a constant sampling rate of 3. It can be observed that the regular search region delimited by the SearchRangeWidth parameter is significantly less than the range encompassed by two standard deviations (2σ), which comprises a probability close to 68%. In this example, the likelihood of finding a BV candidate beyond 500 samples is less than 5%. Using a uniform sampling rate of 3 for the entire extended search region may prove inefficient from a complexity standpoint.

Figure 4. Example of the  PDF of the horizontal component of the BVs in intraTMP for 16x16 blocks

Consequently, for searching beyond the regular search region, this method proposes a stepwise adjustment of the sampling rate parameter inversely proportional to the likelihood of encountering a BV candidate within the extended area. The probability distribution is subjected to estimation, and the resulting sampling rate parameters are stored in a table for the block dimensions. In this proposal, the sampling rate undergoes adjustment based on the distance multiple of 64 samples, and it ranges from a minimum value of 3 within the regular search region (a*W, a*H) to a maximum sampling factor of 80, as it is depicted in Fig. 5.

Figure 5. Example of Sampling rate factor for horizontal dimension within intraTMP

Test-1.11b: Sampling rate based on the IntraTMP regions dimension.

A new subsampling factor is determined for each region (Ri) according to the region dimensions as follows:

SamplingFactorWidth(Ri) = Width(Ri)/ F*BlkW

SamplingFactorHeight(Ri) = Height(Ri)/ F*BlkH

where ‘F’ is the ECM constant that controls the gain/complexity trade-off. Therefore, the added number of template comparisons remains small for all the added regions. The new sampling rate factors are only applied at the extended region location, using the same sampling factor (SamplingFactor = 3) inside the regular research region.

Test-1.11c: Combination of Test-1.11a and Test-1.11b

This test combines both methods as follows:

- The multifactor parameter 'a' for block sizes of 8 and 4 pixels is unified to 5.

- The sampling factor rate for both methods is calculated as described in Test-1.11b.

- The most significant sampling factor is selected for the current location:

  - SamplingFactor= max(SamplingFactor1,SamplingFactor2)

# Experimental Results

The proposed methods are tested on top of ECM-11.0 software [6] under JVET ECM common test conditions [7] in AI and RA configurations. The following results were obtained:

## Test 1.11a: Sampling rate based on the search distance

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.32% | -0.38% | -0.18% | 101.2% | 104.0% | 102.6% | 100.0% |
| Class TGM | -0.49% | -0.50% | -0.52% | 101.6% | 105.6% | 98.9% | 100.0% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.20% | -0.27% | 0.00% | 97.7% | 99.7% | 100.4% | 100.9% |
| Class TGM | -0.45% | -0.64% | -0.46% | 96.4% | 98.6% | 101.3% | 100.4% |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.08% | 0.01% | 102.4% | 102.9% | 99.7% | 100.0% |
| Class A2 | -0.02% | -0.03% | 0.03% | 102.5% | 106.2% | 99.7% | 100.1% |
| Class B | -0.02% | -0.07% | -0.17% | 100.6% | 104.7% | 99.3% | 100.1% |
| Class C | -0.02% | -0.11% | -0.01% | 104.9% | 104.3% | 100.1% | 100.0% |
| Class E | -0.02% | 0.18% | 0.00% | 101.6% | 103.8% | 102.9% | 99.9% |
| Overall | -0.02% | 0.00% | -0.04% | 102.3% | 104.4% | 100.2% | 100.0% |
| Class D | 0.01% | 0.09% | 0.03% | 100.7% | 102.5% | 100.1% | 100.0% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.01% | 0.07% | 0.06% | 100.3% | 99.6% | 100.3% | 99.8% |
| Class C | 0.04% | -0.13% | -0.05% | 100.5% | 98.9% | 100.0% | 99.9% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.04% | 0.05% | 0.09% | 100.4% | 100.3% | 99.7% | 100.0% |

## Test 1.11b: Sampling rate based on the IntraTMP regions dimension

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.28% | -0.40% | -0.31% | 101.8% | 105.3% | 100.0% | 100.0% |
| Class TGM | -0.54% | -0.52% | -0.57% | 101.9% | 106.6% | 99.9% | 99.8% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.13% | -0.24% | -0.29% | 97.9% | 98.6% | 100.5% | 101.0% |
| Class TGM | -0.41% | -0.53% | -0.45% | 96.5% | 97.8% | 100.7% | 100.2% |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | -0.03% | 0.04% | 102.4% | 103.6% | 98.6% | 100.3% |
| Class A2 | -0.01% | 0.03% | 0.09% | 102.2% | 104.2% | 100.1% | 100.0% |
| Class B | 0.00% | -0.03% | -0.18% | 99.7% | 102.4% | 99.9% | 100.0% |
| Class C | -0.03% | -0.08% | 0.01% | 104.3% | 101.2% | 100.6% | 100.0% |
| Class E | 0.04% | 0.03% | 0.05% | 101.1% | 102.1% | 101.1% | 99.9% |
| Overall | 0.00% | -0.02% | -0.02% | 101.8% | 102.6% | 100.1% | 100.0% |
| Class D | 0.02% | -0.01% | 0.08% | 100.2% | 100.6% | 100.1% | 100.0% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | 0.04% | 0.09% | -0.20% | 99.1% | 98.9% | 100.0% | 100.0% |
| Class C | 0.02% | -0.05% | -0.09% | 98.3% | 99.1% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |
| Class D | -0.01% | 0.04% | -0.39% | 99.9% | 99.7% | 99.8% | 100.0% |

## Test 1.11c: Combination of Test-1.11a and Test-1.11b

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.22% | -0.25% | 0.06% | 100.5% | 100.8% | 102.6% | 99.9% |
| Class TGM | -0.35% | -0.34% | -0.40% | 100.7% | 100.8% | 98.9% | 99.8% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class F | -0.15% | -0.04% | 0.02% | 98.0% | 99.1% | 100.2% | 100.0% |
| Class TGM | -0.32% | -0.49% | -0.46% | 96.2% | 98.8% | 99.1% | 100.1% |

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.01% | 0.00% | 0.01% | 101.4% | 101.9% | 99.9% | 100.3% |
| Class A2 | 0.01% | 0.05% | 0.04% | 101.4% | 102.1% | 99.7% | 100.0% |
| Class B | 0.01% | 0.01% | -0.02% | 99.6% | 101.3% | 99.1% | 99.9% |
| Class C | 0.00% | -0.03% | -0.09% | 104.3% | 101.9% | 99.6% | 100.0% |
| Class E | 0.05% | 0.26% | 0.24% | 101.1% | 102.6% | 99.9% | 99.9% |
| Overall | 0.01% | 0.05% | 0.02% | 101.5% | 101.9% | 99.6% | 100.0% |
| Class D | 0.02% | 0.04% | -0.04% | 100.5% | 99.6% | 100.3% | 100.0% |
|  | Random Access Intra Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1* | 0.00% | 0.04% | -0.12% | 100.1% | 99.1% | 100.5% | 100.3% |
| Class A2* | 0.05% | 0.04% | -0.03% | 99.4% | 98.3% | 100.4% | 100.1% |
| Class B | 0.03% | 0.01% | -0.08% | 100.4% | 100.3% | 100.0% | 99.7% |
| Class C | 0.01% | -0.08% | -0.02% | 98.5% | 98.1% | 100.0% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.02% | 0.00% | -0.06% | 99.6% | 99.1% | 100.2% | 100.0% |
| Class D | 0.03% | 0.25% | -0.30% | 98.5% | 98.0% | 99.4% | 100.0% |

# * Test-1.11c: Some RA Class A1 and A2 results are copied from the Reference results

# Conclusion

This contribution proposes to use an adaptive sampling factor rate when extending the IntraTMP search region to align with the IBC search region. The method proposed in Test-1.11c brings some coding gains for Screen Content (SC) with a negligible encoding runtime increase. It is recommended to adopt the Test-1.11c method for SC into the upcoming EE2.

- References

- J. Xu and N. Zhang, “EE2-3.2: IBC Reference Area Extension,” JVET-Z0153, by teleconference, April 2022.

- K. Naser, T. Poirier, F. Le Léannec, G. Martin-Cocher, “EE2: Intra Template Matching”, JVET-V0130, April 2021.

- F. Wang, L. Zhang, Y. Yu, H. Yu, D. Wang, “Non-EE2: multi-candidate IntraTMP”, JVET-AC0068, January 2023.

- D. Ruiz Coll, B. Chen, K. Naser, P. Bordes, F. Le Léannec, and F. Galpin, “AHG12: Harmonized IBC/ITMP search area with adaptive sampling,” JVET-AF0161, Hannover, October 2023.

- V. Seregin, et al., “Exploration experiment on enhanced compression beyond VVC capability (EE2)”, JVET-AF2024, October 2023.

- https://vcgit.hhi.fraunhofer.de/ecm/ECM/-/tags/ECM-11.0

- M. Karczewicz and Y. Ye, "Common Test Conditions and evaluation procedures for enhanced compression tool testing," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29, JVET-AF2017, 32nd Meeting, Hannover, Oct. 2023.

# Patent rights declaration(s)

Ofinno, LLC may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).