| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0137-v2 |
| --- | --- |

| Title: | EE2-1.6/1.7a/1.7b: combinations of tests 1.2, 1.3, 1.4 and 1.5 |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | F. Le Léannec, K. Naser, T. Dumas, Y. Chen, M. Radosavljević, T. Poirier | Tel: Email: | fabrice.leleannec@interdigital.com |
| Source: | InterDigital |  |  |

_____________________________

# Abstract

This contribution reports the results of EE2 tests 1.6, 1.7a, and 1.7b, which were proposed in JVET-AF0275. Test 1.6 is the combination of test 1.5 (IntraTMP extension to LIC) and test 1.4 (IntraTMP extension to DIMD). Test 1.7a is the combination of test 1.6 and test 1.3 (SGPM with IntraTMP and IBC). Test 1.7b is the combination of test 1.7a and test 1.2 (IntraTMP with merge candidates).

The impact on coding efficiency and runtimes of these tests over ECM-11.0 are reportedly as follows for {Y, U, V, EncT, DecT, VmPeak}. Results reported in italic are not complete at the time this contribution is submitted, and missing test points are replaced by corresponding anchor points.

- Test 1.6:

  - AI  : { -0,05%	0,02%	-0,03%	102,4%	98,9%	100,2%	 93,2% }

  - RA : { -0,05%	-0,05%	-0,03%	#NUM!	#NUM!	#NUM!	#NUM!}

- Test 1.7a:

  - AI  : { -0,10%	-0,01%	-0,04%	103,2%	101,0%	101,4%	 94,1% }

  - RA : { -0,07%	-0,08%	-0,15%	#NUM!	#NUM!	#NUM! #NUM! }

- Test 1.7b:

  - AI  : { -0,21%	-0,15%	-0,15%	102,1%	99,9%	101,9%	 94,6% }

  - RA : { -0,12%	-0,17%	-0,13%	99,6%	99,6%	100,2%	 100,6% }

# Introduction

In tests EE2 test 1.2 to 1.5, several extensions of the block-vector based tools IntraTMP and IBC are proposed. Their basic principles are the following ones.

- Test 1.2 proposes to extend IntraTMP block vector candidates with IBC merge block vector candidates.

- Test1.3 proposes to extend geometric prediction by combining block-vector-based prediction and regular intra prediction from IBC-GPM to SGPM.

- Test 1.4 proposes to adaptively replace planar mode with block-vector based prediction in DIMD.

- Test 1.5 proposes to allow the application of LIC to blocks coded in IntraTMP mode.

In this contribution, tests 1.6, 1.7a and 1.7b are reported. They are the following combinations of above individual tests.

- Test 1.6 is the combination of tests 1.4 and 1.5.

- Test 1.7a is the combination of tests 1.6 and 1.3.

- Test 1.7b is the combination of test 1.7a and 1.2.

# Experimental results

This section reports the performances of each combined test.

Note that with respect to test 1.5 (IntraTMP-LIC), in all these combined tests, the fast trade-off of test 1.5, as reported in contribution JVET-AG0136, is used.

To understand the synergy that exists between proposed individual tools, this section also reports some additional tool-off experiment, which consists in the full combination 1.7b where test 1.5 (IntraTMP-LIC) is deactivated.

## Test 1.6

The following Table 1 shows the performances of test 1.6 as delivered in EE2 source code. Luma coding gain is -0.05% in AI, and -0.05% in RA. Encoding and decoding times are respectively 102.4% and 98.9% in AI. No runtime impact is observed in RA.

Table 1: performances of test 1.6 over ECM-11.0.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,04% | -0,04% | -0,08% | 103,8% | 100,3% | 101,7% | 95,5% |
| Class A2 | -0,12% | -0,08% | -0,02% | 103,3% | 98,7% | 99,8% | 95,1% |
| Class B | -0,05% | 0,05% | -0,07% | 101,9% | 97,9% | 98,0% | 94,1% |
| Class C | -0,03% | -0,04% | -0,06% | 101,5% | 99,1% | 101,3% | 89,8% |
| Class E | -0,01% | 0,23% | 0,11% | 102,4% | 98,9% | 101,5% | 92,2% |
| Overall | -0,05% | 0,02% | -0,03% | 102,4% | 98,9% | 100,2% | 93,2% |
| Class D | -0,03% | 0,02% | 0,00% | 100,6% | 98,9% | 99,8% | 91,2% |
| Class F | -0,11% | -0,15% | 0,24% | 102,3% | 102,1% | 98,0% | 90,9% |
| Class TGM | 0,12% | 0,14% | 0,11% | 100,0% | 98,4% | 101,1% | 95,3% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,06% | -0,07% | 0,03% | 102,1% | 102,8% | 99,8% | 99,8% |
| Class A2 | -0,03% | -0,06% | 0,02% | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0,06% | 0,04% | -0,15% | 101,1% | 99,3% | 100,0% | 100,0% |
| Class C | -0,05% | -0,14% | 0,02% | 98,3% | 102,6% | 100,0% | 101,5% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,05% | -0,05% | -0,03% | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,08% | -0,02% | -0,25% | 100,0% | 98,6% | 100,4% | 98,7% |
| Class F | -0,22% | -0,05% | -0,15% | 100,8% | 104,0% | 100,1% | 100,3% |
| Class TGM | -0,03% | -0,03% | -0,04% | 101,6% | 107,3% | 100,5% | 101,2% |

## Test 1.7a

The following Table 2 shows the performances of test 1.7a as delivered in EE2 source code. Luma coding gain is -0.10% in AI, and -0.07% in RA. Encoding and decoding times are respectively 103.2% and 101.0% in AI. No runtime impact is observed in RA.

Table 2: performance of test 1.7a over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,06% | -0,08% | -0,01% | 104,0% | 101,4% | 103,0% | 96,9% |
| Class A2 | -0,19% | -0,05% | -0,07% | 104,0% | 100,8% | 101,8% | 97,8% |
| Class B | -0,08% | 0,01% | -0,07% | 101,8% | 98,8% | 100,0% | 96,0% |
| Class C | -0,07% | -0,04% | 0,01% | 103,4% | 102,5% | 102,3% | 89,3% |
| Class E | -0,10% | 0,09% | -0,08% | 103,5% | 102,4% | 100,9% | 91,1% |
| Overall | -0,10% | -0,01% | -0,04% | 103,2% | 101,0% | 101,4% | 94,1% |
| Class D | -0,07% | -0,13% | 0,13% | 101,9% | 103,6% | 100,0% | 91,4% |
| Class F | -0,47% | -0,48% | -0,27% | 101,2% | 103,0% | 98,4% | 91,9% |
| Class TGM | -1,03% | -0,94% | -0,95% | 99,1% | 102,1% | 101,1% | 96,9% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,04% | -0,11% | -0,08% | 100,3% | 99,0% | 99,7% | 99,7% |
| Class A2 | -0,07% | -0,11% | -0,13% | #NUM! | #NUM! | #NUM! | #NUM! |
| Class B | -0,09% | -0,09% | -0,24% | 99,3% | 99,6% | 100,0% | 100,1% |
| Class C | -0,08% | -0,03% | -0,11% | 97,5% | 99,9% | 100,1% | 100,6% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,07% | -0,08% | -0,15% | #NUM! | #NUM! | #NUM! | #NUM! |
| Class D | -0,07% | -0,22% | -0,36% | 99,6% | 99,6% | 100,2% | 99,5% |
| Class F | -0,53% | -0,40% | -0,51% | 99,6% | 103,9% | 100,0% | 100,4% |
| Class TGM | -0,59% | -0,76% | -0,75% | 99,7% | 105,7% | 100,5% | 100,7% |

## Test 1.7b

The following Table 2 shows the performances of test 1.7b as delivered in EE2 source code. Luma coding gain is -0.21% in AI, and -0.12% in RA. Encoding and decoding times are respectively 102.1% and 99.9% in AI. No runtime impact is observed in RA.

Table 3: performances of test 1.7b over ECM-11.0

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,07% | -0,08% | -0,02% | 103,4% | 98,4% | 102,4% | 95,6% |
| Class A2 | -0,33% | -0,29% | -0,26% | 103,4% | 100,3% | 100,8% | 97,6% |
| Class B | -0,24% | -0,15% | -0,20% | 100,9% | 99,9% | 102,4% | 99,2% |
| Class C | -0,12% | -0,12% | -0,10% | 101,0% | 98,8% | 101,8% | 88,7% |
| Class E | -0,33% | -0,13% | -0,14% | 102,9% | 102,3% | 101,6% | 91,1% |
| Overall | -0,21% | -0,15% | -0,15% | 102,1% | 99,9% | 101,9% | 94,6% |
| Class D | -0,10% | -0,13% | 0,02% | 101,6% | 102,5% | 99,9% | 91,3% |
| Class F | -0,66% | -0,47% | -0,43% | 101,0% | 103,8% | 98,5% | 91,3% |
| Class TGM | -1,25% | -1,11% | -1,12% | 97,8% | 98,4% | 103,4% | 98,3% |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0,05% | -0,28% | -0,01% | 99,8% | 99,9% | 99,8% | 99,8% |
| Class A2 | -0,24% | -0,32% | -0,23% | 101,4% | 99,2% | 100,3% | 100,3% |
| Class B | -0,15% | -0,10% | -0,22% | 99,6% | 97,9% | 100,4% | 100,4% |
| Class C | -0,06% | -0,06% | -0,03% | 98,3% | 101,9% | 100,1% | 101,6% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0,12% | -0,17% | -0,13% | 99,6% | 99,6% | 100,2% | 100,6% |
| Class D | -0,08% | -0,19% | -0,49% | 98,7% | 99,5% | 100,2% | 97,5% |
| Class F | -0,60% | -0,51% | -0,42% | 100,3% | 106,0% | 100,1% | 101,3% |
| Class TGM | -0,82% | -1,05% | -1,06% | 99,2% | 106,7% | 100,1% | 100,5% |

## Additional tool-off test: Test 1.7b without test 1.5 (IntraTMP-LIC)

This section reports tool-off testing of test 1.5 (IntraTMP-LIC), i.e., combined test 1.7b with test 1.5 deactivated.

The following depicts the obtained performances relative to the full combination 1.7b. By deactivating test 1.5, a luma coding loss of 0.1% is obtained over test 1.7b.

This is to be compared with the individual gain of fast test 1.5 used, which is -0.3%. This demonstrates that coding gain of test 1.5 with other tools is more than additive.

Table 5: performances of test 1.7b without LIC over test 1.7b.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over Test EE2-1.7b |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0,04% | 0,03% | 0,07% | 97,5% | 101,1% | 99,8% | 102,2% |
| Class A2 | 0,18% | 0,15% | 0,15% | 97,4% | 100,7% | 100,2% | 101,2% |
| Class B | 0,10% | 0,09% | 0,07% | 98,6% | 102,0% | 97,0% | 98,6% |
| Class C | 0,05% | 0,07% | 0,18% | 99,5% | 104,5% | 98,4% | 100,7% |
| Class E | 0,12% | 0,05% | 0,04% | 97,6% | 100,4% | 99,2% | 99,4% |
| Overall | 0,10% | 0,08% | 0,10% | 98,2% | 101,9% | 98,7% | 100,2% |
| Class D | 0,02% | -0,08% | 0,10% | 98,0% | 100,4% | 99,7% | 99,7% |
| Class F | 0,17% | 0,15% | 0,18% | 100,1% | 100,2% | 98,7% | 99,9% |
| Class TGM | -0,01% | -0,08% | -0,07% | 101,6% | 106,1% | 96,5% | 98,9% |

# Patent rights declaration(s)

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).