| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0139-v1 |
| --- | --- |

| Title: | AHG16: Depthwise separable convolution for generative face video compression |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Renjie Zou Ru-Ling Liao Bolin Chen Jie Chen Yan Ye | Email: | {zourenjie.zrj, ruling.lrl, chenbolin.chenboli, jiechen.cj, yan.ye}@alibaba-inc.com |
| Source: | Alibaba Group |  |  |

_____________________________

- Abstract

This contribution proposes to introduce the depthwise separable convolution into the CFTE method of generative face video compression. More specifically, partial 3x3 convolution layers in CFTE are replaced by depthwise separable convolution to reduce the number of parameters and computation complexity. It is reported that compared with the original CFTE, the proposed method reduces number of parameters and kMACs per pixel by 41% and 4%, respectively. The overall coding performance impact of the proposed method for {DISTS, LPIPS, RGB444_PSNR, RGB444_SSIM, YUV420_PSNR, YUV420_SSIM} is {-62.16%, -60.17%, -5.49%, -33.8%, -2.6%, #VALUE!} when comparing with VTM-22.2 and {-4.46%, -5.16%, -1.83%, -10.7%, -8.6%, -1.6%} when comparing with the original CFTE.

Introduction

CFTE [1] is a talking face video compression framework, and its block diagram is illustrated in Figure 1. There are three models in CFTE including analysis model, dense motion model and generative model. In CFTE, the base picture is encoded using VTM software, whereas the subsequent pictures are analyzed using the analysis model and the features are coded at the encoder side. At the decoder side, the base picture is firstly decoded using VTM software and the reconstructed basic picture is fed into the analysis model to get its reconstructed features. Then, the dense motion model uses the base picture features and the decoded features of the subsequent pictures as input to generate motion information. Finally, the generative model generates reconstructed subsequent pictures using the generated motion information and reconstructed base picture.

In the current software package proposed in JVET-AG0042 [2], the number of parameters of the CFTE decoder is 58M and computation complexity is 852 kMAC per pixel. In this contribution, we propose to simplify the CFTE by replacing 3x3 convolution layers with depthwise separable convolution [3]. The depthwise separable convolution is an efficient convolutional operation used in deep learning, particularly within Convolutional Neural Networks (CNNs), to reduce the computational cost and the number of parameters of the network without significantly reducing performance.

Figure 1. Illustration of framework of CFTE

Proposed method

An analysis is performed to measure the number of parameters of each model in CFTE framework, and the results is shown in Table 1. As can be seen, the dense motion model has the highest number of parameters among all three models. Therefore, we propose to modify the dense motion model by replacing the 3x3 convolution with depthwise separable convolution. The structure of dense motion model is shown in Figure 2. The motion down block and motion up block consist of 3x3 convolution, batchnorm, relu, and an interpolation function. The 3x3 convolution is a vanilla convolution without group convolution, as the groups equal 1.

Table 1. Number of parameters of each model in CFTE framework

| Model | Analysis model | Dense motion model | Generative model |
| --- | --- | --- | --- |
| Number of parameters | 14M | 36M | 7M |

Figure 2. Illustration of (a) dense motion model and (b) motion down block and (c) motion up block

Figure 3 shows the proposed dense motion model, called depthwise dense motion model. The main changes are that the 3x3 convolution is group convolution with groups equal to the number of channels and a 1x1 convolution is introduced.

Figure 3. Illustration of (a) depthwise dense motion model and (b) depthwise motion down block and (c) depthwise motion up block

Simulation results

The proposed method is implemented on top of the software package in JVET-AG0042, and is evaluated using the same test conditions as proposed in JVET-AG0042. Table 2 and Table 3 show the simulation results of the proposed CFTE over VTM-22.2 and the original CFTE in JVET-AG0042 software, respectively. It can be seen that overall, the proposed CFTE performs better than both VTM-22.2 and the original CFTE. It should be mentioned that the result “#VALUE!” means the RD curves cross over each other for some sequences so that the BD-rate cannot be calculated.

Table 2. Simulation results of proposed CFTE over VTM-22.2

|  | Over VVC (VTM-22.2) |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -67.12% | -65.88% | -5.26% | -37.0% | -2.1% | -34.8% |
| Class B | -57.21% | -54.46% | -5.72% | -30.6% | -3.1% | #VALUE! |
| Overall | -62.16% | -60.17% | -5.49% | -33.8% | -2.6% | #VALUE! |

Table 3. Simulation results of proposed CFTE over the original CFTE

|  | Over the original CFTE |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | DISTS | LPIPS | RGB444_PSNR | RGB444_SSIM | YUV420_PSNR | YUV420_SSIM |
| Class A | -6.52% | -6.32% | -4.56% | -8.0% | -3.7% | -6.9% |
| Class B | -2.40% | -4.01% | 0.90% | -13.5% | -13.5% | 3.8% |
| Overall | -4.46% | -5.16% | -1.83% | -10.7% | -8.6% | -1.6% |

Figure 4 shows the RD curves of the CFTE in JVET-AG0042 and the proposed CFTE, and the results are measured on RGB444_PSNR, RGB444_SSIM, DISTS, LPIPS.

Figure 4 The RD curves of the CFTE in JVET-AG0042 and the proposed CFTE

Table 4 summarizes the number of parameters and kMAC per pixel of the original and proposed CFTE. It can be seen that with the depthwise separable convolution, the number of parameters is reduced from 58M to 34M, which is roughly decreased by 41%. The kMAC per pixel is reduced from 852 to 819, which is roughly decreased by 4%.

Table 4. kMACs/pixel and total parameter number of the original CFTE and proposed CFTE

|  | kMACs/pixel | Total Parameter Number |
| --- | --- | --- |
| Original CFTE | 852 | 58M |
| Proposed CFTE | 819 | 34M |

Conclusions

This contribution presents a lightweight method reducing parameters and computation complexity of CFTE. In the proposed CFTE, we replace 3x3 convolution with depthwise separable convolution. The results show that the number of parameters and kMAC per pixel can be reduced by 41% and 4%, respectively. Moreover, the proposed CFTE achieves more than 1% coding gain over the original CFTE. The simulation results prove the effectiveness of our proposed CFTE. We recommend to adopt this contribution to the GFVC software package.

References

- B. Chen, et al., “Beyond keypoint coding: Temporal evolution inference with compact feature representation for talking face video compression,” Proc. Data Compression Conference (DCC), Mar. 2022.

- B. Chen, et al., “AHG16: Proposed Common Software Tools and Testing Conditions for Generative Face Video Compression,” JVET-AG0042, Jan. 2024.

- F. Chollet, “Xception: Deep learning with depthwise separable convolutions,” IEEE Conference on Computer Vision and Pattern Recognition (CVPR), Nov. 2017.

Patent rights declaration(s)

Alibaba group may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).