| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0142-v1 |
| --- | --- |

| Title: | EE2-2.11d/e/f: Combination of Test 2.9, Test 2.10 and Test 2.11a |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lei Zhao, Kai Zhang, Li Zhang (Bytedance Inc.) Philippe Bordes, Kevin Reuzé (InterDigital) | Tel: Email: | {zhaolei.0324, zhangkai.video, lizhang.idm} @bytedance.com {philippe.bordes,kevin.reuze}@interdigital.com |
| Source: | Bytedance Inc. InterDigital |  |  |

_____________________________

Abstract

This proposal reports the performance of Test-2.11d, Test-2.11e and Test-2.11f. It is reported that on top of ECM-11.0, simulation results of the proposed methods are as below:

Test 2.11d: Combination of Test 2.10 and Test 2.11a

RA: -0.22 % (Y), -0.25 % (U), -0.26 % (V), xxx% (EncT), xxx% (DecT)

LB: -0.20 % (Y), 0.07 % (U), -0.30% (V), xxx% (EncT), xxx% (DecT)

Test 2.11e: Combination of Test 2.9 and Test 2.10

RA: xxx % (Y), xxx% (U), xxx % (V), xxx% (EncT), xxx% (DecT)

LB: xxx % (Y), xxx % (U), xxx % (V), xxx% (EncT), xxx% (DecT)

Test 2.11f: Combination of Test 2.9, Test 2.10 and Test 2.11a

RA: xxx % (Y), xxx % (U), xxx % (V), xxx% (EncT), xxx% (DecT)

LB: xxx % (Y), xxx % (U), xxx % (V), xxx% (EncT), xxx% (DecT)

- Introduction

At the 32th JVET meeting, Test 2.9 is established to investigate CIIP with subblock-based motion compensation as presented in JVET-AF0170, where CIIP is improved by allowing subblock-based inter prediction. In Test 2.10, a method of geometric partitioning mode (GPM) with affine prediction is studied, where a GPM partition predicted by AMC can be blended with the other GPM partition predicted by AMC, non-AMC, or intra-prediction. In Test 2.11a, an additional GPM implicit mode is proposed, which derives the splitting shape and blending function based on the template, where the blending matrices are calculated based on LDL decomposition method. In this contribution, the experimental results of three combination tests, i.e., Test 2.11d (the combination of Test 2.10 and Test 2.11a), Test 2.11e (the combination of Test 2.9 and Test 2.10) and Test 2.11f (the combination of Test 2.9, Test 2.10 and Test 2.11a), are presented.

- Experimental results

Three experiments are tested on top of ECM-11.0 under the common test conditions [1], which are listed as follows:

Test 2.11d: Combination of Test 2.10 and Test 2.11a

Test 2.11e: Combination of Test 2.9 and Test 2.10

Test 2.11f: Combination of Test 2.9, Test 2.10 and Test 2.11a

Table 1: Coding performance of Test 2.11d

|  | RA |  |  |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Vm Peak | Y | U | V | EncT | DecT | Vm Peak |
| Class A1 | -0.16% | -0.25% | -0.23% |  |  |  |  |  |  |  |  |  |
| Class A2 | -0.27% | -0.17% | -0.28% |  |  |  |  |  |  |  |  |  |
| Class B | -0.22% | -0.27% | -0.26% |  |  |  | -0.16% | 0.11% | 0.04% |  |  |  |
| Class C | -0.24% | -0.29% | -0.26% |  |  |  | -0.10% | -0.11% | -0.40% |  |  |  |
| Class E |  |  |  |  |  |  | -0.39% | 0.26% | -0.73% |  |  |  |
| Overall | -0.22% | -0.25% | -0.26% |  |  |  | -0.20% | 0.07% | -0.30% |  |  |  |
| Class D | -0.17% | -0.10% | -0.17% |  |  |  | -0.23% | 0.07% | -0.23% |  |  |  |
| Class F | -0.15% | -0.14% | -0.15% |  |  |  | -0.08% | -0.05% | -0.07% |  |  |  |
| Class TGM | -0.28% | -0.25% | -0.27% |  |  |  | -0.62% | -0.52% | -0.54% |  |  |  |

Table 2: Coding performance of Test 2.11e

|  | RA |  |  |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Vm Peak | Y | U | V | EncT | DecT | Vm Peak |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.19% | -0.36% | -0.27% |  |  |  |  |  |  |  |  |  |
| Class C | -0.23% | -0.13% | -0.19% |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.23% | -0.02% | -0.06% |  |  |  |  |  |  |  |  |  |
| Class F |  |  |  |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

Table 3: Coding performance of Test 2.11f

|  | RA |  |  |  |  |  | LB |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  | Y | U | V | EncT | DecT | Vm Peak | Y | U | V | EncT | DecT | Vm Peak |
| Class A1 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |  |  |  |  |  |
| Class B | -0.28% | -0.21% | -0.34% |  |  |  |  |  |  |  |  |  |
| Class C | -0.31% | -0.28% | -0.31% |  |  |  |  |  |  |  |  |  |
| Class E |  |  |  |  |  |  |  |  |  |  |  |  |
| Overall |  |  |  |  |  |  |  |  |  |  |  |  |
| Class D | -0.20% | -0.11% | -0.14% |  |  |  |  |  |  |  |  |  |
| Class F | -0.18% | -0.10% | -0.19% |  |  |  |  |  |  |  |  |  |
| Class TGM |  |  |  |  |  |  |  |  |  |  |  |  |

- Conclusions

This proposal reports the performance of Test-2.11d, Test-2.11e and Test-2.11f. Simulation results indicate attractive coding gain in RA/LDB configurations with acceptable computational complexity. It is recommended to adopt this contribution in ECM.

- References

[1] M. Karczewicz, Y. Ye, “Common Test Conditions and evaluation procedures for enhanced compression tool testing,” JVET-Y2017, Jan. 2022.

- Patent rights declaration(s)

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

InterDigital may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).