| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0158-v1 |
| --- | --- |

| Title: | EE2-4.3: Adaptive precision for luma ALF coefficients |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Nan Hu, Marta Karczewicz, Hongtao Wang, Vadim Seregin 5775 Morehouse Dr, San Diego, CA 92121, USA Wenbin Yin1, Kai Zhang2, Li Zhang2 1Zijinshumayuan 4th building F9, 18 Zhongguancun Nansijie, Haidian District, 100190, Beijing, China 28910 University Center Lane, San Diego, CA 92122, USA | Tel: Email: | nanh@qti.qualcomm.com martak@qti.qualcomm.com hongtaow@qti.qualcomm.com vseregin@qti.qualcomm.com yinwenbin.hit@bytedance.com zhangkai.video@bytedance.com lizhang.idm@bytedance.com |
| Source: | Qualcomm Incorporated and Bytedance Inc. |  |  |

_____________________________

# Abstract

In adaptive loop filter of ECM-11.0, when filtering a luma sample with an on-line derived filter, 7 bits are used to represent the fractional part of a coefficient. In test EE2-4.3, the number of bits used to represent the fractional part of a luma coefficient is adaptive from 5 to 8, inclusively. A two-bit syntax element is signalled for each set of luma filter set to indicate the number of bits. On top of ECM-11.0 with common test condition, simulation results of the proposed method are reported as follows.

|  | BD-rate Y | BD-rate U | BD-rate V | EncT | DecT | VmPeakE | VmPeakD |
| --- | --- | --- | --- | --- | --- | --- | --- |
| All intra | -0.04% | 0.00% | 0.00% | 99.2% | 97.5% | 100.1% | 100.1% |
| Random access | -0.07% | -0.04% | -0.06% | 99.9% | 100.2% | 100.0% | 100.1% |
| Low delay B | -0.10% | -0.01% | 0.00% | 98.4% | 98.8% | 101.1% | 99.3% |

# Introduction

In ECM-11.0, a luma coefficient of adaptive loop filter (ALF) is represented by a fixed-point 8-bit decimal number. The fractional part is represented by a 7-bit number and 1 bit is used for the sign. The real value range of a coefficient is within [-1, 1).

# Proposed method

In test EE2-4.3, the number of bits used to represent the fractional part of a luma coefficient can vary from 5 to 8 adaptively. For each luma filter set, which contains up to 25 filters, a 2-bit syntax element is signalled to indicate the number of bits used for the coefficients in this set. The real value range of a coefficient is not changed.

# Simulation results

The proposed method was implemented on top of ECM-11.0 and tested following common test condition described in JVET-AF2017. Table 1 presents the simulation results of test EE2-4.3.

Table 1. Results of test EE2-4.3.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.01% | 0.00% | 0.00% | 98.4% | 96.3% | 100.1% | 100.1% |
| Class A2 | -0.02% | 0.00% | 0.00% | 100.2% | 97.1% | 100.1% | 100.1% |
| Class B | -0.03% | 0.00% | 0.00% | 98.6% | 96.5% | 100.1% | 100.1% |
| Class C | -0.07% | 0.00% | 0.00% | 100.3% | 100.7% | 100.2% | 100.0% |
| Class E | -0.05% | 0.00% | 0.00% | 98.4% | 96.9% | 100.2% | 100.0% |
| Overall | -0.04% | 0.00% | 0.00% | 99.2% | 97.5% | 100.1% | 100.1% |
| Class D | -0.07% | 0.02% | 0.02% | 96.7% | 100.2% | 100.0% | 100.0% |
| Class F | -0.06% | 0.03% | 0.03% | 100.0% | 101.2% | 100.2% | 100.0% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | -0.05% | -0.12% | -0.07% | 99.5% | 99.9% | 99.9% | 100.0% |
| Class A2 | -0.02% | 0.08% | -0.03% | 99.6% | 99.4% | 100.0% | 100.2% |
| Class B | -0.08% | -0.02% | -0.10% | 99.4% | 101.0% | 99.8% | 100.1% |
| Class C | -0.12% | -0.09% | -0.01% | 101.3% | 100.1% | 100.2% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | -0.07% | -0.04% | -0.06% | 99.9% | 100.2% | 100.0% | 100.1% |
| Class D | -0.13% | 0.13% | -0.01% | 92.7% | 101.4% | 100.4% | 100.0% |
| Class F | -0.09% | -0.04% | 0.01% | 98.4% | 101.0% | 100.6% | 100.1% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.11% | -0.12% | 0.01% | 97.5% | 98.5% | 100.6% | 98.4% |
| Class C | -0.04% | 0.21% | 0.08% | 101.4% | 99.1% | 100.7% | 99.9% |
| Class E | -0.17% | -0.13% | -0.11% | 95.8% | 99.0% | 102.6% | 100.0% |
| Overall | -0.10% | -0.01% | 0.00% | 98.4% | 98.8% | 101.1% | 99.3% |
| Class D | -0.27% | -0.15% | -1.22% | 93.4% | 103.0% | 100.6% | 100.0% |
| Class F | -0.10% | -0.09% | -0.42% | 96.8% | 98.1% | 99.2% | 100.3% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

In the encoder of ECM-11.0, after deriving the filter coefficients from Cholesky decomposition, each coefficient could be further decreased or increased by 1 iteratively to reduce the distortion. When calculating the distortion, the whole covariance matrix is used. Since only one coefficient is changed each time, during the development of test EE2-4.3, the encoder is optimized by calculating the delta distortion by using the partial covariance matrix related to the changed coefficient in every step. Table 2 presents the simulation results of the encoder-only optimization implemented on top of ECM-11.0.

Table 2. Results of encoder-only optimization.

|  | All Intra Main 10 |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.00% | 98.9% | 98.7% | 100.0% | 100.0% |
| Class A2 | 0.00% | 0.00% | 0.00% | 101.0% | 98.8% | 100.0% | 100.0% |
| Class B | 0.00% | 0.00% | 0.00% | 99.0% | 98.0% | 100.0% | 100.0% |
| Class C | 0.00% | 0.00% | 0.00% | 99.5% | 102.4% | 100.0% | 100.0% |
| Class E | 0.00% | 0.00% | 0.00% | 98.4% | 99.7% | 100.0% | 100.0% |
| Overall | 0.00% | 0.00% | 0.00% | 99.3% | 99.5% | 100.0% | 100.0% |
| Class D | 0.00% | 0.00% | 0.00% | 97.1% | 102.1% | 99.9% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 98.5% | 99.6% | 100.0% | 99.9% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Random Access Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 | 0.00% | 0.00% | 0.02% | 99.9% | 100.0% | 99.9% | 99.8% |
| Class A2 | 0.00% | 0.00% | 0.00% | 100.5% | 100.6% | 100.0% | 100.4% |
| Class B | 0.00% | 0.01% | -0.02% | 99.5% | 100.6% | 99.7% | 100.1% |
| Class C | 0.00% | 0.01% | 0.00% | 100.4% | 101.7% | 98.7% | 100.0% |
| Class E |  |  |  |  |  |  |  |
| Overall | 0.00% | 0.00% | 0.00% | 100.0% | 100.8% | 99.5% | 100.1% |
| Class D | 0.00% | 0.00% | 0.00% | 92.6% | 101.1% | 99.9% | 100.0% |
| Class F | 0.00% | 0.00% | 0.00% | 98.5% | 100.4% | 100.1% | 100.2% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |
|  | Low delay B Main 10 |  |  |  |  |  |  |
|  | Over ECM-11.0 |  |  |  |  |  |  |
|  | Y | U | V | EncT | DecT | EncVmPeak | DecVmPeak |
| Class A1 |  |  |  |  |  |  |  |
| Class A2 |  |  |  |  |  |  |  |
| Class B | -0.01% | 0.17% | 0.18% | 98.8% | 100.0% | 100.1% | 99.5% |
| Class C | 0.03% | -0.02% | -0.03% | 100.5% | 103.0% | 100.0% | 99.7% |
| Class E | 0.00% | -0.03% | 0.01% | 94.2% | 99.1% | 102.2% | 99.7% |
| Overall | 0.01% | 0.06% | 0.07% | 98.2% | 100.8% | 100.6% | 99.6% |
| Class D | 0.00% | 0.00% | 0.00% | 92.6% | 103.3% | 99.4% | 100.0% |
| Class F | 0.04% | -0.01% | -0.02% | 97.8% | 101.3% | 100.1% | 99.5% |
| Class TGM | #VALUE! | #VALUE! | #VALUE! | #DIV/0! | #DIV/0! | #DIV/0! | #DIV/0! |

# Patent rights declaration(s)

Qualcomm Incorporated may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).

Bytedance Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).