| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–27 January 2024 | Document: JVET-AG0165-v1 |
| --- | --- |

| Title: | AHG9: On the processing order nesting SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

This contribution proposes to modify the syntax structure for signlling the number of the processing-order-nested SEI messages and the processing orders for each of the target processing order id pon_target_po_id[ i ], adding flexibilities for multiple SEI processing order SEI messages with different values of po_id in CVS.

# Introduction

One aspect of JVET-AF2027 updated from the JVET 32nd meeting is the addition of the processing order nesting SEI message in that po_sei_wrapping_flag[ i ] equal to 1 enables for SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message.

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(8) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1 | u(8) |
| for( i = 0; i <= pon_num_seis_minus1; i++ ) { |  |
| pon_processing_order[ i ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |

The processing order nesting SEI message includes one or more SEI messages that should be applied only as parts of the processing chain identified by an associated SEI processing order SEI message and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI message.

The SEI messages contained in the processing order nesting SEI message are also referred to as the processing-order-nested SEI messages.

The persistence of all the SEI messages included in the same processing order nesting SEI message shall be the same.

…

However, in the syntax structure as designed above, each of the target processing order id pon_target_po_id[ i ] associates with the same number of the processing-order-nested SEI messages and the processing orders rather than a target processing order id specific signalling.

# Proposal

This contribution proposes to modify the syntax structure for signlling the number of the processing-order-nested SEI messages and the processing orders for each of the target processing order id pon_target_po_id[ i ], adding flexibilities for multiple SEI processing order SEI messages with different values of po_id in CVS. The modified parts are highlighted as follows.

### D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for a group of types of SEI messages that may be present in a CVS.

…

po_sei_wrapping_flag[ i ] equal to 1 specifies that one or more processing order nesting SEI messages with both of the following constraints should be present:

- pon_target_po_id[ j ] with any value of j is equal to po_id.

- There is a k-th loop entry in the SEI processing order nesting SEI message such that the payloadType of the k-th nested SEI message is equal to po_sei_payload_type[ i ] and pon_processing_order[po_id][ k ] is equal to po_sei_processing_order[ i ].

When po_sei_wrapping_flag[ i ] is equal to 0, an SEI message with payloadType equal to po_sei_payload_type[ i ] (and, when po_sei_prefix_flag[ i ] equal to 1, prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ]) should be present outside of the processing order nesting SEI message. However, if po_sei_wrapping_flag[ i ] is equal to 0 and no SEI message is present with payloadType equal to po_sei_payload_type[ i ] or po_sei_wrapping_flag[ i ] is equal to 0 and po_sei_prefix_flag[ i ] is equal to 1 and no SEI message is present with with payloadType equal to po_sei_payload_type[ i ] that has prefix data that matches the values of po_sei_prefix_data_bit[ i ][ j ], the following applies:

- If po_sei_importance_flag[ i ] is equal to 1, the decoder should ignore the entire SEI processing order SEI message.

- Otherwise, the decoder should ignore all data associated with the loop variable value of i.

NOTE 2 – po_sei_wrapping_flag[ i ] equal to 1 enables SEI messages to be carried within the processing order nesting SEI message to prevent such SEI messages from being incorrectly interpreted by decoders that do not process the SEI processing order SEI message. Thus, po_sei_wrapping_flag[ i ] equal to 1 is intended to be used when po_sei_wrapping_flag[ i ] equal to 0 can lead to unintended results being produced by such decoders.

…

## D.12 Processing order nesting SEI message

### D.12.1 Processing order nesting SEI message syntax

| processing_order_nesting( payloadSize ) { | Descriptor |
| --- | --- |
| pon_num_po_ids_minus1 | u(8) |
| for( i = 0; i <= pon_num_po_ids_minus1; i++ ) { |  |
| pon_target_po_id[ i ] | u(8) |
| pon_num_seis_minus1[ i ] | u(8) |
| for(ij = 0; ij <= pon_num_seis_minus1[ i ]; ij++ ) { |  |
| pon_processing_order[ i ] [ j ] | u(8) |
| sei_message( ) |  |
| } |  |
| } |  |
| } |  |

### D.12.2 Processing order nesting SEI message semantics

The processing order nesting SEI message includes one or more SEI messages that should be applied only as parts of the processing chain identified by an one or more associated SEI processing order SEI messages and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI messages.

The SEI messages contained in the processing order nesting SEI message are also referred to as the processing-order-nested SEI messages.

The persistence of all the SEI messages included in the same processing order nesting SEI message shall be the same.

pon_num_po_ids_minus1 plus 1 specifies the number of the SEI processing order SEI messages SEI associated with this processing order nesting SEI message.

pon_target_po_id[ i ] indicates the po_id of the i-th associated SEI processing order SEI message.

pon_num_seis_minus1[ i ] plus 1 specifies the number of the processing-order-nested SEI messages that are included in this SEI message associated with the i-th SEI processing order SEI message (with po_id equal to pon_target_po_id[ i ]).

pon_processing_order[ i ][ j ] specifies the position of the ij-th processing-order-nested SEI message within the processing order defined by the i-th associated SEI processing order SEI message (with po_id equal to pon_target_po_id[ i ]). When ij is greater than 0, pon_processing_order[ i ][ j ] shall be greater than or equal to pon_processing_order[ i ] [ j - 1 ].

For each associated SEI processing order SEI message with po_id equal to pon_target_po_id[ i ] for i in the range of 0 to pon_num_po_ids_minus1, inclusive, there shall be at least one value of ij in the range of 0 to pon_num_seis_minus1[ i ], inclusive, in the processing order nesting SEI message for which the i-th associated SEI processing order SEI message has some entry k for which all of the following are true:

–	po_sei_processing_order[ k ] is equal to pon_processing_order[ i ][ j ]

–	po_sei_payload_type[ k ] is equal to the payloadType value of the ij-th processing-order-nested SEI message corresponding to pon_target_po_id[ i ].

–	When po_sei_prefix_flag[ k ] is equal to 1, po_sei_prefix_data_bit[ k ][ jm ] for jm in the range of 0 to po_num_bits_in_prefix_indication_minus1[ k ], inclusive, contain the same content as the po_num_bits_in_prefix_indication_minus1[ k ] plus 1 initial bits of the SEI message payload of the ij-th processing-order-nested SEI message corresponding to pon_target_po_id[ i ].

The ij-th processing-order-nested SEI message should be applied as the k-th loop entry of the associated SEI processing order SEI message with po_id equal to pon_target_po_id[ i ].

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).