| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–27 January 2024 | Document: JVET-AG0166-v1 |
| --- | --- |

| Title: | AHG9: On grouping the SEI prefix data as a new SEI message |  |  |
| --- | --- | --- | --- |
| Status: | Input document to JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Lulin Chen Olena Chubach Yu-Wen Huang Shawmin Lei | Email: | {lulin.chen, olena.chubach, yuwen.huang, shawmin.lei}@mediatek.com |
| Source: | MediaTek, Inc. |  |  |

_____________________________

# Abstract

Considering both the wrapping flag and the prefix flag are used as the special cases for the SEI processing order SEI message, to further simplify the syntax structure of the SEI processing order SEI message, this contribution proposes to group the SEI prefix data related syntax structure as a new SEI message – the processing order prefix matching SEI message.

The modified text for the SEI processing order SEI message in this document (JVET-AG0166) and a separate contribution JVET-AG00165 are combined in another contribution JVET-AG0168 for overall clarity and completeness.

Introduction

One aspect of JVET-AF2027 updated from the JVET 32nd meeting is the addition of the processing order nesting SEI message. Another updated aspect is the separate loop for the SEI prefix data related syntax structure. (As briefly enclosed below).

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

### D.11.2 SEI processing order SEI message semantics

The SEI processing order SEI message carries information indicating the preferred processing order, as determined by the encoder (i.e., the content producer), for a group of types of SEI messages that may be present in a CVS.

The semantics of the SEI processing order SEI message uses the concept of types of SEI messages. SEI messages that have different payloadType values are considered different types of SEI messages. Additionally, different SEI messages that have the same payloadType value but are differentiated by values of syntax elements in the SEI payload are considered different types of SEI messages. Such differentiation by values of syntax elements in the SEI payload is to be performed by comparing values sent using po_sei_prefix_data_bit[ i ][ j ] syntax elements (when present) or values sent as SEI messages within a processing order nesting SEI message (when present). For example, neural-network post-filter characteristics (NNPFC) SEI messages can be differentiated by having different nnpfc_id values.

When an SEI processing order SEI message with a particular value of po_id is present in any access unit of a CVS, an SEI processing order SEI message with that particular value of po_id shall be present in the first access unit of the CVS in decoding order. The number of SEI messages and the payloadType codes of the SEI messages indicated within each SEI processing order SEI message with the same value of po_id persists in decoding order from the current access unit until the end of the CVS in output order.

The SEI processing order SEI message can carry one or more SEI prefix indications of a particular payloadType. When present, each SEI prefix indication is a bit string that follows the SEI payload syntax of that value of payloadType and contains a number of complete syntax elements starting from the first syntax element in the SEI payload.

These SEI prefix indications should provide sufficient information to determine the specific processing order for types of SEI messages having the same value of payloadType but a different preferred processing order.

…

# Proposal

This contribution proposes to group the SEI prefix data related syntax structure as a new SEI message, as the proponents view the wrapping flag and/or the prefix flag equal to 1 as the special cases for the SEI processing order SEI message and in general use-cases the two flags may not be equal to 1. The proposed grouping may further simplify the syntax structure of the SEI processing order SEI message. Below are the proposed syntax and semantic modifications (with some highlights in green) for the new SEI message – the processing order prefix matching SEI message as Clause D.13 for the SEI processing order SEI message. Accordingly, the SEI prefix data related syntax is removed from the SEI processing order SEI message.

The modified text for the SEI processing order SEI message in this document (JVET-AG0166) and a separate contribution JVET-AG00165 are combined in another contribution JVET-AG0168 for overall clarity and completeness.

D.11.1 SEI processing order SEI message syntax

| sei_processing_order( payloadSize ) { | Descriptor |
| --- | --- |
| po_id | u(8) |
| po_num_sei_messages_minus2 | u(8) |
| for( i = 0, i < po_num_sei_messages_minus2 + 2; i++ ) { |  |
| po_sei_wrapping_flag[ i ] | u(1) |
| po_sei_importance_flag[ i ] | u(1) |
| po_sei_payload_type[ i ] | u(13) |
| po_sei_prefix_flag[ i ] | u(1) |
| po_sei_processing_order[ i ] | u(8) |
| } |  |
| for( i = 0; i < po_num_sei_messages_minus2 + 2; i++ ) |  |
| if( po_sei_prefix_flag[ i ] ) { |  |
| po_num_bits_in_prefix_indication_minus1[ i ] | u(8) |
| for( j = 0; j <= po_num_bits_in_prefix_indication_minus1[ i ]; j++ ) |  |
| po_sei_prefix_data_bit[ i ][ j ] | u(1) |
| while( !byte_aligned( ) ) |  |
| po_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |

## D.13 Processing order prefix matching SEI message

### D.13.1 Processing order prefix matching SEI message syntax

| processing_order_prefix_matching( payloadSize ) { | Descriptor |
| --- | --- |
| popm_number_po_ids_minus1 | u(8) |
| for( i = 0; i <= popm_num_po_ids_minus1; i++ ) { |  |
| popm_target_po_id[ i ] | u(8) |
| popm_num_sei_prefix_indications_minus1[ i ] | u(8) |
| for( j = 0; j <= popm_num_sei_prefix_indications_minus1[ i ]; j++ ) { |  |
| popm_processing_order[ i ][ j ] | u(8) |
| popm_num_bits_in_prefix_indication_minus1[ i ][ j ] | b(8) |
| for( n = 0; n <= popm_num_bits_in_prefix_indication_minus1[ i ][ j ]; n++ ) |  |
| popm_sei_prefix_data_bit[ i ][ j ][ n ] | u(1) |
| while( !byte_aligned( ) ) |  |
| popm_byte_alignment_bit_equal_to_one /* equal to 1 */ | f(1) |
| } |  |
| } |  |
| } |  |

### D.13.2 SEI processing order prefix matching SEI message semantics

The processing order prefix matching SEI message includes one or more prefix indications of SEI messages with payloadType equal to po_sei_payload_type[ i ] when po_sei_prefix_flag[ i ] equal to 1. The SEI messages with the prefix data matching the prefix indications contained in this SEI message should be applied only as parts of the processing chain identified by one or more associated SEI processing order SEI messages and should not be applied in a manner that would contradict with the processing chain identified by the associated SEI processing order SEI messages.

The SEI messages with SEI prefix data matching the SEI prefix indications contained in the processing order prefix matching SEI message are also referred to as the processing-order-prefix-matched SEI messages

popm_num_po_ids_minus1 plus 1 specifies the number of the SEI processing order SEI messages SEI associated with this processing order prefix matching SEI message.

popm_target_po_id[ i ] indicates the po_id of the i-th associated SEI processing order SEI message.

popm_num_sei_prefix_indications_minus1[ i ] plus 1 specifies the number of the prefix indications of the processing-order-prefix-matched SEI messages that are included in this SEI message associated with the i-th SEI processing order SEI message (with po_id equal to popm_target_po_id[ i ]).

popm_processing_order[ i ][ j ] specifies the position of the j-th processing-order-prefix-matched SEI message within the processing order defined by the i-th associated SEI processing order SEI message. When j is greater than 0, popm_processing_order[ i ][ j ] shall be greater than or equal to popm_processing_order[ i ][ j − 1 ].

For each associated SEI processing order SEI message with po_id equal to popm_target_po_id[ i ] for i in the range of 0 to popm_num_po_ids_minus1, inclusive, there shall be at least one value of j in the range of 0 to popm_num_sei_prefix_indications_minus1[ i ], inclusive, in the processing order prefix matching SEI message for which the i-th associated SEI processing order SEI message has some entry r for which all of the following are true:

–	po_sei_processing_order[ r ] is equal to popm_processing_order[ po_id ][ j ]

- po_sei_payload_type[ r ] is equal to the payloadType value of the j-th processing-order-prefix-matched SEI message corresponding to po_id.

–	When po_sei_prefix_flag[ r ] is equal to 1, popm_sei_prefix_data_bit[ po_id ][ r ][ n ] for n in the range of 0 to popm_num_bits_in_prefix_indication_minus1[ po_id ][ r ], inclusive, contain the same content as the popm_num_bits_in_prefix_indication_minus1[ po_id ][ r ] plus 1 initial bits of the SEI message payload of the j-th processing-order-prefix-matched SEI message corresponding to po_id.

The j-th processing-order-prefix-matched SEI message should be applied as the r-th loop entry of the associated SEI processing order SEI message (with po_id equal to popm_target_po_id[ i ]).

popm_num_bits_in_prefix_indication_minus1[ po_id ][ j ] and popm_sei_prefix_data_bit[ po_id ][ j ][ n ], for the j-th associated SEI processing order SEI message, when present, have the same semantics as the num_bits_in_prefix_indication_minus1[ j ] and sei_prefix_data_bit[ j ][ n ] syntax elements of the SEI prefix indication SEI message, with prefix_sei_payload_type replaced by po_sei_payload_type[ j ].

popm_byte_alignment_bit_equal_to_one shall be equal to 1.

# Patent rights declaration(s)

MediaTek Inc. may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).