| Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 33rd Meeting, by teleconference, 17–26 January 2024 | Document: JVET-AG0178-v3 |
| --- | --- |

| Title: | AHG8: Truncating bit depth in video coding for machine tasks |  |  |
| --- | --- | --- | --- |
| Status: | Input document of JVET |  |  |
| Purpose: | Proposal |  |  |
| Author(s) or Contact(s): | Ding Ding Xin Zhao Shan Liu | Email: | ddding@global.tencent.com xinzzhao@global.tencent.com shanl@global.tencent.com |
| Source: | Tencent |  |  |

_____________________________

# Abstract

Currently, video content is usually represented with a bit depth of eight or higher, such as 8-bit or 10-bit, widely used for video communication tailored to human vision. However, for machine vision tasks like object tracking and detection, the quality score, for instance, measured by detection rate, exhibits less sensitivity to bit depth. Hence, a lower sample value range, represented by fewer bits, can better balance bitrate and machine task score.

This work introduces a method based on truncating bit depth to enhance RD performance in line with common test conditions of optimization of encoders and receiving systems for machine analysis of coded video content. Specifically, on the encoder side, the luma sample values are right shifted by 1 bit, while the chroma sample values remain unchanged. On the decoder side, the reconstructed luma samples are conditionally shifted back (left shift) accordingly.

BD rate coding gain (Pareto MOTA) on the AI, LD, RA configurations are respectively -33.43%, -25.54%, -31.70% in TVD dataset. The encoding time on AI, LD, RA configurations are 48.45%, 67.64%, 64.20% of the anchor.

# Introduction

A method for truncating the bit depth of video frames is proposed. Taking a 10-bit sequence as an example,  it converts a video frame with 10-bit sample representation to video frame with a lower bit depth for luma (luma represented by 9-bit sample with sample values range from 0 to 511, i.e., 9 bit range, no change on chroma samples) and decoded frame after converting back to 10 bits (represented by 10 bit sample with sample values range from 0 to 1023, i.e., 10 bit range). By converting the luma component of original video frame to lower value range before feeding the sequence into the encoder, the coding bits can be lower, but the decoded video frame can keep a similar (or slightly lower) machine task score.

# Proposal

With the proposed method, before the sequences are fed into the encoder, a right shift operation is applied to truncate the bit-depth of each luma sample, while the chroma components are kept unchanged. For 10-bit sequences, the luma sample values are righted shifted from 10 bits to 9 bits, and the sample value range is changed from [0, 1023] to [0, 511]. For 8-bit sequences, the luma sample values are righted shifted from 8 bits to 7 bits, and the sample value range is changed from [0, 255] to [0, 127]. At the decoder, the reconstructed samples are conditionally shifted back to the original bit depth and then further processed by the machine task pipeline.

# Experimental results

The performance of the proposed bit depth truncation method is evaluated following the common test conditions specified in JVET-AF2031 [1]. Table 1 presents the object detection performance of the proposed method with the SFU-HW dataset, providing a detailed breakdown across different classes. Table 2 examines both the encoding and decoding times in SFU dataset. Table 3 presents the object tracking performance on the TVD dataset. Table 4 shows the encoding and decoding times for the TVD dataset.

Table 1: The object detection performance of the proposed technology on SFU-HW dataset.

| Random Accesss | Object Detection | End-to-End BD-Rate [%] |  |  | End-to-End BD-mAP |
| --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP |  |
| SFU-HW | Class A | 19.32% | -2.63% | 19.32% | -1.58 |
|  | Class B | ########### | -13.01% | -12.75% | 2.74 |
|  | Class C | -13.88% | -18.50% | -13.88% | 1.33 |
|  | Class D | -11.85% | -12.71% | -11.85% | 0.92 |
|  | Average | ########### | -13.81% | -10.36% | 1.41 |
|  | Class O | ########### | ########### | #VALUE! | #VALUE! |

| Low Delay | Object Detection | End-to-End BD-Rate [%] |  |  | End-to-End BD-mAP |
| --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP |  |
| SFU-HW | Class A | -42.70% | -45.41% | -42.70% | 5.43 |
|  | Class B | #VALUE! | #VALUE! | -13.16% | #VALUE! |
|  | Class C | -14.33% | -14.40% | -14.33% | 2.19 |
|  | Class D | #NUM! | #NUM! | -8.50% | 1.31 |
|  | Average | #VALUE! | #VALUE! | -14.36% | #VALUE! |
|  | Class O | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

| All Intra | Object Detection | End-to-End BD-Rate [%] |  |  | End-to-End BD-mAP |
| --- | --- | --- | --- | --- | --- |
|  |  | mAP | mAP (low4) | Pareto mAP |  |
| SFU-HW | Class A | -18.63% | -21.08% | -19.06% | 2.34 |
|  | Class B | -17.69% | -18.09% | -17.91% | 4.38 |
|  | Class C | -3.41% | -2.29% | -3.41% | 0.30 |
|  | Class D | -2.84% | 4.16% | -2.84% | 0.16 |
|  | Average | -8.80% | -6.61% | -8.90% | 1.67 |
|  | Class O | #VALUE! | #VALUE! | #VALUE! | #VALUE! |

Table 2: The encoding time and decoding time of the proposed technology on SFU-HW dataset.

| config | AI |  | LD |  | RA |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Enc-time | Dec-time | Enc-time | Dec-time | Enc-time | Dec-time |
| Class A | 55.61% | 80.95% | 67.50% | 81.94% | 73.74% | 99.33% |
| Class B | 57.54% | 86.46% | 65.55% | 90.47% | 64.14% | 92.14% |
| Class C | 52.89% | 74.46% | 58.94% | 81.64% | 59.18% | 84.55% |
| Class D | 65.94% | 78.37% | 61.20% | 81.99% | 62.37% | 84.86% |
| AVG | 58.31% | 79.72% | 62.25% | 84.39% | 62.70% | 88.00% |

Table 3: The object tracking performance of the proposed technology on TVD dataset.

| Random Accesss | Object Tracking | End-to-End BD-Rate [%] |  |  |
| --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA |
| TVD | TVD-01-1 | -38.03% | -39.47% | -38.03% |
|  | TVD-01-2 | ########### | ########### | -32.77% |
|  | TVD-01-3 | ########### | -32.44% | -40.90% |
|  | TVD-02-1 | -13.22% | -10.32% | -13.22% |
|  | TVD-03-1 | -25.51% | -27.85% | -25.51% |
|  | TVD-03-2 | ########### | -43.44% | -39.39% |
|  | TVD-03-3 | -32.10% | -33.12% | -32.10% |
|  | Average | #NUM! | ########### | -31.70% |

| Low Delay | Object Tracking | End-to-End BD-Rate [%] |  |  |
| --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA |
| TVD | TVD-01-1 | ########### | ########### | -54.36% |
|  | TVD-01-2 | ########### | -4.42% | -26.96% |
|  | TVD-01-3 | -22.74% | -19.30% | -22.74% |
|  | TVD-02-1 | 20.77% | 41.36% | 20.77% |
|  | TVD-03-1 | -33.40% | -30.82% | -33.40% |
|  | TVD-03-2 | ########### | ########### | -36.84% |
|  | TVD-03-3 | -25.23% | -36.32% | -25.23% |
|  | Average | #NUM! | #NUM! | -25.54% |

| All Intra | Object Tracking | End-to-End BD-Rate [%] |  |  |
| --- | --- | --- | --- | --- |
|  |  | MOTA | MOTA (low4) | Pareto MOTA |
| TVD | TVD-01-1 | -35.18% | -34.16% | -35.18% |
|  | TVD-01-2 | -35.69% | -33.65% | -35.69% |
|  | TVD-01-3 | -36.95% | -26.56% | -36.95% |
|  | TVD-02-1 | -19.15% | 8.75% | -19.15% |
|  | TVD-03-1 | -33.37% | -32.44% | -33.37% |
|  | TVD-03-2 | -38.28% | -40.38% | -38.28% |
|  | TVD-03-3 | -35.41% | -39.14% | -35.41% |
|  | Average | -33.43% | -28.22% | -33.43% |

Table 4: The encoding time and decoding time of the proposed technology on TVD dataset.

| config | AI |  | LD |  | RA |  |
| --- | --- | --- | --- | --- | --- | --- |
|  | Enc-time | Dec-time | Enc-time | Dec-time | Enc-time | Dec-time |
| AVG | 48.45% | 75.53% | 67.64% | 82.40% | 64.20% | 88.88% |

# Conclusion

This contribution describes a method for truncating bit depth for luma component for enhancing coding efficiency in machine-oriented video coding. Experiment results show -33.43%, -25.54%, -31.70% BD rates (Pareto MOTA) in TVD dataset for AI, RA and LD configurations, respectively.

# References

- S. Liu, C. Hollmann, "Common test conditions for optimization of encoders and receiving systems for machine analysis of coded video content," Joint Video Experts Team (JVET) of ITU-T SG 16 WP 3 and ISO/IEC JTC 1/SC 29 32nd Meeting, JVET-AF2031, Hannover, DE, 13–20 October 2023.

# Patent rights declaration(s)

Tencent may have current or pending patent rights relating to the technology described in this contribution and, conditioned on reciprocity, is prepared to grant licenses under reasonable and non-discriminatory terms as necessary for implementation of the resulting ITU-T Recommendation | ISO/IEC International Standard (per box 2 of the ITU-T/ITU-R/ISO/IEC patent statement and licensing declaration form).